3GPP TSG-SA5 Meeting #159	S5-250860

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia, Huawei

# Title:	TS28.561 NDT concept and scope

# Document for:	Approval

Agenda Item:	6.19.5.1

# 1	Decision/action requested

Group is requested to discuss and agree on the “Detailed Proposal”.

# 2	References

[1]	None

# 3	Rationale

Add a description of NDT concept and scope

# 4	Detailed proposal

Start of Change

# 4	Concepts and overview

## 4.1	Introduction and Overview

### 4.1.1	Digital Twins and Network Digital Twins

A Digital twin is a representation of an object that models or simulates the characteristics and behaviours of the physical object. The Digital twin provides support to network management and operation by creating a virtual representation of the corresponding physical network process.  A digital twin can be created for any physical object, including any objects in communication networks. The digital twin may also be created for a group of objects, e.g. for the sets of network objects that form the RAN segment.

Accordingly, a digital twin modelling an object of a communication network is called Network Digital twin

Network Digital Twin (NDT) is used as a replica of a mobile network, in order to learn how an actual mobile network would behave in certain scenarios, without causing any changes to the actual mobile network. To provide meaningful results, the Network digital twin needs to emulate (or simulate) the behavior of the mobile network, so that the result of the operations on the virtual replica are good approximations to similar operations on the actual network.

NOTE:	Simulation implies that the NDT uses a mathematical model to characterize the behavior of the system. This is used e.g. to model the traffic generated by users in the RAN or to model the movement of users. Emulation implies that the NDT uses a realizable object such as software that replaces the live object to characterize the behavior of the system. This is used e.g. to model the behavior of core network function which can be modelled using the real software of the core network function.

### 4.1.2	Scope of NDTs

Existing automation capabilities include analytics services and decision-making capabilities with the assumption that the knowledge about network behavior is available within the automation functions. Accordingly, the unaddressed gap for network automation is the capabilities for modelling the behavior of the network. The Network Resource Model (NRM) to describes the attributes of a mobile network and their relationships. The concept of Network Digital Twin also adds the ability to model the behavior of a mobile network. This behavior is modelled by emulating or simulating a complete mobile network or limited aspects of a mobile network.

### 4.1.3	Relations between digital twins and network automation functions

The NDT is requested by the consumer to model a mobile network or part of one, support evaluating the corresponding impact, and return the report of the simulated impact generated by the NDT. The NDT is used as a virtual representation of a mobile network, which may synchronize data from the managed network for modelling. The digital twins provide modelling capabilities that are used by the network automation functions (e.g. MDA, SON, etc.) to accomplish their automation functionality. The related automation capabilities are provided by the network automation functions regardless of whether the digital twin models are external to or are integrated within the network automation functions. NDTs may also use network automation functionality.

## 4.2	NDT Life-cycle management

An NDT is an object that can be managed. NDT LCM may include the following capabilities:

-	Creation: The MnS producer who provides the NDT creates an instance to simulate/emulate the specific scenarios.

-	Activation: NDT MnS producer receives the request of a simulation/emualation task and prepares the execution of this simulation/emulation task. NDT can be reactivated when receives the new or modified request of a simulation/emulation task or continues a suspended simulation/emulation task.

.

-	Deactivation: The MnS producer may stop execution of an NDT instance or suspend execution of an instance that is running.

-	.

-	Deletion: the MnS producer may delete an NDT instance that is not under execution.

NOTE: All of the life cycle operations may not be valid for all NDT instances. For example, a particular NDT instance may not support suspending the execution..

End Change