3GPP TSG-SA5 Meeting #159	S5-250921

Sophia-Antipolis, France, 17 - 21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0670 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Enhanced the PRB usage with subcounter S-NSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PRB related measurements per S-NSSAI is of importance to the operator to monitor the Packet Loss Rate for different S-NSSAIs. As defined in A.6, it states “Network slicing is an important feature in 5GS, monitoring physical radio resource utilization per S-NSSAI is helpful for opeators to be aware of the load.”. In this case, the distribution of total PRB usage and the total available PRBs also need to be measured at the granularity of per S-NSSAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following Packet Loss Rate measurements are updated in line with the other measurements in 28.552 (that supports S-NSSAI) to have subcounters per S-NSSAI: - Distribution of DL Total PRB Usage - Distribution of UL total PRB usage - DL total available PRB - UL total available PRB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Packet Loss Rate measurements in 28.552 will not be defined for network slicing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.2.3, 5.1.1.2.4, 5.1.1.2.6, 5.1.1.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of next change

5.1.1.2.3	Distribution of DL Total PRB Usage

a)	This measurement provides the distribution of samples with total usage (in percentage) of physical resource blocks (PRBs) on the downlink in different ranges. This measurement is a useful measure of whether a cell is under high loads or not in the scenario which a cell in the downlink may experience high load in certain short times (e.g. in a millisecond) and recover to normal very quickly. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC)) and subcounters per supported S-NSSAI and subcounters per supported PLMN ID.

b)	CC

c)	Each measurement sample is obtained as: Mn=M1nPn*100, where  Mn  is total PRB usage at sample n for DL, which is a percentage of PRBs used, averaged during time period tn (e.g. a millisecond) with value range: 0-100%; M1n is a count of full physical resource blocks and all PRBs used for DL traffic transmission shall be included; Pn is the total number of PRBs available for DL traffic transmission during time period tn and n is the sample with time period tn during which the measurement is performed.

Distribution of total PRB usage is calculated in the time-frequency domain only. The reference point is the Service Access Point between MAC and L1. The distribution of PRB usage provides the histogram result of the samples collected during time period tn.

Depending on the value of the sample, the proper bin of the counter is increased. The number of samples during one measurement period is defined by the vendor.

d)	A set of integers. Each representing the (integer) number of samples with a DL total PRB percentage usage in the range represented by that bin.

e)	RRU.PrbTotDlDist.Bin, which Bin indicates the distribution of DL PRB Usage for all traffic which is vendor specific; or 
RRU.PrbTotDlDist.PLMN, where PLMN identifies the PLMN ID, 
RRU.PrbTotDlDist.QoS, where the QoS identifies the target quality of service class 
RRU.PrbTotDlDist.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer.

5.1.1.2.4	Distribution of UL total PRB usage

a)	This measurement provides the distribution of samples with total usage (in percentage) of physical resource blocks (PRBs) on the uplink in different usage ranges. This measurement is a useful measure of whether a cell is under high loads or not in the scenario which a cell in the uplink may experience high load in certain short times (e.g. in a millisecond) and recover to normal very quickly. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC)) and subcounters per supported S-NSSAI and subcounters per supported PLMN ID.

b)	CC

c)	Each measurement sample is obtained as: Mn=M1nPn*100, where  Mn  is total PRB usage at sample n for UL, which is a percentage of PRBs used, averaged during time period tn (e.g. a millisecond) with value range: 0-100%; M1n is a count of full physical resource blocks and all PRBs used for UL traffic transmission shall be included; Pn is the total number of PRBs available for UL traffic transmission during time period tn and n is the sample with time period tn during which the measurement is performed.

Distribution of total PRB usage is calculated in the time-frequency domain only. The reference point is the Service Access Point between MAC and L1. The distribution of PRB usage provides the histogram result of the samples collected during time period tn.

Depending on the value of the sample, the proper bin of the counter is increased. The number of samples during one measurement period is defined by the vendor.

d)	A set of integers, each representing the (integer) number of samples with a UL PRB percentage usage in the range represented by that bin.

e)	RRU.PrbTotUlDist. Bin, which indicates the distribution of UL PRB Usage for all traffic which is vendor specific;  or.

RRU.PrbTotUlDist.PLMN, where PLMN identifies the PLMN ID, 
RRU.PrbTotUlDist.QoS, where the QoS identifies the target quality of service class 
RRU.PrbTotUlDist.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the load of the radio physical layer.

Next change

5.1.1.2.6	DL total available PRB

a)	This measurement provides the total number of physical resource blocks (PRBs) in average available downlink. The measurement is optionally split into subcounter per supported PLMN ID and subcounters per QoS level (mapped 5QI or QCI in EN-DC)) and subcounters per supported S-NSSAI.

b)	SI.

c)	Each measurement is obtained as the average (arithmetic mean) of total availible count of PRBs available for DL traffic transmission per S-NSSAI and per PLMN ID during time period T.

d)	Each measurement (average number of DL PRBs) is a single integer value. If the optional measurements are performed, the number of measurements is equal to the number of supported PLMN.

e)	RRU.PrbAvailDl, or optionally RRU.PrbAvailDl.PLMN, where PLMN identifies the PLMN ID, or optionally RRU. PrbAvailDl.QoS, where the QoS identifies the target quality of service class and RRU. PrbAvailDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the total number of available PRBs in average for DL traffic.

Next change

5.1.1.2.8	UL total available PRB

a)	This measurement provides the total number of physical resource blocks (PRBs) available uplink. The measurement is optionally split into subcounter per supported PLMN ID and subcounters per QoS level (mapped 5QI or QCI in EN-DC)) and subcounters per supported S-NSSAI.

b)	SI.

c)	Each measurement is obtained as the average number (arithmetic mean) of total available count of PRBs available for UL traffic transmission per S-NSSAI and per PLMN ID during time period T.

d)	Each measurement (average of total number of UL PRBs) that is a single integer value. If the optional measurements are performed, the number of measurements is equal to the number of supported PLMN.

e)	RRU.PrbAvailUl, which indicates the UL PRB available. Or optionally RRU.PrbAvailUl.PLMN, where PLMN identifies the PLMN ID or optionally RRU. PrbAvailUl.QoS, where the QoS identifies the target quality of service class and RRU. PrbAvailUl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for monitoring the total number of available PRBs in average UL.

End of change