3GPP TSG-SA5 Meeting #159	S5-250974

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0166 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.104 Correct the MDAType definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MDAType <<enumeration>> is defined in clause 8.6.1, however, following MDAType definitions are not aligned: - In clause 9.5.1, the type of “mDAType” is string. Also the reference to MDAType <<enumeration>> in clause 8.6.1 is missing. - In OpenAPI/TS28104_MdaNrm.yaml, the type of attribute “mDAType” in MDAOutputPerMDAType is string. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the type of attribute “mDAType” to Enum and add reference to MDAType <<enumeration>> in both stage 2 and stage 3 definition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect definition for attribute “mDAType”. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.1, Following YAML files normatively defined in the forge are updated: - TS28104_MdaNrm.yaml - TS28104_MdaReport.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1565 at commit 20ac1cf66ed5cc7e960aae4c9990718c55e871f0 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

9.5.1	Attribute properties

Table 9.5.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mDAType | It indicates the MDA type (corresponding to the MDA capability). AllowedValues: the value of MDA type see clause 8.6.1 MDAType <<enumeration>>. | type: MDAType multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| requestedMDAOutputs | It indicates the requested analytics outputs for an MDA request. | type: MDAOutputPerMDAType multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mDAOutputIEFilters | It provides the filters for the analytics output information elements of an MDA type for an MDA request. | type: MDAOutputIEFilter multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mDAOutputIEName | It indicates the analytics output information element name. allowedValues: the analytics output information element names for each MDA type as specified in clause 8. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| filterValue | It indicates the filter value for analytics output information element for an MDA request. The MDA output information element is only requested and reported when its value equals to the value of this attribute. allowedValues: depends on the definitions of the analytics output information element (see clause 8) indicated by mDAOutputIEName attribute. | The type for the corresponding mDAOutputIEName as defined in clause 8 |
| threshold | It indicates the threshold for analytics output information element for an MDA request. | type: ThresholdInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| analyticsPeriod | It indicates a list of time durations, or a time-period related to a time schedule for analytics. | type: AnalyticsSchedule multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| timeOut | It indicates a time until which an MDA MnS consumer needs to obtain an MDA output. Beyond this time the MDA output is no longer needed by the MDA MnS consumer. | type: DateTime (see TS 32.156 [18]) multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportingMethod | It indicates the reporting method of the analytics output selected by the MnS consumer. allowedValues: File, Streaming, Notification. | type: Enum multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportingTarget | It indicates the reporting target of the MDA outputs. Allowed values: URI. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| analyticsScope | It indicates the scope of the analytics requested by the MnS consumer. | type: AnalyticsScopeType multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| managedEntitiesScope | It indicates the scope of the analytics by the DNs of the managed entities. It carries the DN(s) of SubNetwork MOI(s), ManagedElement MOI(s), and/or the MOI(s) of the derivative IOCs of ManagedFunction (see TS 28.622 [19]). For each MOI provided by this attribute, the MOI itself and all of its subordinated MOIs are in the scope of analytics. | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| areaScope | It indicates the scope of the analytics by the geographical area information. | type: GeoArea (see TS 28.622 [19]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| startTime | It indicates the start time of the periodical analytics requested by the MnS consumer. | type: DateTime (see TS 32.156 [18]) multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| stopTime | It indicates the stop time of the periodical analytics requested by the MnS consumer. This attribute shall contain a NULL value in case the analytics is requested for an indefinite time period. | type: DateTime (see TS 32.156 [18]) multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mDAReportID | It indicates the identifier for the MDAReport. | type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mDAOutputList | It indicates a list of output results related to particular MDA type. | type: MDAOutputEntry multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| analyticsWindow | It indicates the time duration related to an MDA output. It can be in the past, when the analytics is statistics, or in the future for a prediction. | type: TimeWindow multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mDAOutputIEValue | It indicates the MDA output result that can be numeric or non-numeric. | The type for the corresponding mDAOutputIEName as defined in clause 8 |
| confidenceDegree | A probability range that contains the degree of accuracy of the analytics output statistics or prediction. | type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| supportedMDACapabilities | It indicates the MDA capabilities supported by the MDA function. AllowedValues: the value of MDA types defined for the MDA capabilities in clause 8. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mDAOutputs | It indicates the analytics output results of one or more MDA types delivered to MDA consumer. | type: MDAOutputs multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mDARequestRef | It indicates the DN of the MDARequest MOI for which the results are generated by the MDA producer. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| monitoredMDAOutputIE | It indicates the analytics output information element name monitored by a threshold. AllowedValues: the analytics output information element names for each MDA type as specified in clause 8. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thresholdValue | It specifies the value against which the monitored MDA output information element is compared at a threshold level in case the hysteresis is zero. | type: Float or Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| hysteresis | It specifies the hysteresis of a threshold. If this attribute is present the monitored MDA output information element value is not compared against the threshold value as specified by the thresholdValue attribute but against a high and low threshold value given by highThresholdValue- = thresholdValue + hysteresis lowThresholdValue = thresholdValue - hysteresis When going up, the threshold is triggered when the MDA output information element value reaches or crosses the high threshold value. When going down, the threshold is triggered when the MDA output information element value reaches or crosses the low threshold value. allowedValues: values | type: Float or Integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thresholdDirection | It indicates the direction of a threshold indicating the direction for which a threshold crossing triggers a threshold. When the threshold direction is configured to "UP", the associated treshold is triggered only when the subject MDA output information element value is going up upon reaching or crossing the threshold value. The treshold is not triggered, when the MDA output information element value is going down upon reaching or crossing the threshold value. Vice versa, when the threshold direction is configured to "DOWN", the associated treshold is triggered only when the MDA output information element value is going down upon reaching or crossing the threshold value. The treshold is not triggered, when the MDA output information element value is going up upon reaching or crossing the threshold value. When the threshold direction is set to "UP_AND_DOWN" the treshold is active in both direcions. In case a threshold with hysteresis is configured, the threshold direction attribute shall be set to "UP_AND_DOWN". allowedValues: - UP - DOWN - UP_AND_DOWN | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mDAOutputStartTime | It indicates the analytics start time for an MDA output. | type: DateTime (see TS 32.156 [18]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mDAOutputEndTime | It indicates the analytics end time for an MDA output. | type: DateTime (see TS 32.156 [18]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| timeDurations | It indicates a list of time duration. | type: TimeWindow multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| granularityPeriod | It indicates the granularity period (in unit of second) of the analytics for an MDA output. In case of PM prediction, this indicates the granularity period of the prediction of the PMs. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLModelRefList | This attribute holds a DN list of MLModel (See TS 28.105 [24]). | type: DN multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aIMLInferenceFunctionRefList | This attribute holds a DN list of AIMLInferenceFunction (See TS 28.105 [24]) | type: DN multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendationFilter | It indicates the entities for which no recommendation should be generated for the specific MDAOutputPerMDAType. This could be provided either as managedEntitiesScope or as areaScope. | type: AnalyticsScopeType multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

2nd Change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1565 at commit 20ac1cf66ed5cc7e960aae4c9990718c55e871f0

*** START OF CHANGE 1 ***

*** OpenAPI/TS28104_MdaNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: MDA NRM

version: 18.6.0

description: >-

OAS 3.0.1 specification of the MDA NRM

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.104; MDA

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.104/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

MDAType:

type: string

enum:

- COVERAGEANALYTICS_COVERAGEPROBLEMANALYSIS

- COVERAGEANALYTICS_PAGINGOPTIMIZATION

- SLSANALYSIS_SERVICEEXPERIENCEANALYSIS

- SLSANALYSIS_NETWORKSLICETHROUGHPUTANALYSIS

- SLSANALYSIS_NETWORKSLICETRAFFICANALYSIS

- SLSANALYSIS_E2ELATENCYANALYSIS

- SLSANALYSIS_NETWORKSLICELOADANALYSIS

- MDAASSISTEDFAULTMANAGEMENT_FAILUREPREDICTION

- MDAASSISTEDENERGYSAVING_ENERGYSAVINGANALYSIS

- MOBILITYMANAGEMENTANALYTICS_MOBILITYPERFORMANCEANALYSIS

- MOBILITYMANAGEMENTANALYTICS_HANDOVEROPTIMIZATION

- MAINTENANCE_MAINTENANCEANALYTICS

- RESOURCEANALYTICS_VIRTUALIZEDRESOURCEUTILIZATIONANALYSISNF

- RESOURCEANALYTICS_PHYISCALRESOURCEUTILIZATIONANALYSISNF

- RESOURCEANALYTICS_5GCCONTROLPLANECONGESTIONANALYSIS

- PREDICTIONS_PMDATA

MDATypes:

type: array

items:

$ref: '#/components/schemas/MDAType'

MDAOutputs:

type: array

items:

$ref: '#/components/schemas/MDAOutputPerMDAType'

AnalyticsScopeType:

oneOf:

- type: object

properties:

managedEntitiesScope:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

- type: object

properties:

areaScope:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/GeoArea'

MDAOutputPerMDAType:

type: object

properties:

mDAType:

$ref: '#/components/schemas/MDAType'

mDAOutputIEFilters:

type: array

items:

$ref: '#/components/schemas/MDAOutputIEFilter'

MDAOutputIEFilter:

type: object

properties:

mDAOutputIEName:

type: string

filterValue:

type: string

threshold:

type: array

items:

$ref: '#/components/schemas/ThresholdInfo'

analyticsPeriod:

$ref: '#/components/schemas/AnalyticsSchedule'

timeOut:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

ReportingMethod:

type: string

enum:

- FILE

- STREAMING

- NOTIFICATION

ReportingTarget:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

AnalyticsSchedule:

oneOf:

- type: object

properties:

timeDurations:

type: array

items:

$ref: 'TS28104_MdaReport.yaml#/components/schemas/TimeWindow'

- type: object

properties:

granularityPeriod:

type: integer

ThresholdInfo:

type: object

properties:

monitoredMDAOutputIE:

type: string

thresholdDirection:

type: string

enum:

- UP

- DOWN

- UP_AND_DOWN

thresholdValue:

oneOf:

- type: integer

- $ref: 'TS28623_ComDefs.yaml#/components/schemas/Float'

hysteresis:

oneOf:

- type: integer

minimum: 0

- type: number

format: float

minimum: 0

#-------- Definition of types for name-containments ------

SubNetwork-ncO-MdaNrm:

type: object

properties:

MDAFunction:

$ref: '#/components/schemas/MDAFunction-Multiple'

MDAReport:

$ref: '#/components/schemas/MDAReport-Multiple'

ManagedElement-ncO-MdaNrm:

type: object

properties:

MDAFunction:

$ref: '#/components/schemas/MDAFunction-Multiple'

#-------- Definition of abstract IOCs --------------------------------------------

#-------- Definition of concrete IOCs --------------------------------------------

MDAFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

supportedMDACapabilities:

$ref: '#/components/schemas/MDATypes'

mLModelRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

aIMLInferenceFunctionRefList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

MDARequest:

$ref: '#/components/schemas/MDARequest-Multiple'

MDAReport:

$ref: '#/components/schemas/MDAReport-Multiple'

MDARequest-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

requestedMDAOutputs:

$ref: '#/components/schemas/MDAOutputs'

reportingMethod:

$ref: '#/components/schemas/ReportingMethod'

reportingTarget:

$ref: '#/components/schemas/ReportingTarget'

analyticsScope:

$ref: '#/components/schemas/AnalyticsScopeType'

startTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

stopTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

recommendationFilter:

$ref: '#/components/schemas/AnalyticsScopeType'

MDAReport-Single:

$ref: 'TS28104_MdaReport.yaml#/components/schemas/MDAReport'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

MDAFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/MDAFunction-Single'

MDARequest-Multiple:

type: array

items:

$ref: '#/components/schemas/MDARequest-Single'

MDAReport-Multiple:

type: array

items:

$ref: '#/components/schemas/MDAReport-Single'

#-------- Definitions in TS 28.104 for TS 28.532 ---------------------------------

resources-mdaNrm:

oneOf:

- $ref: '#/components/schemas/MDAFunction-Single'

- $ref: '#/components/schemas/MDARequest-Single'

- $ref: '#/components/schemas/MDAReport-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***

*** START OF CHANGE 2 ***

*** OpenAPI/TS28104_MdaReport.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: MDA Report

version: 18.6.0

description: >-

OAS 3.0.1 specification of the MDA Report

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.104; MDA Report

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.104/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

MDAOutputs:

type: object

properties:

mDAType:

$ref: 'TS28104_MdaNrm.yaml#/components/schemas/MDAType'

mDAOutputList:

type: array

items:

$ref: '#/components/schemas/MDAOutputEntry'

analyticsWindow:

$ref: '#/components/schemas/TimeWindow'

confidenceDegree:

type: number

format: float

MDAOutputEntry:

type: object

properties:

mDAOutputIEName:

type: string

mDAOutputIEValue: {}

TimeWindow:

type: object

properties:

mDAOutputStartTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

mDAOutputEndTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

#-------- Definition of MDA Report --------------------------------------------

MDAReport:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

mDAReportID:

type: string

mDAOutputs:

type: array

items:

$ref: '#/components/schemas/MDAOutputs'

mDARequestRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

<CODE ENDS>

*** END OF CHANGE 2 ***

End of Changes