3GPP TSG-SA5 Meeting #159	S5-251005

Sophia-Antipolis, France, 17 - 21 February 2025

# Source:	Nokia, China Mobile, Huawei

# Title:	TS28.561 NDTs for verification

# Document for:	Approval

Agenda Item:	6.19.5.3

# 1	Decision/action requested

Group is requested to discuss and agree on the “Detailed Proposal”.

# 2	References

[1]	None

# 3	Rationale

Add a description of NDT use cases and requirements

# 4	Detailed proposal

Start of First change

# 5	Management Capabilities

## 5.3	NDT support for verification - NDTVER

### 5.3.1	Description

This describes NDT use in support of verification. The MnS consumer should be enabled to configure the NDT with data defining a network scenario to be verified/validated as well as the configurations or policies that should be applied to the network scenario to be verified/validated. Given the scenario and configuration/policies, the consumer can then control the simulation (e.g. to start or stop the simulation) as described in clause 5.3. The NDT applies the configurations and policies and collects metrics that measure the impacts of those configuration and policies. These are then provided as the outcomes of the verification/validation.

### 5.3.2	Use cases

#### 5.3.2.2	Verification of network configuration - VERCFG

Several configurations can be provided to the network, either as modifications to single parameters or as bulk configurations, e.g., as a configuration plan. There may be multiple configuration that are iteratively executed in NDT until the network performance, e.g. energy efficiency of NG-RAN, UE throughput in gNB, etc., meets the operators’ requirements.

The MnS consumer should be enabled to configure the NDT with data defining a network scenario to be verified/validated as well as the configurations or policies that should be applied to the network scenario to be verified/validated. Given the scenario and configuration/policies, the consumer can then control the simulation (e.g. to start or stop the simulation) as described in clause 5.3.

The following are scenarios for of verifications that the NDT might support include:

-	verification of a network management RAN ES configuration plans

-	validation of a signaling storm solution

-	evaluating the network’s emergency preparedness, e.g., to evaluate how a given network configuration will react to a natural disaster

-	verification of various network resource and network function configurations and software/hardware modifications

In this scenario, MnS consumer (the entity who consumes the network simulation service) sends the request to MnS producer to verify the configurations. The request includes the configuration plan listing specific configurations to be made (e.g. precision of energy saving parameters, maximum run time, etc.) and performance requirements (e.g. Energy efficiency, RAN UE throughput, etc). The NDT simulates the defined scenario of the network applying the provided configuration plans according to the performance requirements. The NDT collects performance metrics indicating the impact of the configuration plans. The MnS producer needs to send the simulation report to MnS consumer to indicates the network simulation results.

#### 5.3.2.3	Verification of network response to failure events - VEREVT

A failure event refers to the situation where users in the network are completely unable to get service form the network.

Example:	A signaling storm is an example failure events where a large number of signaling messages suddenly surge in the mobile communication network, resulting in the network processing capacity overload, thus affecting the network performance and stability. During this period, users will repeatedly try to establish the connection until reconnected, thus generating a large number of signaling messages surge suddenly, causing signaling storm.

The NDT can be used to validate whether the proposed solution can resolve the failure event, e.g., the signaling storm issue. The NDT can model network behaviors and provide information on potential impact of network failures including on surges in signaling requests on the network. For a signaling storm, for example, the information can that enable to identify the optimal flow control parameters for each signaling impact point.

The following are examples of verifications that the NDT might support:

-	verification of a Network management RAN ES policy

-	validation of a signaling storm solution

-	evaluating the network’s emergency preparedness, e.g., to evaluate how a given network configuration will react to a natural disaster

-	verification of various network configurations and software/hardware modifications

### 5.X.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTVER-VERRQT-01: | The management system should support a capability to provide a report/output on the simulation/emulation of verification of network scenarios and configurations. | NDTVER-VERRQT |
| REQ- NDTVER-VERRQT-02: | The management system should support a capability enabling the MnS consumer to define the data that should be included in the report/output on the simulation/emulation of verification of network scenarios and configurations. | NDTVER-VERRQT |
| REQ-NDTVER-VERCFG-01: | The management system should have the capability allowing the consumer to configure the network configurations. | NDTVER-VERCFG |
| REQ- NDTVER-VERCFG -02: | The management system should have the capability to estimate the impact of a network configuration. | NDTVER-VERCFG |
| REQ- NDTVER-VERCFG -03: | The management system should have the capability to report the simulated impact of network configuration. | NDTVER-VERCFG |
| REQ-NDTVER-VEREVT-01: | The management system should have the capability to simulate/emulate the behaviour of a failure event. | NDTVER-VEREVT |
| REQ-NDTVER-VEREVT-02: | The management system should have a capability enabling the MnS consumer to define a network configuration to be modelled (e.g., for checking the network response to a failure event). | NDTVER-VEREVT |
| REQ-NDTVER-VEREVT-03: | The management system should have the capability to report the network behavior related to a failure event (e.g. a signaling storm). | NDTVER-VEREVT |

End Change