3GPP TSG-SA5 Meeting #159	S5-251133

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.622 | CR | 0534 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.622 Clarify usage of notifyFileReady for PM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Inc., Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As specified, when only the fileReportingPeriod and fileLocation attributes are present, the MnS producer shall store the files on a MnS consumer, that can be any entity such as a file server, at the location specified by fileLocation. No notification is emitted by the MnS producer. However, there are use cases that the entity receiving the file and the entity initiated the PM/Trace are not the same. Therefore, there is a need to allow the MnS producer to infom the PM/Trace initiator to receive the notification of a file ready. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allowing the MnS producer to infom the MnS consumer to receive the notification of a file ready. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Difficult for MnS consumer to know if a file is ready when only the fileReportingPeriod and fileLocation attributes are present. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.33.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR 28.623 CR 0512 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 4.3.33	ReportingCtrl <<choice>>

#### 4.3.33.1	Definition

This <<choice>> defines the method for reporting collected performance metrics to MnS consumers as well as the parameters for configuring the reporting function. It is a choice between the control parameter required for the reporting methods, whose presence selects the reporting method as follows:

When only the fileReportingPeriod attribute is present (CHOICE_1), the MnS producer shall store files on the MnS producer at a location selected by the MnS producer and, on condition that an appropriate subscription is in place, inform the MnS consumer about the availability of new files and the file location using the notifyFileReady notification. In case the preparation of a file fails, "notifyFilePreparationError" shall be sent instead.

When the fileReportingPeriod and notificationRecipientAddress attributes are present (CHOICE_2), then the MnS producer shall behave like described for the case that only the fileReportingPeriod is present. In addition, the MnS producer shall create on behalf of the MnS consumer a subscription, using NtfSubscriptionControl, for the notification types notifyMOICreation and notifyMOIDeletion related to the File instances that will be produced later. In case an existing subscription does already include the File instances to be produced, no new subscription shall be created. The notificationRecipientAddress attribute in the created NtfSubscriptionControl instance shall be set to the value of the "notificationRecipientAddress" in the related PerfMetricJob. This feature is called implicit notification subscription, as opposed to the case where the MnS consumer creates the subscription (explicit notification subscription). When the related PerfMetricJob is deleted, the NtfSubscriptionControl instance created due to the request for implicit subscription shall be deleted as well.

When only the “fileReportingPeriod” and “fileLocation” attributes are present (CHOICE_3), the MnS producer shall store the files at the location specified by “fileLocation”. The file location may identify any entity such as a file server or a MnS consumer. The identified MnS consumer may or may not be identical to the MnS consumer creating the “PerfMetricJob”. As for CHOICE_1 the MnS producer may emit “notifyFileReady” and "notifyFilePreparationError" notifications to inform subscribers that a file has been made available at the location specified by “fileLocation”.

When only the streamTarget attribute is present (CHOICE_4), the MnS producer shall stream the data to the location specified by streamTarget.

For the file-based reporting methods the fileReportingPeriod attribute specifies the time window during which collected measurements are stored into the same file before the file is closed and a new file is opened.

#### 4.3.33.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| CHOICE_1.1 fileReportingPeriod | CM | T | T | F | T |
| CHOICE_2.1 fileReportingPeriod | CM | T | T | F | T |
| CHOICE_2.2 notificationRecipientAddress | CM | T | T | F | T |
| CHOICE_3.1 fileReportingPeriod | CM | T | T | F | T |
| CHOICE_3.2 fileLocation | CM | T | T | F | T |
| CHOICE_4.1 streamTarget | CM | T | T | F | T |

#### 4.3.33.3	Attribute constraints

| Name | Definition |
| --- | --- |
| CHOICE_1.1 fileReportingPeriod | This attribute shall be supported, when the MnS producer supports file based reporting and storing files on the MnS producer. |
| CHOICE_2.1 fileReportingPeriod CHOICE_2.2 notificationRecipientAddress | These attributes shall be supported, when the MnS producer supports file based reporting, storing files on the MnS producer and implicit notification subscription. |
| CHOICE_3.1 fileReportingPeriod CHOICE_3.2 fileLocation | These attributes shall be supported, when MnS producer supports file based reporting and storing files on a MnS consumer. |
| CHOICE_4.1 streamTarget | This attribute shall be supported, when the MnS producer supports stream-based reporting. |

#### 4.3.33.4	Notifications

The subclause 4.5 of the <<IOC>> using this <<dataType>> as one of its attributes, shall be applicable.