3GPP TSG-SA5 Meeting #160	S5-251290

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 1489 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.541 YANG stage-3 corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson India Private Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | YANG SS is not matching the approved stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update YANG code to match existing stage 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 and Stage 3 mismatch; interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge only |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1636 at commit d20ac4b8bce24b4d1c06a9edb347a2fe56bf8a54 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-5g-common-yang-types.yang ***

<CODE BEGINS>

module _3gpp-5g-common-yang-types {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-5g-common-yang-types";

prefix "types5g3gpp";

import ietf-yang-types { prefix yang; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "The model defines common types for 5G networks and

network slicing.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-03-25 { reference CR-1489 ; }

revision 2024-11-01 { reference CR-1405; }

revision 2024-10-06 { reference CR-1389; }

revision 2024-05-24 { reference CR-1273 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-05-10 { reference CR-0916; }

revision 2021-08-05 { reference S5-214053/CR-0518; }

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-20 { reference "Initial version."; }

grouping MappedCellIdInfoGrp {

description "This data type represents the mapping relationship between

Mapped Cell IDs and geographical areas (see clause 16.14.5 of TS 38.300";

list ntnGeoArea {

description "This attribute indicates a specific geographical location

mapped to Mapped Cell ID(s).";

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32; }

uses types3gpp:GeoAreaGrp;

}

list mappedCellId  {

description "This attribute is in format of NCGI to indicate a fixed

geographical area (See subclause 16.14.5 in TS 38.300)";

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32; }

uses NcgiGrp;

}

}

grouping NcgiGrp {

description "Represents the Ncgi datatype";

list plmnId {

description "This attribute represents a PLMN Identity.";

min-elements 1;

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses types3gpp:PLMNId ;

}

leaf nrCellId {

type string;

mandatory true;

description "This attribute represents NR Cell Identity.

It's a 36-bit string identifying an NR Cell Id as specified in

clause 9.3.1.7 of TS 38.413, in hexadecimal representation. Each

character in the string shall take a value of

'0' to '9', 'a' to 'f' or 'A' to 'F' and shall represent 4 bits.

The most significant character representing the 4 most significant

bits of the Cell Id shall appear first in the string, and the

character representing the 4 least significant bit of the

Cell Id shall appear last in the string.

Pattern: '^[A-Fa-f0-9]{9}$'

Example:

An NR Cell Id 0x225BD6007 shall be encoded as '225BD6007'.";

}

leaf nId {

type string;

mandatory true;

description "Network Identity; Shall be present if PlmnIdNid identifies

an SNPN (see clauses 5.30.2.3, 5.30.2.9, 6.3.4, and 6.3.8 in

3GPP TS 23.501.";

}

}

typedef NRTAC {

type string;

description "This holds the identity of the common Tracking Area Code

for the PLMNs.

allowedValues:

a) It is the TAC or Extended-TAC.

b) A cell can only broadcast one TAC or Extended-TAC. See TS 36.300,

subclause 10.1.7 (PLMNID and TAC relation).

c) TAC is defined in subclause 19.4.2.3 of 3GPP TS 23.003

and Extended-TAC is defined in subclause 9.3.1.29 of 3GPP TS 38.473.

d) For a 5G SA (Stand Alone), it has a non-null value.";

}

grouping SNssai {

description

"Single Network Slice Selection Assistance Information(S-NSSAI)";

reference "3GPP TS 23.003";

leaf sd {

description "Slice Differentiator

If not needed, the value can be set to ff:ff:ff.";

type yang:hex-string {

length 8;

}

reference "3GPP TS 23.003";

}

leaf sst {

type uint8;

description "Slice/Service Type.

Values 0 to 127 belong to standardized SST range and are defined in

3GPP TS 23.501. Values 128 to 255 belong to operator-specific range.";

}

}

grouping PlmnIdNid {

description "Represents the SCP domain specific information as defined

in TS 29.510 ";

uses types3gpp:PLMNId;

leaf nid {

type string;

description "This attribute represents network Identity;

Shall be present if PlmnIdNid identifies an SNPN.

(see clauses 5.30.2.3, 5.30.2.9, 6.3.4, and 6.3.8 in TS 23.501";

}

}

grouping PLMNInfo {

description "The PLMNInfo data type define a S-NSSAI member in a specific

PLMNId, and it have two attributes PLMNId and S-NSSAI (PLMNId, S-NSSAI).

The PLMNId represents a data type that is comprised of mcc

(mobile country code) and mnc (mobile network code), (See TS 23.003

subclause 2.2 and 12.1) and S-NSSAI represents an data type, that is

comprised of an SST (Slice/Service type) and an optional

SD (Slice Differentiator) field";

uses types3gpp:PLMNId;

uses SNssai;

}

typedef CommModelType {

reference "3GPP TS 23501";

type enumeration {

enum DIRECT_COMMUNICATION_WO_NRF {

value 0;

description "Directly communicate to other pre-configured NF service.";

}

enum DIRECT_COMMUNICATION_WITH_NRF {

value 1;

description "Directly communicate to other NF service discovered

by NRF.";

}

enum INDIRECT_COMMUNICATION_WO_DEDICATED_DISCOVERY {

value 2;

description "Communicate to pre-configured other NF service through

SCP as a proxy.";

}

enum INDIRECT_COMMUNICATION_WITH_DEDICATED_DISCOVERY {

value 3;

description "Communication to NF service discovered by NRF through SCP

as a proxy.";

}

}

}

grouping CommModel {

leaf groupId {

type uint16;

}

leaf commModelType {

type CommModelType;

}

leaf-list targetNFServiceList {

type types3gpp:DistinguishedName;

}

leaf commModelConfiguration {

type string;

}

}

grouping TaiRangeGrp {

list plmnId {

description "PLMN ID related to the TacRange.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list tacRangeList {

description "The range of the TACs.";

min-elements 1;

key "start end";

uses TacRange;

}

}

grouping TacRange {

leaf start {

description "First value identifying the start of a TAC range,

to be used when the range of TAC's can be represented

as a hexadecimal range (e.g., TAC ranges).";

type string {

pattern '^([A-Fa-f0-9]{4}|[A-Fa-f0-9]{6}$)';

}

}

leaf end {

description "Last value identifying the end of a TAC range,

to be used when the range of TAC's can be represented as

a hexadecimal range (e.g. TAC ranges).";

type string {

pattern '^([A-Fa-f0-9]{4}|[A-Fa-f0-9]{6})$';

}

}

leaf nRTACpattern {

description "Pattern (regular expression according to the ECMA-262)

representing the set of TAC's belonging to this range.

A TAC value is considered part of the range if and only if the

TAC string fully matches the regular expression.";

type string;

}

}

grouping SupportedFunc {

leaf function {

type string;

}

leaf policy {

type string;

}

}

typedef EnergySavingLoadThresholdT {

type uint32 {

range 0..10000;

}

units 1/10000;

}

typedef EnergySavingTimeDurationT {

type uint32 {

range 0..900;

}

units seconds;

}

typedef PhysCellID {

type uint32 {

range "0..1007";

}

reference "clause 7.4.2 of TS 38.211";

}

typedef UTC24TimeOfDayT {

description "Time of day in HH:MM or H:MM 24-hour format per UTC

time zone.";

type string {

pattern "(([01]?[0-9])|(2[0-3])):([0-5][0-9])";

}

}

typedef DayOfWeekT {

type enumeration {

enum Monday;

enum Tuesday;

enum Wednesday;

enum Thursday;

enum Friday;

enum Saturday;

enum Sunday;

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***

*** START OF CHANGE 2 ***

*** yang-models/_3gpp-5gc-nrm-amffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-amffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amffunction;

prefix amf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import ietf-inet-types { prefix inet; }

import ietf-yang-types { prefix yang; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "AMFFunction derived from basic ManagedFunction.

Copyright 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-03-25 { reference CR-1489 ; }

revision 2024-11-01 { reference CR-1405; }

revision 2024-10-06 { reference CR-1389; }

revision 2024-04-04 { reference CR-1139; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2022-01-07 { reference CR-0643; }

revision 2020-11-06 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 { reference "Ericsson refactoring."; }

revision 2018-08-07 { reference "Initial revision"; }

grouping NTNGlobalRanNodeIDGrp{

list pLMNId {

description "The PLMN Identifier is composed of

a Mobile Country Code (MCC) and

a Mobile Network Code (MNC).";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

choice GlobalRanNodeID{

description "Unique identifier of an NG-RAN node.";

reference "3GPP TS 38.413";

mandatory true;

case gNB {

leaf GlobalgNBID{

description "Unique identifier of a gNB.";

reference "3GPP TS 38.413";

type int64;

mandatory true;

}

}

case ng-eNB{

leaf Globalng-eNBID{

type string{

pattern '^(MacroNGeNB-[A-Fa-f0-9]{5}|LMacroNGeNB-

[A-Fa-f0-9]{6}|SMacroNGeNB

-[A-Fa-f0-9]{5})$';

}

description "This represents the identifier of

the ng-eNB ID as specified in

clause 9.3.1.8 of 3GPP TS 38.413.";

mandatory true;

}

}

case N3IWF{

leaf N3IWFID{

type string{

pattern '^[A-Fa-f0-9]+$';

}

description "This IE shall contain the N3IWF

identifier received over NGAP and

shall be encoded as a string of

hexadecimal characters.";

mandatory true;

}

}

case TNGF{

leaf TNGFID{

type string{

pattern '^[A-Fa-f0-9]+$';

}

description "This represents the identifier of

the TNGF ID as specified in clause

9.3.1.161 of 3GPP TS 38.413 in hexadecimal

representation.";

mandatory true;

}

}

case TWIF{

leaf TWIFID{

type string;

description "This represents the TWIF identification

as specified in clause

9.3.1.163 of TS 38.413";

mandatory true;

}

}

case W-AGF{

leaf W-AGFID{

type string{

pattern '^[A-Fa-f0-9]+$';

}

description "This represents the identifier of

the W-AGF ID as specified in

clause 9.3.1.162 of 3GPP TS 38.413

in hexadecimal representation.";

mandatory true;

}

}

}

}

grouping SatelliteBackhaulInfoGrp{

description "Defines information related to satellite backhaul category

and corresponding information of gNB.";

reference "3GPP TS 23.501";

list nTNGlobalRanNodeID{

description "Unique identifier of an NG-RAN node.";

min-elements 1;

max-elements 1;

leaf GlobalRanNodeIDValue{

type string;

description "this relies on the choice of GlobalRanNodeID";

}

key "GlobalRanNodeIDValue";

uses NTNGlobalRanNodeIDGrp;

}

leaf satelliteBackhaulCategory{

description "Satellite backhaul category refers to the type of the

satellite used in the backhaul. Only a single backhaul category

can be indicated.";

reference "3GPP TS 29.571";

type enumeration{

enum GEO;

enum MEO;

enum LEO;

enum OTHER_SAT;

enum DYNAMIC_GEO;

enum DYNAMIC_MEO;

enum DYNAMIC_LEO;

enum DYNAMIC_OTHER_SAT;

enum NON_SATELLITE;

}

mandatory true;

}

leaf geoSatelliteId{

description "Unique identifier of a GEO satellite.";

reference "3GPP TS 29.571";

type string;

//condition present only if the UPF is deployed on the satellite.

}

}

grouping GUAMInfoGrp {

description "Represents the GUAMInfo datatype.";

list pLMNId {

description "This attribute represents a PLMN Identity.";

min-elements 1;

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses types3gpp:PLMNId ;

}

leaf aMFIdentifier  {

type int64;

description "The AMFI is constructed from an AMF Region ID, an

AMF Set ID and an AMF Pointer. The AMF Region ID identifies the region,

the AMF Set ID uniquely identifies the AMF Set within the AMF Region,

and the AMF Pointer uniquely identifies the AMF within the AMF Set.

(Ref. 3GPP TS 23.003)";

}

}

grouping N2InterfaceAmfInfoGrp {

description "Represents the N2InterfaceAmfInfo datatype.";

leaf-list ipv4EndpointAddress {

type inet:ipv4-address;

min-elements 1;

description "This attribute represents available AMF endpoint

IPv4 address(es) for N2.";

}

leaf-list ipv6EndpointAddress {

type inet:ipv6-address;

min-elements 1;

description "This attribute represents available AMF endpoint

IPv6 address(es) for N2.";

}

leaf amfName {

type inet:host-name;

description "This attribute represents AMF Name FQDN as defined in

clause 28.3.2.5 of TS 23.003.";

}

}

grouping BlockedLocationInfoGrp {

description "Repreasents the datatype BlockedLocationInfo";

list blockedLocation {

description "This provides the geographical location at which the PLMN

are not allowed in case of NTN.";

min-elements 1;

max-elements 1;

key "mnc mcc";

uses types3gpp:PLMNId ;

}

list blockedDurWindow {

description "This provides the time durations for which the PLMN are

not allowed at a given location in case of NTN";

key idx;

leaf idx { type uint32 ; }

uses types3gpp:TimeWindowGrp;

}

list blockedSlice {

description "This provides the slice for which the access is

not allowed at a given location in case of NTN. ";

max-elements 1;

key idx;

unique "sst sd";

leaf idx { type uint32; }

uses types5g3gpp:SNssai;

}

}

grouping NTNPLMNRestrictionsInfoGrp {

description "Represents the datatype NTNPLMNRestrictionsInfo";

list pLMNId {

description "This attribute represents a PLMN Identity.";

min-elements 1;

max-elements 1;

key "mnc mcc";

uses types3gpp:PLMNId ;

}

list blockedLocationInfoList {

description "This defines the information related with the location

for which the access restrictions are to be applied in case of NTN.";

key "idx";

leaf idx { type uint32 ; }

uses BlockedLocationInfoGrp;

}

}

grouping NtnLocationInfoGrp {

description  "Represents the NtnLocationInfo datatype.";

list location {

description "This defines the Location (geographical area) under

consideration to which the satellite coverage info belongs";

key idx;

leaf idx { type uint32 ; }

uses types3gpp:GeoAreaGrp;

}

list availabilityWindows  {

description "This attribute defines the list of time windows at which

the satellite coverage will be available for this location.

Either availabilityWindows or nonAvailabilityWindows shall be present.";

key idx;

leaf idx { type uint32 ; }

uses types3gpp:TimeWindowGrp;

}

list nonAvailabilityWindows {

description "This attribute defines the list of time windows at which

the satellite coverage will not be available for this location.

Either availabilityWindows or nonAvailabilityWindows shall be present.";

key idx;

leaf idx { type uint32 ; }

uses types3gpp:TimeWindowGrp;

}

}

grouping SatelliteCoverageInfoGrp {

description "Represents the datatype SatelliteCoverageInfo";

leaf nRSatelliteRATtype {

type enumeration {

enum NRLEO;

enum NRMEO;

enum NRGEO;

enum NROTHERSAT;

}

description "This attribute defines the RAT Type for NR satellite

access.";

}

list locationInfo {

description "This attribute defines the information about location and

corresponding time windows for which the satellite coverage will be

available or unavailable.";

key idx;

leaf idx { type uint32 ; }

uses NtnLocationInfoGrp;

}

}

grouping SliceExpiryInfoGrp {

description "Represents the datatype SliceExpiryInfo";

list pLMNInfo {

min-elements 1;  // ???

config false;

description "It defines the PLMN(s) of a Network Function. ";

key idx;

leaf idx { type uint32 ; }

uses types5g3gpp:PLMNInfo;

}

leaf expiryTime {

type yang:date-and-time;

config false;

description "This attribute provides information about the time at

which the slice is scheduled to be expired as it is not required

anymore.

This attribute will be set based on the sliceAvailability coming as

part of ServiceProfile.";

}

}

grouping MappedCellIdInfoGrp {

description "Represents the datatype MappedCellIdInfo";

list ntnGeoArea {

description "This attribute indicates a specific geographical location

mapped to Mapped Cell ID(s).";

min-elements 1;

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses types3gpp:GeoAreaGrp;

}

list mappedCellId {

description "This attribute is in format of NCGI to indicate a fixed

geographical area (See subclause 16.14.5 in TS 38.300";

min-elements 1;

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses types5g3gpp:NcgiGrp;

}

}

grouping AmfInfoGrp {

description "Represents the data type AmfInfo.";

leaf amfRegionId {

type int64;

mandatory true;

description "It represents the AMF Region ID, which identifies the

region. AllowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 ";

}

leaf amfSetId {

type int64;

mandatory true;

description "It represents the AMF Set ID, which is uniquely identifies

the AMF Set within the AMF Region.

allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003";

}

list taiList {

min-elements 1;

description "The list of TAIs.";

key "idx";

leaf idx { type uint32 ; }

uses types3gpp:TaiGrp;

}

list taiRangeList {

min-elements 1;

description "The range of TAIs.";

key "idx";

leaf idx { type uint32 ; }

uses types5g3gpp:TaiRangeGrp;

}

list gUAMIdList {

description "List of supported Globally Unique AMF Ids (GUAMIs).";

config false;

min-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses GUAMInfoGrp;

}

list backupInfoAmfFailure {

description "List of GUAMIs for which the AMF acts as a backup for AMF

failure.";

min-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses GUAMInfoGrp;

}

list backupInfoAmfRemoval {

description "List of GUAMIs for which the AMF acts as a backup for

planned AMF removal.";

min-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses GUAMInfoGrp;

}

list n2InterfaceAmfInfo {

description "This attribute represents the N2 interface information of

the AMF. ";

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses N2InterfaceAmfInfoGrp;

}

leaf amfOnboardingCapability {

type boolean;

default false;

description "This attribute indicates the AMF supports SNPN Onboarding

capability. This is used for the case of Onboarding of UEs for SNPNs

(see TS 23.501, clause 5.30.2.10).

- FALSE: AMF does not support SNPN Onboarding;

- TRUE: AMF supports SNPN Onboarding.";

}

leaf highLatencyCom {

type boolean;

description "This attribute indicates whether the AMF supports High

Latency communication (e.g. for NR RedCap UE). This is used for CP NF

to discover AMF supporting High Latency communication (see TS 23.501,

clause 6.3.5).

- FALSE: AMF does not support High Latency communication e.g. for

NR RedCap UE;

- TRUE: AMF supports High Latency communication e.g. for NR RedCap UE;";

}

}

grouping AMFFunctionGrp {

description "Represents the AMFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNInfoList {

min-elements 1;

description "It defines the PLMN(s) of a Network Function. ";

key idx;

leaf idx { type uint32 ; }

uses types5g3gpp:PLMNInfo;

}

container aMFIdentifier {

description "An AMF identifier, comprising an AMF Region ID, an

AMF Set ID and an AMF Pointer.";

uses types3gpp:AmfIdentifier;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

leaf-list cNSIIdList {

type string;

description "It is a set of NSI ID. NSI ID is an identifier for

identifying the Core Network part of a Network Slice instance when

multiple Network Slice instances of the same Network Slice are deployed,

and there is a need to differentiate between them in the 5GC.

See NSI ID definition in clause 3.1 of TS 23.501 and

subclause 6.1.6.2.7 of  TS 29.531";

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

description "";

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

list amfInfo {

max-elements 1;

description "This attribute represents information of an AMF

NF Instance.";

key "idx";

leaf idx { type uint32 ; }

uses AmfInfoGrp;

}

list nTNPLMNRestrictionsList {

description "This attribute defines the location restrictions per PLMN

that relates to non-terrestrial network access.";

key "idx";

leaf idx { type uint32 ; }

uses NTNPLMNRestrictionsInfoGrp;

}

list satelliteCoverageInfoList {

description "This attribute defines the information related to NR

Satellite RAT type and corresponding information of satellite coverage";

key "idx";

leaf idx { type uint32 ; }

uses SatelliteCoverageInfoGrp;

}

list sliceExpiryInfo {

description "This provides information related to a network slice

validity.";

key "idx";

leaf idx { type uint32 ; }

uses SliceExpiryInfoGrp;

}

list satelliteBackhaulInfoList{

description "Specifies a list of satellite backhaul information. It can be

used by NF and NF services.";

leaf GlobalRanNodeIDValue{

type string;

description "this relies on the choice of GlobalRanNodeID";

}

key "GlobalRanNodeIDValue";

min-elements 1;

uses SatelliteBackhaulInfoGrp;

}

leaf-list mdtUserConsentReqList {

type enumeration {

enum M1;

enum M2;

enum M3;

enum M4;

enum M5;

enum M6;

enum M7;

enum M8;

enum M9;

enum MDT_UE_LOCATION;

}

description "represents a list of MDT measurement names that are

subject to user consent at MDT activation.

Any MDT measurement, whose name is not specified in this list, is not

subject to user consent at MDT activation.";

}

list mappedCellIdInfoList {

description "It provides the list of mapping between GEO area and

Mapped Cell ID.";

key "idx";

leaf idx { type uint32 ; }

uses MappedCellIdInfoGrp;

}

leaf aMFSetRef {

type types3gpp:DistinguishedName;

description "This is the DN of AMFSet";

}

}

augment "/me3gpp:ManagedElement" {

list AMFFunction {

description "This IOC represents the AMF functionality in 5GC.

For more information about the AMF, see 3GPP TS 23.501.

The attribute sliceExpiryInfo is used when the validity information of

a network slice need to be configured. The attribute

sliceExpiryInfo.pLMNInfo indicates the network slice to which the

validity information applies.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 2 ***

*** START OF CHANGE 3 ***

*** yang-models/_3gpp-5gc-nrm-dynamic5qiset.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-dynamic5qiset {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-dynamic5qiset;

prefix dyn5QIs3gpp;

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbdufunction { prefix gnbdu3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import  _3gpp-nr-nrm-gnbcuupfunction { prefix gnbcuup3gpp; }

import _3gpp-5gc-nrm-configurable5qiset { prefix Conf5QIs3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the dynamic 5QIs including their QoS

characteristics.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-03-25 { reference CR-1489 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2022-07-28 { reference "CR-0770"; }

revision 2022-01-07 { reference CR-0643; }

revision 2020-10-01 { reference "CR-0393"; }

revision 2020-08-06 { reference "CR-0333"; }

grouping Dynamic5QISetGrp {

description "Represents the Dynamic5QISet IOC.";

list dynamic5QIs {

description "This IOC specifies the dynamically assigned 5QIs

including their QoS characteristics, see 3GPP TS 23.501.

The instance of this IOC shall not be created or modified by the

MnS consumer except for the instance associated to

PCFFunction MOI or SMFFunction MOI when the PCF is not deployed.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses Conf5QIs3gpp:FiveQICharacteristicsGrp;

}

}

}

grouping Dynamic5QISetSubtree {

description "Helps augmenting Dynamic5QISet into multiple places.";

list Dynamic5QISet {

description "Specifies the dynamic 5QIs including their QoS

characteristics, see 3GPP TS 23.501.";

key "id";

uses top3gpp:Top_Grp;

container attributes {

uses Dynamic5QISetGrp;

}

uses Conf5QIs3gpp:FiveQICharacteristicsSubtree;

}

}

feature Dynamic5QISetUnderSubNetwork {

description "Dynamic5QISet shall be contained under SubNetwork.";

}

augment "/subnet3gpp:SubNetwork" {

if-feature Dynamic5QISetUnderSubNetwork;

uses Dynamic5QISetSubtree;

}

feature Dynamic5QISetUnderManagedElement {

description "Dynamic5QISet shall be contained under ManagedElement.";

}

augment "/me3gpp:ManagedElement" {

if-feature Dynamic5QISetUnderManagedElement;

uses Dynamic5QISetSubtree;

}

feature Dynamic5QISetUnderGNBDUFunction {

description "Dynamic5QISet shall be contained under GNBDUFunction.";

}

augment "/me3gpp:ManagedElement/gnbdu3gpp:GNBDUFunction" {

if-feature Dynamic5QISetUnderGNBDUFunction;

uses Dynamic5QISetSubtree;

}

feature Dynamic5QISetUnderGNBCUCPFunction {

description "Dynamic5QISet shall be contained under GNBCUCPFunction.";

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction" {

if-feature Dynamic5QISetUnderGNBCUCPFunction;

uses Dynamic5QISetSubtree;

}

feature Dynamic5QISetUnderGNBCUUPFunction {

description "Dynamic5QISet shall be contained under GNBCUUPFunction.";

}

augment "/me3gpp:ManagedElement/gnbcuup3gpp:GNBCUUPFunction" {

if-feature Dynamic5QISetUnderGNBCUUPFunction;

uses Dynamic5QISetSubtree;

}

}

<CODE ENDS>

*** END OF CHANGE 3 ***

*** START OF CHANGE 4 ***

*** yang-models/_3gpp-5gc-nrm-gmlcfunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-gmlcfunction {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-5gc-nrm-gmlcfunction";

prefix gmlc3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This module defines the GMLC Function IOC per 3GPP TS 28.541.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-03-25 { reference CR-1489 ; }

revision 2025-03-25 {

description "Initial revision.";

reference "3GPP TS 28.541";

}

grouping GmlcInfoGrp {

description "Information of a GMLC NF Instance (see TS 29.510, clause 6.1.6.2.41).";

leaf servingClientTypes {

description "Identity of the GMLC group that is served by the GMLC instance.";

type enumeration {

enum EMERGENCY_SERVICES {

description "External client for emergency services";

}

enum VALUE_ADDED_SERVICES {

description "External client for value added services";

}

enum PLMN_OPERATOR_SERVICES {

description "External client for PLMN operator services";

}

enum LAWFUL_INTERCEPT_SERVICES {

description "External client for Lawful Intercept services";

}

enum EXTERNAL_CLIENT_FOR_LAWFUL_INTERCEPT_SERVICES {

description "External client for PLMN Operator Broadcast services";

}

enum PLMN_OPERATOR_OM {

description "External client for PLMN Operator O&M";

}

enum PLMN_OPERATOR_ANONYMOUS_STATISTICS {

description "External client for PLMN Operator anonymous statistics";

}

enum PLMN_OPERATOR_TARGET_MS_SERVICE_SUPPORT {

description "External client for PLMN Operator target MS service support";

}

}

}

leaf gmlcNumbers{

type string{

pattern "^[0-9]{5,15}$";

}

description "This attribute represents each item of the array

shall carry an OctetString indicating the ISDN number of the GMLC in

international number format as described in ITU-T Rec. E.164 [94] and

shall be encoded as a TBCD-string.";

}

}

grouping GMLCFunctionGrp {

description "GMLC Function IOC grouping.";

uses mf3gpp:ManagedFunctionGrp;

list pLMNInfoList {

key "idx";

leaf idx { type uint32; }

min-elements 1;

uses types3gpp:PLMNId;

uses types5g3gpp:SNssai;

}

leaf sBIFqdn {

type string;

description "FQDN of the registered NF instance in the service-based interface.";

}

list managedNFProfile {

description "This parameter defines profile for managed NF. See TS 23.501";

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

key "groupId";

min-elements 1;

uses types5g3gpp:CommModel;

}

list gmlcInfo {

key "idx";

leaf idx { type uint32; }

max-elements 1;

uses GmlcInfoGrp;

}

}

augment "/me3gpp:ManagedElement" {

list GMLCFunction {

description "5G Core LMF Function defined in TS 23.501";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses GMLCFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 4 ***

*** START OF CHANGE 5 ***

*** yang-models/_3gpp-5gc-nrm-lmffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-lmffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-lmffunction;

prefix lmf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-ntnfunction { prefix ntn3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the LMF function defined in 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-03-25 { reference CR-1489 ; }

revision 2024-11-01 { reference CR-1405 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-25 { reference "S5-194457 S5193518"; }

revision 2019-05-15 { reference "initial revision"; }

grouping LmfInfoGrp {

description "Represents the LmfInfo datatype.";

leaf-list servingClientTypes {

type enumeration {

enum EMERGENCY_SERVICES {

description "External client for emergency services";

}

enum VALUE_ADDED_SERVICES {

description "External client for value added services";

}

enum PLMN_OPERATOR_SERVICES {

description "External client for PLMN operator services";

}

enum LAWFUL_INTERCEPT_SERVICES {

description "External client for Lawful Intercept services";

}

enum PLMN_OPERATOR_BROADCAST_SERVICES {

description "External client for PLMN Operator Broadcast services";

}

enum PLMN_OPERATOR_OM {

description "External client for PLMN Operator O&M";

}

enum PLMN_OPERATOR_ANONYMOUS_STATISTICS {

description "External client for PLMN Operator anonymous statistics";

}

enum PLMN_OPERATOR_TARGET_MS_SERVICE_SUPPORT {

description "External client for PLMN Operator target MS service

support";

}

}

description "This attribute represents a list of external client type(s),

e.g. emergency client. The NRF should only include this LMF instance

to NF discovery with 'client-type' query parameter indicating one of

the external client types in the list.

Absence of this attribute means the LMF is not dedicated to serve

specific client types.";

}

leaf lmfId {

type string;

description "This attribute represents the LMF identification.

See clause 6.1.6.3.6 TS 29.572";

}

leaf-list servingAccessTypes {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

min-elements 1;

description "This attribute contains the access type (3GPP_ACCESS and/or

NON_3GPP_ACCESS) supported by the SMF.

If not included, it shall be assumed the both access types are

supported.";

}

leaf-list servingAnNodeTypes {

type enumeration {

enum GNB;

enum NG;

enum ENB;

}

min-elements 1;

description "This attribute contains the AN node type (i.e. gNB or NG-eNB)

supported by the LMF.

If not included, it shall be assumed that all AN node types are

supported.";

}

leaf-list servingRatTypes {

type string;

min-elements 1;

description "This attribute contains the RAT type (e.g. 5G NR, eLTE or

any of the RAT Types specified for NR satellite access) supported by

the LMF.

If not included, it shall be assumed that all RAT types are supported

allowedValues: see clause 5.4.3.2 of TS 29.571";

}

list taiList {

description "This attribute contains TAI list that the LMF can serve.

It may contain one or more non-3GPP access TAIs.

The absence of both this attribute and the taiRangeList attribute

indicates that the LMF can be selected for any TAI in the serving

network.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses types3gpp:TaiGrp;

}

list taiRangeList {

description "This attribute contains TAI range list that the LMF can

serve. It may contain one or more non-3GPP access TAI ranges.

The absence of both this attribute and the taiList attribute indicates

that the LMF can be selected for any TAI in the serving network.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses types5g3gpp:TaiRangeGrp;

}

leaf-list supportedGADShapes {

type enumeration {

enum POINT {

description "indicates Ellipsoid Point";

}

enum POINT_UNCERTAINTY_CIRCLE {

description "indicates Ellipsoid point with uncertainty circle";

}

enum POINT_UNCERTAINTY_ELLIPSE {

description "indicates  Ellipsoid point with uncertainty ellipse";

}

enum POLYGON {

description "indicates Polygon";

}

enum POINT_ALTITUDE {

description "indicates Ellipsoid point with altitude";

}

enum POINT_ALTITUDE_UNCERTAINTY {

description "indicates  Ellipsoid point with altitude and

uncertainty ellipsoid";

}

enum ELLIPSOID_ARC {

description "indicates Ellipsoid Arc";

}

enum LOCAL_2D_POINT_UNCERTAINTY_ELLIPSE {

description "indicates Local 2D point with uncertainty ellipse";

}

enum LOCAL_3D_POINT_UNCERTAINTY_ELLIPSOID {

description "indicates  Local 3D point with uncertainty ellipsoid";

}

}

min-elements 1;

description "This attribute contains the GAD shapes supported by the LMF.

If not included, it doesn't indicate that the LMF doesn't support any

GAD shapes.

The allowedValues are: see clause 6.1.6.3.4 of TS 29.572 ";

}

}

grouping TrpMappingInfoGrp {

description "This data type represents the mapping relationship between

satellite and TRPs.";

leaf satelliteId {

type string {

pattern  '[0-9]{5}';

}

mandatory true;

description "This attribute indicates satellite Id. It shall be formatted

as a fixed 5-digit string, padding with leading digits '0' to complete

a 5-digit length.";

}

leaf-list trpIds {

type uint8 {

range 1..max;

}

description "This attribute indicates TRPs uniquely within an NG-RAN

node (see TS 38.455 clause 9.2.24). A gNB may serve several TRPs.

For NTN, a TRP may be located on board the satellite.";

}

}

grouping TrpInfoGrp {

description "This data type represents the mapping relationship between

gNB and TRPs.";

leaf gNBId {

type uint32;

mandatory true;

description "It identifies a gNB within a PLMN. The gNB ID is part of

the NR Cell Identifier (NCI) of the gNB cells.

See 'gNB Identifier (gNB ID)' of subclause 8.2 of TS 38.300. See

'Global gNB ID' in subclause 9.3.1.6 of TS 38.413";

}

list trpMappingInfoList {

description "This is the list of TRP mapping between satellite and TRPs.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses TrpMappingInfoGrp;

}

}

grouping LMFFunctionGrp {

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list managedNFProfile {

description "This parameter defines profile for managed NF. See TS 23.501";

key idx;

min-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

description "The attribute specifies a list of commModel. It can be used

by NF and NF services to interact with each other in 5G Core network

(see TS 23.501)";

min-elements 1;

key "groupId";

uses types5g3gpp:CommModel;

}

list lmfInfo {

description "This attribute represents information of an LMF NF Instance";

max-elements 1;

key idx;

leaf idx { type uint32; }

uses LmfInfoGrp;

}

list mappedCellIdInfoList {

description "It provides the list of mapping between GEO area and

Mapped Cell ID.";

key idx;

leaf idx { type uint32; }

uses types5g3gpp:MappedCellIdInfoGrp;

}

list ephemerisInfos {

description "This is the list of Ephemeris related information";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses ntn3gpp:EphemerisGrp;

}

list trpInfoList {

description "This is the list of TRP (Transmission-Reception Point)

related information on LMF (see TS 38.305 clause 5.4.4).";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses TrpInfoGrp;

}

}

augment "/me3gpp:ManagedElement" {

list LMFFunction {

description "5G Core LMF Function defined in TS 23.501";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses LMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 5 ***

*** START OF CHANGE 6 ***

*** yang-models/_3gpp-5gc-nrm-neffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-neffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-neffunction;

prefix nef3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-5gc-nrm-nfprofile { prefix nfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NEF function in 5GC. For more

information about the NEF, see 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-03-25 { reference CR-1489 ; }

revision 2025-01-25 { reference CR-1442 ; }

revision 2024-05-18 { reference CR-1273 ; }

revision 2024-04-12 { reference CR-1218 ; }

revision 2023-09-18 { reference CR-1103 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916 ; }

revision 2022-01-07 { reference CR-0643; }

revision 2020-11-06 { reference CR-0412 ; }

revision 2019-10-20 { reference "initial revision"; }

grouping DnnInfoItemGrp {

description "This data type represents set of parameters supported by NF

for a given S-NSSAI. (See clause 6.1.6.2.97 of TS 29.510)";

leaf dnn {

type string;

mandatory true;

description "It represents supported DNN or Wildcard DNN if the NF

supports all DNNs for the related S-NSSAI. The DNN shall contain the

Network Identifier and it may additionally contain an Operator

Identifier. If the Operator Identifier is not included, the DNN is

supported for all the PLMNs in the plmnList of the NF Profile.";

}

}

grouping SnssaiInfoItemGrp {

description "This data type represents set of parameters supported by NF

for a given S-NSSAI. (See clause 6.1.6.2.97 of TS 29.510)";

list sNssai {

description "It represents the S-NSSAI the NetworkSlice managed object

is supporting. The S-NSSAI is defined in TS 23.003.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses types5g3gpp:SNssai;

}

list dnnInfoList{

description "It represents list of parameters supported by the

NF per DNN.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses DnnInfoItemGrp;

}

}

grouping AfEventExposureDataGrp {

description "This data type represents the AF Event Exposure data managed

by a given NEF Instance. (See clause 6.1.6.2.50 TS 29.510";

leaf-list afEvents {  // stage 2 double defined

type string;

min-elements 1;

description "It represents AF Event(s) exposed by the NEF after

registration of the AF(s) at the NEF.";

}

leaf-list afIds {

type string;

min-elements 1;

description "It represents list of application function identifiers of

the managed PFDs.";

}

leaf-list appIds {  // stage 2 shaky

type string;

min-elements 1;

description "It represents list of internal application identifiers.";

}

}

grouping PfdDataGrp {

description "This data type represents the list of Application IDs

and/or AF IDs managed by a given NEF Instance.

(See clause 6.1.6.2.49 TS 29.510)";

leaf-list appIds {  // stage 2 shaky

type string;

min-elements 1;

description "It represents list of internal application identifiers.";

}

leaf-list afIds {

type string;

min-elements 1;

description "It represents list of application function identifiers of

the managed PFDs.";

}

}

grouping UnTrustAfInfoGrp {

description "This data type represents information of an untrusted AF

Instance. (See clause 6.1.6.2.95 TS 29.510)";

leaf afId {

type string;

mandatory true;

description "It represents associated AF id.";

}

list sNssaiInfoList {

description "It represents S-NSSAIs and DNNs supported by the AF.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses SnssaiInfoItemGrp;

}

leaf mappingInd {

type boolean;

yext3gpp:initial-value false;

description "When present, this attribute indicates whether the

AF supports mapping between UE IP address (IPv4 address or IPv6 prefix)

and UE ID (i.e. GPSI).

True: the AF supports mapping between UE IP address and UE ID;

False: the AF does not support mapping between UE IP address

and UE ID.";

}

}

grouping NefInfoGrp {

description "This data type represents information of an NEF Instance.

(See clause 6.1.6.2.48 TS 29.510).";

list taiList {

config false;

description "List of TAIs";

yext3gpp:inVariant;

yext3gpp:notNotifyable;

key idx;

leaf idx { type uint32 ; }

uses types3gpp:TaiGrp;

}

list taiRangeList {

config false;

description "Range of TAIs";

yext3gpp:inVariant;

yext3gpp:notNotifyable;

key idx;

leaf idx { type uint32 ; }

uses types5g3gpp:TaiRangeGrp;

}

leaf nefId {

type string;

mandatory true;

description "It represents the NEF ID.

(see clause 6.1.6.3.2 of TS 29.510)";

}

list pfdData {

config false;

description "It represents PFD data, containing the list of internal

application identifiers and/or the list of application function

identifiers for which the PFDs can be provided.

Absence of this attribute indicates that the PFDs for any internal

application identifier and for any application function identifier

can be provided.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses PfdDataGrp;

}

list afEeData {

config false;

description "It represents the AF provided event exposure data. The NEF

registers such information in the NRF on behalf of the AF.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses AfEventExposureDataGrp;

}

list gpsiRanges {

description "It represents list of ranges of GPSIs whose profile data

is available.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses nfp3gpp:IdentityRange;

}

list externalGroupIdentifiersRanges {

description "It represents list of ranges of external groups whose

profile data is available.";  // stage 2 definition is shaky

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses nfp3gpp:IdentityRange;

}

leaf-list servedFqdnList {

type string;

min-elements 1;

description "It represents pattern (regular expression according to

the ECMA-262 dialect [75]) representing the Domain names served by

the NEF.";

}

leaf-list dnaiList {

type string;

min-elements 1;

description "It represents list of Data network access identifiers

supported by the NEF. The absence of this attribute indicates that

the NEF can be selected for any DNAI.";

}

list unTrustAfInfoList {

description "It represents list of information corresponding to the AFs.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses UnTrustAfInfoGrp;

}

leaf uasNfFunctionalityInd {

type boolean;

yext3gpp:initial-value false;

description "When present, this attribute shall indicate whether the ,

NEF supports UAS NF functionality:

- True: UAS NF functionality is supported by the NEF.

- False (default): UAS NF functionality is not supported by the NEF";

}

leaf multiMemAfSessQosInd {

type boolean;

default false;

description "This attribute indicates whether the NEF supports

Multi-member AF session with required QoS functionality:

- TRUE: Multi-member AF session with required QoS functionality is

supported by the NEF

- FALSE (default): Multi-member AF session with required QoS

functionality is not supported by the NEF.";

}

leaf memberUESelAssistInd {

type boolean;

default false;

description "This attribute indicates whether the NEF supports member

UE selection assistance functionality:

- TRUE: member UE selection assistance functionality is supported by

the NEF

- FALSE (default): member UE selection assistance functionality is

not supported by the NEF.";

}

}

grouping NEFFunctionGrp {

description "Represents the NEFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

key "sd sst";

uses types5g3gpp:SNssai;

}

list managedNFProfile {

description "This parameter defines profile for managed NF(See TS 23.501)";

min-elements 1;

max-elements 1;

key idx;

uses types3gpp:ManagedNFProfile;

}

leaf-list capabilityList {

description "List of supported capabilities of the NEF.";

reference "3GPP TS 23.003";

type string;

}

leaf isCAPIFSup {

type boolean;

yext3gpp:inVariant;

}

list nefInfo {

description "This attribute represents information of an NEF NF Instance.";

key idx;

leaf idx { type uint32 ; }

uses NefInfoGrp;

}

}

augment "/me3gpp:ManagedElement" {

list NEFFunction {

description "5G Core NEF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NEFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 6 ***

*** START OF CHANGE 7 ***

*** yang-models/_3gpp-5gc-nrm-nfprofile.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-nfprofile {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nfprofile;

prefix nfp3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-inet-types { prefix inet; }

import ietf-yang-types { prefix yang; }

import _3gpp-5gc-nrm-nfservice { prefix nfs3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-5gc-nrm-nwdaffunction { prefix nwdaf3gpp; }

import _3gpp-5gc-nrm-lmffunction {prefix lmf3gpp; }

import _3gpp-5gc-nrm-gmlcfunction {prefix gmlc3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "NF profile class.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 29.510";

revision 2025-03-25 { reference CR-1489 ; }

revision 2025-01-25 { reference CR-1442 ; }

revision 2024-04-12 { reference CR-1218 ; }

revision 2023-11-18 { reference CR-1103 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-02-14 { reference CR-0891; }

revision 2019-06-17 { reference "initial revision"; }

grouping NFProfileGrp {

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

mandatory true;

type string;

}

leaf nfType {

description "Type of Network Function.";

mandatory true;

type types3gpp:NfType;

}

leaf nfStatus {

description "Status of the NF Instance.";

mandatory true;

type NFStatus;

}

leaf heartBeatTimer {

description "Time in seconds expected between 2 consecutive

heart-beat messages from an NF Instance to the NRF.

It may be included in the registration request.

When present in the request it shall contain the

heartbeat time proposed by the NF service consumer.";

type uint16;

}

list plmnList {

description "PLMN(s) of the Network Function.

This IE shall be present if this information

is available for the NF. If not provided, PLMN ID(s)

of the PLMN of the NRF are assumed for the NF.";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNssais {

description "S-NSSAIs of the Network Function. If not

provided, the NF can serve any S-NSSAI. When present

this IE represents the list of S-NSSAIs supported in

all the PLMNs listed in the plmnList IE.";

min-elements 1;

key "sst sd";

uses Snssai;

}

list perPlmnSnssaiList {

description "This IE may be included when the list of

S-NSSAIs supported by the NF for each PLMN it is supporting

is different. When present, this IE shall include the S-NSSAIs

supported by the Network Function for each PLMN supported by

the Network Function. When present, this IE shall

override sNssais IE.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses PlmnSnssai;

}

leaf-list nsiList {

description "NSI identities of the Network Function.

If not provided, the NF can serve any NSI.";

min-elements 1;

type string;

}

leaf fqdn {

description "FQDN of the Network Function. For AMF, the

FQDN registered with the NRF shall be that of the AMF Name.";

type inet:domain-name;

}

leaf interPlmnFqdn {

description "If the NF needs to be discoverable by other

NFs in a different PLMN, then an FQDN that is used

for inter-PLMN routing is specified.";

type inet:domain-name;

}

leaf-list ipv4Addresses {

description "IPv4 address(es) of the Network Function.";

min-elements 1;

type inet:ipv4-address;

}

leaf-list ipv6Addresses {

description "IPv6 address(es) of the Network Function.";

min-elements 1;

type inet:ipv6-address;

}

list allowedPlmns {

description "PLMNs allowed to access the NF instance.

If not provided, any PLMN is allowed to access the NF.";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list allowedNfTypes {

description "Type of the NFs allowed to access the NF instance.

If not provided, any NF type is allowed to access the NF.";

min-elements 1;

type types3gpp:NfType;

}

leaf-list allowedNfDomains {

description "Pattern representing the NF domain names allowed

to access the NF instance. If not provided,

any NF domain is allowed to access the NF.";

min-elements 1;

type string;

}

list allowedNssais {

description "S-NSSAI of the allowed slices to access the NF instance.

If not provided, any slice is allowed to access the NF.";

min-elements 1;

key "sst sd";

uses Snssai;

}

leaf priority {

description "Priority (relative to other NFs of the same type)

in the range of 0-65535, to be used for NF selection;

lower values indicate a higher priority.

If priority is also present in the nfServiceList

parameters,

those will have precedence over this value.

The NRF may overwrite the received priority value

when exposing

an NFProfile with the Nnrf_NFDiscovery service.";

type uint16;

}

leaf capacity {

description "Static capacity information in the range of 0-65535,

expressed as a weight relative to other NF instances of

the same type; if capacity is also present in the nfServiceList

parameters, those will have precedence over this value.";

type uint16;

}

leaf load {

description "Dynamic load information, ranged from 0 to 100,

indicates the current load percentage of the NF.";

type types3gpp:Load;

}

leaf locality {

description "Operator defined information about the location

of the NF instance (e.g. geographic location, data center).";

type string;

}

grouping udrInfo {

leaf groupId {

description "Identity of the UDR group that is served

by the UDR instance.

If not provided, the UDR instance does not pertain

to any UDR group.";

type string;

}

list supiRanges {

description "List of ranges of SUPI's whose profile data

is available in the UDR instance.";

key "start end pattern";

min-elements 1;

uses SupiRange;

}

list gpsiRanges {

description "List of ranges of GPSIs whose profile data is

available in the UDR instance.";

key "start end pattern";

min-elements 1;

uses IdentityRange;

}

list externalGroupIdentifiersRanges {

description "List of ranges of external groups whose profile

data is available in the UDR instance.";

key "start end pattern";

min-elements 1;

uses IdentityRange;

}

leaf-list supportedDataSets {

description "List of supported data sets in the UDR instance.

If not provided, the UDR supports all data sets.";

min-elements 1;

type DataSetId;

}

}

grouping udmInfo {

leaf groupId {

description "Identity of the UDM group that is served by the

UDM instance. If not provided, the UDM instance does

not pertain to any UDM group.";

type string;

}

list supiRanges {

description "List of ranges of SUPI's whose profile data is

available in the UDM instance.";

key "start end pattern";

min-elements 1;

uses SupiRange;

}

list gpsiRanges {

description "List of ranges of GPSIs whose profile data is

available in the UDM instance.";

key "start end pattern";

min-elements 1;

uses IdentityRange;

}

list externalGroupIdentifiersRanges {

description "List of ranges of external groups whose profile

data is available in the UDM instance.";

key "start end pattern";

min-elements 1;

uses IdentityRange;

}

leaf-list routingIndicators {

description "List of Routing Indicator information that allows

to route network signalling with SUCI

to the UDM instance. If not provided,

the UDM can serve any Routing Indicator.

Pattern: '^[0-9]{1,4}$'.";

min-elements 1;

type string;

}

}

grouping ausfInfo {

leaf groupId {

description "Identity of the AUSF group. If not provided,

the AUSF instance does not pertain to any AUSF group.";

type string;

}

list supiRanges {

description "List of ranges of SUPIs that can be served by

the AUSF instance. If not provided, the AUSF can serve any SUPI.";

key "start end pattern";

min-elements 1;

uses SupiRange;

}

leaf-list routingIndicators {

description "List of Routing Indicator information that allows

to route network signalling with SUCI

to the AUSF instance. If not provided,

the AUSF can serve any Routing Indicator.

Pattern: '^[0-9]{1,4}$'.";

min-elements 1;

type string;

}

}

grouping amfInfo {

leaf amfRegionId {

description "AMF region identifier";

type string;

}

leaf amfSetId {

description "AMF set identifier";

type string;

}

list guamiList {

description "List of supported GUAMIs.";

key idx;

leaf idx { type uint32; }

min-elements 1;

uses Guami;

}

list taiList {

description "The list of TAIs the AMF can serve.

It may contain the non-3GPP access TAI.

The absence of this attribute and

the taiRangeList attribute indicate that

the AMF can be selected for any TAI in the

serving network.";

key idx;

leaf idx { type uint32; }

min-elements 1;

uses types3gpp:TaiGrp;

}

list taiRangeList {

description "The range of TAIs the AMF can serve.

The absence of this attribute and the taiList

attribute indicate that the AMF can be selected

for any TAI in the serving network.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses types5g3gpp:TaiRangeGrp;

}

list backupInfoAmfFailure {

description "List of GUAMIs for which the AMF acts

as a backup for AMF failure.";

key idx;

leaf idx { type uint32; }

min-elements 1;

uses Guami;

}

list backupInfoAmfRemoval {

description "List of GUAMIs for which the AMF acts

as a backup for planned AMF removal.";

key idx;

leaf idx { type uint32; }

min-elements 1;

uses Guami;

}

list n2InterfaceAmfInfo {

description "N2 interface information of the AMF.

This information needs not be sent in NF Discovery responses.

It may be used by the NRF to update the DNS for

AMF discovery by the 5G Access Network.";

max-elements 1;

key idx;

leaf idx { type uint32; }

uses N2InterfaceAmfInfo;

}

}

grouping smfInfo {

list sNssaiSmfInfoList {

description "List of parameters supported by the SMF per S-NSSAI.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses sNssaiSmfInfoItem;

}

list taiList {

description "The list of TAIs the SMF can serve.

It may contain the non-3GPP access TAI.

The absence of this attribute and the taiRangeList

attribute indicate that

the SMF can be selected for any TAI

in the serving network.";

key idx;

leaf idx { type uint32; }

min-elements 1;

uses types3gpp:TaiGrp;

}

list taiRangeList {

description "The range of TAIs the SMF can serve.

The absence of this attribute and the taiList

attribute indicate that the SMF can be selected

for any TAI in the serving network.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses types5g3gpp:TaiRangeGrp;

}

leaf pgwFqdn {

description "The FQDN of the PGW if the SMF is a combined SMF/PGW-C.";

type inet:domain-name;

}

leaf-list accessType {

description "If included, this IE shall contain the access type

(3GPP_ACCESS and/or NON_3GPP_ACCESS) supported by the SMF.

If not included, it shall be assumed the both

access types are supported.";

min-elements 1;

max-elements 2;

type AccessType;

}

}

grouping upfInfo {

list sNssaiUpfInfoList {

description "List of parameters supported by the UPF per S-NSSAI.";

min-elements 1;

key idx;

leaf idx { type uint32; }

uses SnssaiUpfInfoItem;

}

leaf-list smfServingArea {

description "The SMF service area(s) the UPF can serve.

If not provided, the UPF can serve any

SMF service area.";

min-elements 1;

type string;

}

list interfaceUpfInfo {

description "List of User Plane interfaces configured on the UPF.

When this IE is provided in the NF Discovery response,

the NF Service Consumer (e.g. SMF) may use

this information for UPF selection.";

key idx;

leaf idx { type uint32; }

min-elements 1;

uses InterfaceUpfInfoItem;

}

leaf iwkEpsInd {

description "Indicates whether interworking with EPS is

supported by the UPF.

true: Supported

false (default): Not Supported";

type boolean;

}

leaf-list pduSessionTypes {

description "List of PDU session type(s) supported by the UPF.

The absence of this attribute indicates that the UPF can be selected

for any PDU session type.";

min-elements 1;

type PduSessionType;

}

}

grouping ProSeCapability {

leaf proseDirectDiscovery {

type boolean;

description "Indicates support for ProSe Direct Discovery.";

}

leaf proseDirectCommunication {

type boolean;

description "Indicates support for ProSe Direct Communication.";

}

leaf proseL2UetoNetworkRelay {

type boolean;

description "Indicates support for ProSe Layer-2

UE-to-Network Relay..";

}

leaf proseL3UetoNetworkRelay {

type boolean;

description "Indicates support for ProSe Layer-3

UE-to-Network Relay.";

}

leaf proseL2RemoteUe {

type boolean;

description "Indicates support for ProSe Layer-2

Remote UE.";

}

leaf proseL3RemoteUe {

type boolean;

description "Indicates support for ProSe Layer-3

Remote UE.";

}

leaf proseL2UetoUeRelay {

type boolean;

description "Indicates support for ProSe Layer-2

UE-to-UE Relay.";

}

leaf proseL3UetoUeRelay {

type boolean;

description "Indicates support for ProSe Layer-3

UE-to-UE Relay.";

}

leaf proseL2EndUe {

type boolean;

description "Indicates support for ProSe Layer-2 End UE.";

}

leaf proseL3EndUe {

type boolean;

description "Indicates support for ProSe Layer-3 End UE.";

}

leaf proseL3IntermRelay {

type boolean;

description "Indicates support for ProSe Layer-3

Intermediate Relay.";

}

leaf proseL3MultihopRemote {

type boolean;

description "Indicates support for ProSe Layer-3 Multihop Remote.";

}

leaf proseL3NetMultihopRelay {

type boolean;

description "Indicates support for ProSe Layer-3 Network Multihop

Relay.";

}

leaf proseL3UeMultihopRelay {

type boolean;

description "Indicates support for ProSe Layer-3 UE Multihop Relay.";

}

leaf proseL3EndUeMultihop {

type boolean;

description "Indicates support for ProSe Layer-3 End UE Multihop.";

}

}

grouping pcfInfo {

leaf-list dnnList {

description "DNNs supported by the PCF.

If not provided, the PCF can serve any DNN.";

min-elements 1;

type string;

}

list supiRanges {

description "List of ranges of SUPIs that can be served by

the PCF instance. If not provided, the PCF can serve any SUPI.";

key "start end pattern";

min-elements 1;

uses SupiRange;

}

leaf rxDiamHost {

description "This IE shall be present if the PCF supports Rx interface.

When present, this IE shall indicate the Diameter host

of the Rx interface for the PCF.

Pattern: '^([A-Za-z0-9]+(-[A-Za-z0-9]+).)+[a-z]{2,}$'.";

type string;

}

leaf rxDiamRealm {

description "This IE shall be present if the PCF supports Rx interface.

When present, this IE shall indicate the Diameter realm

of the Rx interface for the PCF.

Pattern: '^([A-Za-z0-9]+(-[A-Za-z0-9]+).)+[a-z]{2,}$'.";

type string;

}

list proseCapability {

description "It indicates the supported ProSe Capability by the PCF.";

uses ProSeCapability;

config false;

}

}

grouping bsfInfo {

list ipv4AddressRanges {

description "List of ranges of IPv4 addresses handled by BSF.

If not provided, the BSF can serve any IPv4 address.";

key "start end";

uses types3gpp:Ipv4AddressRange;

}

leaf-list dnnList {

description "List of DNNs handled by the BSF

If not provided, the BSF can serve any DNN.";

min-elements 1;

type string;

}

leaf-list ipDomainList {

description "List of IPv4 address domains, as described in

subclause 6.2 of 3GPP TS 29.513, handled by the BSF.

If not provided, the BSF can serve any IP domain.";

min-elements 1;

type string;

}

list ipv6PrefixRanges {

description "List of ranges of IPv6 prefixes handled by the BSF.

If not provided, the BSF can serve any IPv6 prefix.";

key "start end";

uses types3gpp:Ipv6PrefixRange;

}

}

grouping chfInfo {

list supiRangeList {

description "List of ranges of SUPIs that can be served by

the CHF instance. If not provided, the CHF can serve any SUPI.";

key "start end pattern";

min-elements 1;

uses SupiRange;

}

list gpsiRangeList {

description "List of ranges of GPSI that can be served

by the CHF instance. If not provided, the CHF can serve any GPSI.";

key "start end pattern";

min-elements 1;

uses IdentityRange;

}

list plmnRangeList {

description "List of ranges of PLMNs (including the PLMN

IDs of the CHF instance) that can be served by the CHF instance.

If not provided, the CHF can serve any PLMN.";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

grouping pcscfInfo  {

description "This data type represents the information of a P-CSCF NF Instance.

For more information, see clause 6.1.6.2.53 of TS 29.510.";

leaf-list accessType {

description "It provides the condition of access type of the UE when

the session AMBR shall be enforced, see TS 29.512 [60].If this

attribute is included in SmfInfo, it shall contain the access type

(3GPP_ACCESS and/or NON_3GPP_ACCESS) supported by the SMF.If not

included, it shall be assumed the both access types are supported.";

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

min-elements 1;

max-elements 2;

}

leaf-list dnnList {

description "It represents the DNNs supported by the PCF. The DNN,

as defined in clause 9A of TS 23.003 [13], shall contain the Network

Identifier and it may additionally contain an Operator Identifier,

as specified in TS 23.003 [13] clause 9.1.1 and 9.1.2. If the Operator

Identifier is not included, the DNN is supported for all the PLMNs in

the plmnList of the NF Profile.If not provided, the PCF can serve any

DNN.";

min-elements 1;

type string;

}

leaf-list gmFqdn {

description "This attribute represents FQDN of the P-CSCF for the Gm

interface..";

max-elements 1;

type string;

}

leaf-list gmIpv4Addresses {

description "This attribute represents list of IPv4 addresses of of

the P-CSCF for the Gm interface.";

type inet:ipv4-address;

}

leaf-list gmIpv6Addresses {

description "This attribute represents list of IPv6 addresses of of the

P-CSCF for the Gm interface.";

type inet:ipv6-address;

}

leaf mwFqdn {

description "This attribute represents FQDN of the P-CSCF for the Mw

interface.";

type string;

}

leaf-list mwIpv4Addresses {

description "List of ranges of PLMNs (including the PLMN

IDs of the CHF instance) that can be served by the CHF instance.

If not provided, the CHF can serve any PLMN.";

type inet:ipv4-address;

}

leaf-list mwIpv6Addresses {

description "This attribute represents list of IPv4 addresses of

the P-CSCF for the Mw interface.";

type inet:ipv6-address;

}

list servedIpv4AddressRanges {

description "List of ranges of PLMNs (including the PLMN

IDs of the CHF instance) that can be served by the CHF instance.

If not provided, the CHF can serve any PLMN.";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

grouping HssInfoGrp {

description " Information of an HSS NF Instance";

leaf groupID{

description "This attribute defines the identity of the HSS group

that is served by the HSS instance.If not provided, the HSS instance

does not pertain to any HSS group.";

type string;

}

list imsiRanges {

description "This attribute defines the list of ranges of IMSIs whose

profile data is available in the HSS instance.";

min-elements 1;

key "id";

leaf id {

description "Unique identifier for the IMSI range.";

type string;

}

uses ImsiRange;

}

list imsPrivateIdentityRanges{

description "This attribute defines the list of ranges of IMS Private

Identities whose profile data is available in the HSS instance.";

min-elements 1;

key "id";

leaf id {

description "Unique identifier for the IMSI range.";

type string;

}

uses IdentityRange;

}

list imsPublicIdentityRanges{

description "This attribute defines the list of ranges of MSISDNs whose

profile data is available in the HSS instance. ";

min-elements 1;

key "id";

leaf id {

description "Unique identifier for the IMSI range.";

type string;

}

uses IdentityRange;

}

list msisdnRanges{

description "This attribute defines the list of ranges of IMS Public

Identities whose profile data is available in the HSS instance ";

min-elements 1;

key "id";

leaf id {

description "Unique identifier for the IMSI range.";

type string;

}

uses IdentityRange;

}

list externalGroupIdentifiersRanges{

description "This attribute defines the list of ranges of external

group IDs that can be served by this HSS instance.If not provided,

the HSS instance does not serve any external groups. ";

min-elements 1;

key "id";

leaf id {

description "Unique identifier for the IMSI range.";

type string;

}

uses IdentityRange;

}

list hssDiameterAddress{

description "This attribute defines the Diameter Address of the HSS ";

max-elements 1;

key "id";

leaf id {

description "Unique identifier for the IMSI range.";

type string;

}

uses NetworkNodeDiameterAddress;

}

list additionalDiamAddresses{

description "This attribute defines the Additional Diameter Addresses

of the HSS;may be present if hssDiameterAddress is present ";

min-elements 1;

key "id";

leaf id {

description "Unique identifier for the IMSI range.";

type string;

}

uses NetworkNodeDiameterAddress;

}

list dnnUpfInfoList {

description "List of parameters supported by the UPF per DNN.";

min-elements 1;

key dnn;

uses DnnUpfInfoItem;

}

}

grouping FiveGDdnmfInfo {

description "This data type represents the specific data for the 5G DDMF NF.

See clause 6.1.6.2.74 of TS 29.510.";

list plmnId {

description "PLMN ID for which list of supported S-NSSAI(s)

is provided.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

}

grouping nrfInfoGrp {

list servedUdrInfo {

description "This attribute contains all the udrInfo

attributes

locally configured in the NRF or the NRF received

during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses udrInfo;

}

list servedUdmInfo {

description "This attribute contains all the udmInfo

attributes

locally configured in the NRF or the NRF

received during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses udmInfo;

}

list servedAusfInfo {

description "This attribute contains all the

ausfInfo attributes

locally configured in the NRF or the NRF

received during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses ausfInfo;

}

list servedAmfInfo {

description "This attribute contains all the amfInfo

attributes

locally configured in the NRF or the NRF received

during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses amfInfo;

}

list servedSmfInfo {

description "This attribute contains all the smfInfo

attributes

locally configured in the NRF or the NRF received

during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses smfInfo;

}

list servedUpfInfo {

description "This attribute contains all the upfInfo

attributes

locally configured in the NRF or the NRF received

during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses upfInfo;

}

list servedPcfInfo {

description "This attribute contains all the pcfInfo

attributes

locally configured in the NRF or the NRF received

during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses pcfInfo;

}

list servedBsfInfo {

description "This attribute contains all the bsfInfo

attributes

locally configured in the NRF or the NRF received

during NF registration.";

key nfInstanceId;

leaf nfInstanceId{

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses bsfInfo;

}

list servedChfInfo {

description "This attribute contains all the bsfInfo

attributes

locally configured in the NRF or the NRF received

during NF registration.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses chfInfo;

}

list servedNwdafInfo {

description "This attribute contains all the nwdafInfo attributes

locally configured in the NRF or the NRF received during NF

registration. The key of the map is the nfInstanceId of which

the nwdafInfo belongs to.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses nwdaf3gpp:NwdafInfoGrp;

}

list servedPcscfInfoList {

description "This attribute contains all the pcscfInfo attributes

locally configured in the NRF or the NRF received during NF

registration. The key of the map is the nfInstanceId to which

the map entry belongs to.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses pcscfInfo;

}

list servedGmlcInfo {

description "This attribute contains all the gmlcInfo attributes

locally configured in the NRF or the NRF received during NF

registration. The key of the map is the nfInstanceId of which

the nefInfo belongs to.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses gmlc3gpp:GmlcInfoGrp;

}

list servedLmfInfo {

description "This attribute contains all the lmfInfo attributes

locally configured in the NRF or the NRF received during NF

registration. The key of the map is the nfInstanceId of which

the lmfInfo belongs to.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses lmf3gpp:LmfInfoGrp;

}

list servedNfInfo {

description "This attribute contains information of other NFs

without corresponding NF type specific Info extensions locally

configured in the NRF or the NRF received during NF registration.

The key of the map is the nfInstanceId of the NF. The map entry

is the NfInfo as defined in clause 5.3.229 representing the

information of a generic NF instance.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses nfInfoGrp;

}

list servedHssInfoList {

description "This attribute contains list of HssInfo attribute

locally configured in the NRF or that the NRF received during NF

registration. The key of the map is the nfInstanceId to which the

map entry belongs to.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses HssInfoGrp;

}

list served5gDdnmfInfo {

description "This attribute contains all the 5gDdnmfInfo

attribute locally configured in the NRF or that the NRF

received during NF registration. The key of the map is

the nfInstanceId to which the map entry belongs to.";

key nfInstanceId;

leaf nfInstanceId {

description "String uniquely identifying a NF instance.";

type string;

}

min-elements 1;

uses FiveGDdnmfInfo ;

}

}

list nrfInfo {

description "nrfinfo";

key idx;

leaf idx { type uint32; }

max-elements 1;

uses nrfInfoGrp;

}

list nfInfo {

description "nrfinfo";

key idx;

leaf idx { type uint32; }

max-elements 1;

uses nfInfoGrp;

}

grouping nfInfoGrp {

description "This data type represents the information of a

generic NF Instance. (See clause 6.1.6.2.56 TS 29.510 [23]).";

leaf nfType {

description "This parameter defines type of Network Function";

type uint16;

}

}

leaf customInfo {

description "Specific data for custom Network Functions.";

type string;

}

leaf recoveryTime {

description "Timestamp when the NF was (re)started.";

type yang:date-and-time;

}

leaf nfServicePersistence {

description "If present, and set to true, it indicates that

the different

service instances of a same NF Service

in this NF instance,

supporting a same API version, are capable to persist

their resource state in shared storage and

therefore these resources

are available after a new NF service

instance supporting

the same API version is selected by a NF

Service Consumer (see 3GPP TS 23.527).

Otherwise, it indicates that the NF Service

Instances of

a same NF Service are not capable to share

resource state inside the NF Instance.";

type boolean;

}

list nfServices {

description "List of NF Service Instances. It shall include

the services produced by the NF that can be

discovered by other NFs.";

key serviceInstanceID;

min-elements 1;

uses nfs3gpp:NFServiceGrp;

}

leaf nfProfileChangesSupportInd {

description "NF Profile Changes Support Indicator.

This IE may be present

in the NFRegister or NFUpdate (NF Profile Complete

Replacement) request

and shall be absent in the response.

true: the NF Service Consumer supports receiving NF Profile

Changes in the response.

false (default): the NF Service Consumer does not support

receiving NF Profile Changes in the response.";

type boolean;

}

leaf nfProfileChangesInd {

description "NF Profile Changes Indicator. This IE shall be absent

in the request to the NRF and may be included by the NRF

in NFRegister or NFUpdate (NF Profile Complete Replacement) response.

true: the NF Profile contains NF Profile changes.

false (default): complete NF Profile.";

type boolean;

}

list defaultNotificationSubscriptions {

description "Notification endpoints for different notification types.";

key notificationType;

min-elements 1;

uses types3gpp:DefaultNotificationSubscription;

}

}

typedef NFStatus {

type enumeration {

enum REGISTERED;

enum SUSPENDED;

}

}

typedef DataSetId {

type enumeration {

enum SUBSCRIPTION;

enum POLICY;

enum EXPOSURE;

enum APPLICATION;

}

}

grouping SupiRange {

leaf start {

description "First value identifying the start of

a SUPI range.

To be used when the range of SUPI's can be

represented

as a numeric range (e.g., IMSI ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf end {

description "Last value identifying the end of

a SUPI range.

To be used when the range of SUPI's can be

represented

as a numeric range (e.g. IMSI ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf pattern {

description "Pattern representing the set of SUPI's belonging

to this range.

A SUPI value is considered part

of the range

if and only if the SUPI string

fully matches the regular expression.";

type string;

}

}

grouping IdentityRange {

leaf start {

description "First value identifying the start of an identity range.

To be used when the range of identities can be represented

as a numeric range (e.g., MSISDN ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf end {

description "Last value identifying the end of an identity range.

To be used when the range of identities can be represented

as a numeric range (e.g. MSISDN ranges).";

type string {

pattern '^[0-9]+$';

}

}

leaf pattern {

description "Pattern representing the set of identities

belonging to this range.

An identity value is considered part of the range

if and only if the identity string fully

matches the regular expression.";

type string;

}

}

grouping SnssaiUpfInfoItem {

list sNssai {

description "Supported S-NSSAI.";

min-elements 1;

max-elements 1;

key "sst sd";

uses Snssai;

}

}

grouping DnnUpfInfoItem {

leaf dnn {

description "String representing a Data Network.";

mandatory true;

type string;

}

leaf-list dnaiList {

description "List of Data network access identifiers supported

by the UPF for this DNN.

The absence of this attribute indicates that the UPF

can be selected for this DNN for any DNAI.";

min-elements 1;

type string; //dnai is the type but its only a string with

//desc: DNAI (Data network access identifier),

//is this needed as its own typedef or string is ok

}

leaf-list pduSessionTypes {

description "List of PDU session type(s) supported by

the UPF for a specific DNN.";

min-elements 1;

type PduSessionType;

}

}

grouping Snssai {

leaf sst {

description "Unsigned integer, within the range 0 to 255,

representing the Slice/Service Type.

It indicates the expected Network Slice behaviour

in terms of features and services.";

mandatory true;

type uint32;

}

leaf sd {

description "3-octet string, representing the Slice Differentiator,

in hexadecimal representation.";

type string {

pattern '^[A-Fa-f0-9]{6}$';

}

}

reference "3GPP TS 29.571";

}

typedef PduSessionType {

type enumeration {

enum IPV4;

enum IPV6;

enum IPV4V6;

enum UNSTRUCTURED;

enum ETHERNET;

}

}

grouping Guami {

list plmnId {

description "PLMN Identity.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list amfId {

description "AMF Identity.";

min-elements 1;

max-elements 1;

key "amfRegionId amfSetId amfPointer";

uses types3gpp:AmfIdentifier;

}

}

grouping InterfaceUpfInfoItem {

leaf interfaceType {

description "User Plane interface type.";

mandatory true;

type UPInterfaceType;

}

choice address {

case ipv4EndpointAddresses {

leaf-list ipv4EndpointAddresses {

description "Available endpoint IPv4 address(es) of

the User Plane interface.";

min-elements 1;

type inet:ipv4-address;

}

}

case ipv6EndpointAddresses {

leaf-list ipv6EndpointAddresses {

description "Available endpoint IPv6 address(es) of

the User Plane interface.";

min-elements 1;

type inet:ipv6-address;

}

}

case endpointFqdn {

leaf endpointFqdn {

description "FQDN of available endpoint of the

User Plane interface.";

type inet:domain-name;

}

}

}

leaf networkInstance {

description "Network Instance associated to the

User Plane interface.";

type string;

}

}

typedef UPInterfaceType {

type enumeration {

enum N3;

enum N6;

enum N9;

}

}

typedef AccessType {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

}

grouping N2InterfaceAmfInfo {

choice address {

case ipv4EndpointAddress {

leaf-list ipv4EndpointAddress {

description "Available AMF endpoint IPv4 address(es) for N2.";

min-elements 1;

type inet:ipv4-address;

}

}

case ipv6EndpointAddress {

leaf-list ipv6EndpointAddress {

description "Available AMF endpoint IPv6 address(es) for N2.";

min-elements 1;

type inet:ipv6-address;

}

}

}

leaf amfName {

description "AMF name.";

type string;

}

}

grouping sNssaiSmfInfoItem {

list sNssai {

description "Supported S-NSSAI.";

min-elements 1;

max-elements 1;

key "sst sd";

uses Snssai;

}

list dnnSmfInfoList {

description "List of parameters supported by the SMF per DNN.

The absence indicates the DNN can be selected for any DNAI.";

min-elements 1;

key dnn;

uses DnnSmfInfoItem;

}

}

grouping DnnSmfInfoItem {

leaf dnn {

description "Supported DNN.";

mandatory true;

type string;

}

leaf-list dnaiList {

description "List of Data network access identifiers supported by

the SMF for this DNN. The absence of this attribute indicates that

the SMF can be selected for this DNN for any DNAI.";

min-elements 1;

type string;

}

}

grouping PlmnSnssai {

list plmnId {

description "PLMN ID for which list of supported S-NSSAI(s)

is provided.";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNssaiList {

description "The specific list of S-NSSAIs supported

by the given PLMN.";

min-elements 1;

key "sst sd";

uses Snssai;

}

}

grouping ImsiRange{

leaf start{

description "This attribute indicates the first value identifying

the start of a IMSI range. ";

type string{

pattern "^[0-9]+$";

}

}

leaf end{

description "This attribute indicates the last value identifying

the end of a IMSI range.";

type string{

pattern "^[0-9]+$";

}

}

leaf pattern{

description "This attribute indicates pattern (regular expression

according to the ECMA-262 dialect [75]) representing the set of

IMSIs belonging to this range. An IMSI value is considered part

of the range if and only if the IMSI string fully matches the

regular expression.Either the start and end attributes, or the

pattern attribute, shall be present.";

type string;

}

}

grouping NetworkNodeDiameterAddress {

leaf-list name{

description "This attribute indicates the Diameter

name of the network node diameter address.";

reference " See TS 29.571 [61]. String contains a Diameter Identity

(FQDN).";

type string;

min-elements 1;

max-elements 1;

}

leaf-list realm{

description "This attribute indicates the Diameter realm of the

network node diameter addres. See TS 29.571 [61]. String contains

a Diameter Identity (FQDN).";

type string;

min-elements 1;

max-elements 1;

}

}

}

<CODE ENDS>

*** END OF CHANGE 7 ***

*** START OF CHANGE 8 ***

*** yang-models/_3gpp-5gc-nrm-nwdaffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-nwdaffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nwdaffunction;

prefix nwdaf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NWDAF function in 5GC. For more

information about the NWDAF, see 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-03-25 { reference CR-1489 ; }

revision 2024-08-07 { reference CR-1329; }

revision 2024-04-12 { reference CR-1218; }

revision 2023-09-18 { reference CR-1043; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 {reference "initial revision"; }

typedef NwdafEvent {

description "The detailed ENUM value for NwdafEvent see the

Table 5.1.6.3.4-1 in TS 29.520";

type union {

type enumeration {

enum SLICE_LOAD_LEVEL;

enum NETWORK_PERFORMANCE;

enum NF_LOAD;

enum SERVICE_EXPERIENCE;

enum UE_MOBILITY;

enum UE_COMMUNICATION;

enum QOS_SUSTAINABILITY;

enum ABNORMAL_BEHAVIOUR;

enum USER_DATA_CONGESTION;

enum NSI_LOAD_LEVEL;

enum DN_PERFORMANCE;

enum DISPERSION;

enum RED_TRANS_EXP;

enum WLAN_PERFORMANCE;

enum SM_CONGESTION;

}

type string;

}

}

grouping NwdafCapabilityGrp {

description "This data type represents the capability supported by the

NWDAF.";

reference "TS 29.510";

leaf analyticsAggregation {

type boolean ;

default false;

description "It indicates whether the NWDAF supports analytics

aggregation";

}

leaf analyticsMetadataProvisioning {

type boolean ;

default false;

description "It indicate whether the NWDAF supports analytics metadata

provisioning:";

}

leaf roamingExchange {

type boolean ;

default false;

description "It indicates whether the NWDAF

supports roaming exchange capability";

}

}

grouping MlAnalyticsInfoGrp {

description "This data type represents ML Analytics Filter information

supported by the Nnwdaf_MLModelProvision service.";

reference "TS 29.510";

leaf-list mlAnalyticsIds {

type NwdafEvent;

ordered-by user;

description "This attribute represents the Analytic functionalities

(identified by nwdafEvent defined in TS 29.520 [85]) of the NWDAF

instance. MnS consumer can configure this attribute to specify

which Analytic functionalities (identified by nwdafEvent) can be

performed the NWDAF instance. If the value of this attribute is not

present, the NWDAF instance can perform any NWDAFEvents

Analytics Id(s) supported by the Nnwdaf_MLModelProvision service,

if none are provided the NWDAF can serve any mlAnalyticsId.";

}

list sNSSAIList {

min-elements 1;

description "List of S-NSSAIs the managed object is capable of

supporting.";

reference "3GPP TS 23.003";

key "sd sst";

uses types5g3gpp:SNssai;

}

list trackingAreaList {

description "This attribute represents area of Interest of the ML model,

if none are provided the ML model for the analytics can apply to any

TAIs.

If present, it represents the list of TAIs, it may contain one or

more non-3GPP access TAIs.";

key idx ;

leaf idx { type uint32; }

uses types3gpp:TaiGrp;

}

leaf-list mlModelInterInfo {

type string {

pattern "[0-9]{6}";

}

description "This attribute defines the list of NWDAF vendors that are

allowed to retrieve ML models from the NWDAF containing MTLF. The

absence of this attribute indicates that none of the NWDAF vendors

can retrieve the ML models.

allowedValues: 6 decimal digits; if the SMI code has less than

6 digits, it shall be padded with leading digits '0' to complete a

6-digit string value.";

}

leaf flCapabilityType {

type enumeration {

enum FL_SERVER {

description "NWDAF containing MTLF as Federated Learning Server";

}

enum FL_CLIENT {

description "NWDAF containing MTLF as Federated Learning Client";

}

enum FL_SERVER_AND_CLIENT {

description "NWDAF containing MTLF as Federated Learning Server and

Client.";

}

}

description "This attribute defines the federated learning capability

type supported by NWDAF containing MTLF.";

}

list flTimeInterval {

key idx ;

leaf idx { type uint32; }

description "This attribute defines the time window at which the

indicated flCapabilityType supported by NWDAF MTLF is available.

This attribute shall be present only if flCapabilityType attribute

is present.";

uses types3gpp:TimeWindowGrp;

}

}

grouping NwdafInfoGrp {

description "This data type represents specific data for the NWDAF.";

reference "TS 29.510";

leaf-list nwdafEvents {

type NwdafEvent;

ordered-by user;

description "This attribute represents the Analytic functionalities

(identified by nwdafEvent defined in TS 29.520) of the NWDAF instance.

MnS consumer can configure this attribute to specify which Analytic

functionalities (identified by nwdafEvent) can be performed the NWDAF

instance. If the value of this attribute is not present, the NWDAF

instance can perform any NWDAFEvents.

The detailed ENUM value for NwdafEvent see the Table 5.1.6.3.4-1

in TS 29.520";

}

leaf-list eventIds {

type string ;

min-elements 1;

description "It represents the EventId(s) supported by the

Nnwdaf_AnalyticsInfo service, if none are provided the NWDAF can serve

any eventId. (see clause TS 29.520)";

}

list taiList {

description "The list of TAIs. ";

min-elements 1;

key idx ;

leaf idx { type uint32; }

uses types3gpp:TaiGrp;

}

list taiRangeList {

min-elements 1;

key idx ;

leaf idx { type uint32; }

description "The range of TAIs.";

uses types5g3gpp:TaiRangeGrp;

}

list nwdafCapability {

max-elements 1;

key idx ;

leaf idx { type uint32; }

description "This attribute indicates the capability of the NWDAF.

If not present, the NWDAF shall be regarded with no capability.";

uses NwdafCapabilityGrp;

}

leaf analyticsDelay {

type int32 ;

units seconds;

description "It represents the supported Analytics Delay related to the

eventIds and nwdafEvents.

It is an unsigned integer identifying a period of time in units of

seconds.(see clause 5.2.2 TS 29.571).";

}

leaf-list servingNfTypeList {

type types3gpp:NfType;

min-elements 1;

description "It contains the list of NF type(s) from which the NWDAF NF

can collect data. The absence of this attribute indicates that the

NWDAF can collect data from any NF type.";

}

leaf-list servingNfSetIdList {

type string ;

min-elements 1;

description "It contains the list of NF type(s) from which the NWDAF NF

can collect data. The absence of this attribute indicates that the

NWDAF can collect data from any NF type. (see clause 5.4.2 NfSetId

in TS 29.571)";

}

list mlAnalyticsList {

min-elements 1;

key idx ;

leaf idx { type uint32; }

description "It represents ML Analytics Filter information supported by

the Nnwdaf_MLModelProvision service.";

uses MlAnalyticsInfoGrp;

}

}

grouping NetworkSliceInfoGrp {

description "Represents information of network slice when the NWDAF is

authorized to collect the management data of the network slice. ";

list sNSSAI {

description "It represents the S-NSSAI the NetworkSlice managed object

is supporting. The S-NSSAI is defined in TS 23.003 ";

key idx;

min-elements 1;

max-elements 1;

leaf idx {

type string;

}

uses types5g3gpp:SNssai;

}

leaf-list cNSIId {

type string;

description "It represents NSI ID which is an identifier for identifying

the Core Network part of a Network Slice instance when multiple

Network Slice instances of the same Network Slice are deployed,

and there is a need to differentiate between them in the 5GC.

See NSI ID definition in clause 3.1 of TS 23.501 and

subclause 6.1.6.2.7 of TS 29.531.";

}

leaf networkSliceRef {

type types3gpp:DistinguishedName;

mandatory true;

description "This holds a DN of the NetworkSlice managed object relating

to the NetworkSlice instance differentiated by sNSSAI and optional

cNSIId.";

}

}

grouping NWDAFFunctionGrp {

description "The NWDAFFunction IOC includes attributes inherited from

ManagedFunction IOC(defined in TS 28.622)and the following attributes:";

uses mf3gpp:ManagedFunctionGrp;

list pLMNInfoList {

description "It defines the PLMN(s) of a Network Function.";

min-elements 1;

key "idx";

leaf idx { type uint32; }

uses types3gpp:PLMNId;

uses types5g3gpp:SNssai;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

description "Profile definition of a Managed NF (See TS 23.501)";

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

list networkSliceInfoList {

key idx;

min-elements 1;

description "The attribute specifies a list of NetworkSliceInfo which

is defined as a datatype (see clause 5.3.95). It is used by an

authorized consumer, e.g. NWDAF, to facilitate the data collection

from OAM.";

leaf idx {

type string;

}

uses NetworkSliceInfoGrp;

yext3gpp:inVariant;

}

list nwdafInfo {

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32; }

description "It represents specific data for the NWDAF.";

uses NwdafInfoGrp;

}

leaf administrativeState {

type types3gpp:BasicAdministrativeState ;

mandatory true;

description "This attribute determines whether the NWDAF is enabled or

disabled. MnS consumer can configure this attribute to activate or

de-activate the analytic functionalities (identified by nwdafEvent

defined in TS 29.520) of the NWDAF instance.";

}

leaf nwdafLogicalFuncSupported {

type enumeration {

enum NWDAF_WITH_ANLF {

description "Indicates the NWDAF containing Analytics logical

function (AnLF)";

}

enum NWDAF_WITH_MTLF {

description "Indicates the NWDAF containing Model Training logical

function (MTLF).";

}

enum NWDAF_WITH_ANLF_MTLF {

description "Indicates the NWDAF containing both Analytics logical

function (AnLF) and Model Training logical function (MTLF).";

}

}

config false;

description "It represents the logical functions supported by the NWDAF.

If not present, the NWDAF shall be regarded with no logical

decomposition, in that case the NWDAF only supports the analytics

services.";

}

leaf roamingAnalytics {

type boolean ;

config false;

description "It indicates whether the NWDAF supports

Nnwdaf_RoamingAnalytics service";

}

leaf roamingData {

type boolean ;

config false;

description "It indicates whether the NWDAF supports

Nnwdaf_RoamingData service";

}

}

augment "/me3gpp:ManagedElement" {

list NWDAFFunction {

description "5G Core NWDAF Function.

For more information about the NWDAF, see TS 23.501. Several

attributes (including 'nwdafInfo', 'administrativeState' and

'ManagedNFProfile.servingScope') are used to control the functionalities

(identified by nwdafEvent defined in TS 29.520) of the NWDAF instance.

The attribute 'ManagedNFProfile.servingScope' is used to represent

specified certain geographical area(s) can be served by the NWDAF

instance.

The attribute 'NwdafInfo.taiList' and 'NwdafInfo.taiRangeList' is used

to represent specified certain tracking area(s) can be served by the

NWDAF instance.";

reference "3GPP TS 28.541 3GPP TS 23.501";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NWDAFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 8 ***

*** START OF CHANGE 9 ***

*** yang-models/_3gpp-5gc-nrm-predefinedpccruleset.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-predefinedpccruleset {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-predefinedpccruleset;

prefix PredPccRules3gpp;

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-5gc-nrm-pcffunction { prefix pcf3gpp; }

import ietf-yang-types { prefix yang; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the predefined PCC rules, which are

configured to SMF and referenced by PCF.

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-03-25 { reference CR-1489 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-10-04 { reference "CR-0393"; }

revision 2020-08-21 { reference "CR-0330"; }

grouping TscaiInputContainer {

description "It specifies the transports TSCAI input parameters for TSC

traffic at the ingress interface of the DS-TT/UE for a PCC rule.";

reference " 3GPP TS 29.512";

leaf periodicity {

type uint32;

description "It identifies the time period between the start of two bursts

in reference to the TSN GM.";

reference "3GPP TS 29.571.";

}

leaf burstArrivalTime {

type yang:date-and-time;

description "It Indicates the arrival time (in date-time format) of the

data burst in reference to the TSN GM.";

reference "3GPP,TS 29.571.";

}

}

grouping ConditionData {

description "It specifies the specifies the condition data for a PCC rule.";

leaf condId {

type string;

mandatory true;

description "It uniquely identifies the condition data.";

}

leaf activationTime {

type yang:date-and-time;

description " It indicates the time (in date-time format) when the decision

data shall be activated.";

reference "3GPP29.512 and TS 29.571.";

}

leaf deactivationTime {

type yang:date-and-time;

description "It indicates the time (in date-time format) when the decision

data shall be deactivated.";

reference "3GPPTS 29.512 and TS 29.571.";

}

leaf accessType {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

description "It provides the condition of access type of the UE when the

session AMBR shall be enforced.";

reference "3GPPTS 29.512.";

}

leaf ratType {

type enumeration {

enum NR;

enum EUTRA;

enum WLAN;

enum VIRTUAL;

enum NBIOT;

enum WIRELINE;

enum WIRELINE_CABLE;

enum WIRELINE_BBF;

enum LTE-M;

enum NR_U;

enum EUTRA_U;

enum TRUSTED_N3GA;

enum TRUSTED_WLAN;

enum UTRA;

enum GERA;

}

description "It provides the condition of RAT type of the UE when the

session AMBR shall be enforced.";

reference "3GPPTS 29.512 and TS 29.571.";

}

}

grouping SteeringMode {

description "It specifies the traffic distribution rule, see TS 29.512.";

leaf steerModeValue {

type enumeration {

enum ACTIVE_STANDBY;

enum LOAD_BALANCING;

enum SMALLEST_DELAY;

enum PRIORITY_BASED;

}

mandatory true;

description "It indicates the value of the steering mode, see TS 29.512.";

}

leaf active {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

description "It indicates the active access, see TS 29.571.";

}

leaf standby {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

description "It indicates the Standby access, see TS 29.571.";

}

leaf threeGLoad {

type uint8 {

range 0..100;

}

description "It indicates the traffic load to steer to the 3GPP Access

expressed in one percent.";

}

leaf prioAcc {

type enumeration {

enum 3GPP_ACCESS;

enum NON_3GPP_ACCESS;

}

description "It indicates the high priority access.";

reference "3GPPTS 29.571";

}

}

grouping UpPathChgEvent {

description "It specifies the information about the AF subscriptions of the

UP path change.";

reference "3GPPTS 29.512";

leaf notificationUri {

type string;

mandatory true;

description "It provides notification address (Uri) of AF receiving the

event notification.";

}

leaf notifCorreId {

type string;

mandatory true;

description "It is used to set the value of Notification Correlation ID in

the notification sent by the SMF, see TS 29.512";

}

leaf dnaiChgType {

type enumeration {

enum EARLY;

enum EARLY_LATE;

enum LATE;

}

mandatory true;

description "It indicates the type of DNAI change, see TS 29.512";

}

leaf afAckInd {

type boolean;

default false;

description "It identifies whether the AF acknowledgement of UP path

event notification is expected.";

}

}

grouping RouteInformation {

description "It specifies the traffic routing information.";

leaf ipv4Addr {

type string;

description "It defines the Ipv4 address of the tunnel end point in the

data network, formatted in the dotted decimal notation.";

}

leaf ipv6Addr {

type string;

description "It defines the Ipv6 address of the tunnel end point in the

data network.";

}

leaf portNumber {

type uint32;

mandatory true;

description " It defines the UDP port number of the tunnel end point in

the data network, see TS 29.571";

}

}

grouping RouteToLocation {

description "It specifies a list of location which the traffic shall be

routed to for the AF request.";

leaf dnai {

type string;

mandatory true;

description "It represents the DNAI (Data network access identifier).";

reference "3GPP 3GPP TS 23.501";

}

container routeInfo{

description "It provides the traffic routing information.";

uses RouteInformation;

}

leaf routeProfId {

type string;

description "It identifies the routing profile.";

}

}

grouping RedirectInformaton {

description "It specifies the redirect information for traffic control in

the PCC rule.";

leaf redirectEnabled {

type boolean;

mandatory true;

description "It indicates whether the redirect instruction is enabled.";

}

leaf redirectAddressType {

type enumeration {

enum IPV4_ADDR;

enum IPV6_ADDR;

enum URL;

enum SIP_URI;

}

mandatory true;

description "It indicates the type of redirect address.";

reference "3GPPTS 29.512";

}

leaf redirectServerAddress {

type string;

mandatory true;

description "It indicates the address of the redirect server.";

}

}

grouping TrafficControlDataInformation {

description "It specifies the traffic control data for a service

flow of a PCC rule.";

leaf tcId {

type string;

mandatory true;

description "It univocally identifies the traffic control policy data

within a PDU session.";

}

leaf flowStatus {

type enumeration {

enum ENABLED-UPLINK;

enum ENABLED-DOWNLINK;

enum ENABLED;

enum DISABLED;

enum REMOVED;

}

mandatory true;

description "It represents whether the service data flow(s) are enabled

or disabled.";

}

container redirectInfo {

description "It contains the redirect information indicating whether the

detected application traffic should be redirected to another controlled

address.";

uses RedirectInformaton;

}

container addRedirectInfo {

description "It contains the additional redirect information indicating

whether the detected application traffic should be redirected to another

controlled address.";

list redirectInfo {

description "The list of redirect information indicating whether the

detected application traffic should be redirected to another

controlled address.";

key "redirectServerAddress";

uses RedirectInformaton;

}

}

leaf muteNotif {

type boolean;

default false;

description "It indicates whether applicat'on's start or stop notification

is to be muted.";

}

leaf trafficSteeringPolIdDl {

type string;

description "It references to a pre-configured traffic steering policy for downlink traffic at the SMF, see TS 29.512";

}

leaf trafficSteeringPolIdUl {

type string;

description "It references to a pre-configured traffic steering policy for

uplink traffic at the SMF, see TS 29.512";

}

container routeToLocs {

description "It provides a list of location which the traffic shall be

routed to for the AF request.";

list routeToLoc {

description "The list of location which the traffic shall be routed to

for the AF request.";

key "dnai";

uses RouteToLocation;

}

}

uses UpPathChgEvent;

leaf steerFun {

type enumeration {

enum MPTCP;

enum ATSSS_LL;

}

description "It indicates the applicable traffic steering functionality.";

reference "3GPPTS 29.512";

}

container steerModeDl {

description "It provides the traffic distribution rule across 3GPP and

Non-3GPP accesses to apply for downlink traffic.";

uses SteeringMode;

}

container steerModeUl {

description "It provides the traffic distribution rule across 3GPP and Non-3GPP accesses to apply for uplink traffic.";

uses SteeringMode;

}

leaf mulAccCtrl {

type enumeration {

enum ALLOWED;

enum NOT_ALLOWED;

}

description "It indicates whether the service data flow, corresponding to the service data flow template, is allowed or not allowed.";

}

}

grouping ARP {

description "It specifies the allocation and retention priority of a QoS control policy.";

leaf priorityLevel {

type uint8 {

range 1..15;

}

mandatory true;

description "It defines the relative importance of a resource request.";

}

leaf preemptCap {

type enumeration {

enum NOT_PREEMPT;

enum MAY_PREEMPT;

}

mandatory true;

description "It defines whether a service data flow may get resources that were already assigned to another service data flow with a lower priority level.";

}

leaf preemptVuln {

type enumeration {

enum NOT_PREEMPTABLE;

enum PREEMPTABLE;

}

mandatory true;

description "It defines whether a service data flow may lose the resources assigned to it in order to admit a service data flow with higher priority level.";

}

}

grouping QosDataInformation {

description "It specifies the QoS control policy data for a service flow of a PCC rule.";

leaf qosId {

type string;

mandatory true;

description "It identifies the QoS control policy data for a PCC rule.";

}

leaf fiveQIValue {

type uint8 {

range 0..255;

}

mandatory true;

description "It indicates the 5QI value.";

}

leaf maxbrUl {

type string;

description "It represents the maximum uplink bandwidth.";

}

leaf maxbrDl {

type string;

description "It represents the maximum downlink bandwidth.";

}

leaf gbrUl {

type string;

description "It represents the guaranteed uplink bandwidth.";

}

leaf gbrDl {

type string;

description "It represents the guaranteed downlink bandwidth.";

}

uses ARP;

leaf qosNotificationControl {

type boolean;

default false;

description "It indicates whether notifications are requested from 3GPP NG-RAN when the GFBR can no longer (or again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow.";

}

leaf reflectiveQos {

type boolean;

default false;

description "Indicates whether the QoS information is reflective for the corresponding non-GBR service data flow";

}

leaf sharingKeyDl {

type string;

description "It indicates, by containing the same value, what PCC rules may share resource in downlink direction.";

}

leaf sharingKeyUl {

type string;

description "It indicates, by containing the same value, what PCC rules may share resource in uplink direction.";

}

leaf maxPacketLossRateDl {

type uint16 {

range 0..1000;

}

description "It indicates the downlink maximum rate for lost packets that can be tolerated for the service data flow.";

}

leaf maxPacketLossRateUl {

type uint16 {

range 0..1000;

}

description "It indicates the uplink maximum rate for lost packets that can be tolerated for the service data flow.";

}

leaf extMaxDataBurstVol {

type uint32 {

range 4096..2000000;

}

description "It denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB, see TS 29.512";

}

}

grouping EthFlowDescription {

description "It describes an Ethernet flow.";

leaf destMacAddr {

type string;

mandatory true;

description "It specifies the destination MAC address formatted in the

hexadecimal .";

reference "clause 1.1 and clause 2.1 of IETF RFC";

}

leaf ethType {

type string;

mandatory true;

description "A two-octet string that represents the Ethertype.";

reference "IEEE 802.3 and IETF RFC 7042 in hexadecimal representation.";

}

leaf fDesc {

type string;

description "It contains the flow description for the Uplink or Downlink IP flow. It shall be present when the ethtype is IP.";

}

leaf fDir {

type enumeration {

enum DOWNLINK;

enum UPLINK;

}

mandatory true;

description "It indicates the packet filter direction.";

}

leaf sourceMacAddr {

type string;

mandatory true;

description "It specifies the source MAC address formatted in the

hexadecimal notation.";

reference "clause 1.1 and clause 2.1 of IETF RFC 7042";

}

leaf-list vlanTags {

type string;

description "It specifies the Customer-VLAN and/or Service-VLAN tags

containing the VID, PCP/DEI fields as defined in IEEE 802.1Q and

IETF RFC 7042. The first/lower instance in the array stands for the

Customer-VLAN tag and the second/higher instance in the array stands

for the Service-VLAN tag.";

}

leaf srcMacAddrEnd {

type string;

description "It specifies the source MAC address end. If this attribute is present, the sourceMacAddr attribute specifies the source MAC address start. E.g. srcMacAddrEnd with value 00-10-A4-23-3E-FE and sourceMacAddr with value 00-10-A4-23-3E-02 means all MAC addresses from 00-10-A4-23-3E-02 up to and including 00-10-A4-23-3E-FE.";

}

leaf destMacAddrEnd {

type string;

description "It specifies the destination MAC address end. If this attribute is present, the destMacAddr attribute specifies the destination MAC address start.";

}

}

grouping FlowInformation {

description "It specifies the flow information of a PCC rule.";

leaf flowDescription {

type string;

mandatory true;

description "It defines a packet filter for an IP flow.";

}

uses EthFlowDescription;

leaf packFiltId {

type string;

mandatory true;

description "It is the identifier of the packet filter.";

}

leaf packetFilterUsage {

type boolean;

default false;

description "It indicates if the packet shall be sent to the UE.";

}

leaf tosTrafficClass {

type string;

mandatory true;

description "It contains the Ipv4 Type-of-Service and mask field or the Ipv6 Traffic-Class field and mask field.";

}

leaf spi {

type string;

mandatory true;

description "It is the security parameter index of the IPSec packet.";

reference "IETF RFC 4301";

}

leaf flowLabel {

type string;

description "It specifies the Ipv6 flow label header field.";

}

leaf flowDirection {

type enumeration {

enum DOWNLINK;

enum UPLINK;

enum BIDIRECTIONAL;

enum UNSPECIFIED;

}

mandatory true;

description "It indicates the direction/directions that a filter is applicable.";

}

}

grouping PccRule {

description "It specifies the PCC rule, see TS 29.512";

leaf pccRuleId {

type string;

mandatory true;

description "It identifies the PCC rule.";

}

container flowInfoList {

description "It is a list of IP flow packet filter information.";

list flowInfo {

description "The list of IP flow packet filter information.";

key "packFiltId";

uses FlowInformation;

}

}

leaf applicationId {

type string;

default false;

description "A reference to the application detection filter configured

at the UPF.";

}

leaf appDescriptor {

type string;

description "It is the ATSSS rule application descriptor.";

}

leaf contentVersion {

type uint8;

description "Indicates the content version of the PCC rule.";

}

leaf precedence {

type uint8 {

range 0..255;

}

description "It indicates the order in which this PCC rule is applied

relative to other PCC rules within the same PDU session.";

}

leaf afSigProtocol {

type enumeration {

enum NO_INFORMATION;

enum SIP;

}

description "Indicates the protocol used for signalling between the UE

and the AF, the default value is NO_INFORMATION.";

}

leaf isAppRelocatable {

type boolean;

default false;

description "It indicates the application relocation possibility, the

default value is NO_INFORMATION.";

}

leaf isUeAddrPreserved {

type boolean;

default false;

description "It Indicates whether UE IP address should be preserved.";

}

container qosData {

description "It contains the QoS control policy data for a PCC rule.";

list qosDataInfo {

description "The list of QoS control policy data.";

key "qosId";

uses QosDataInformation;

}

}

container altQosParams {

description "It contains the QoS control policy data for the Alternative

QoS parameter sets of the service data flow.";

list qosDataInfo {

description "The list of QoS control policy data.";

key "qosId";

uses QosDataInformation;

}

}

container trafficControlData {

description "It contains the traffic control policy data for a PCC rule.";

list trafficControlDataInfo {

description "The list of traffic control policy data.";

key "tcId";

uses TrafficControlDataInformation;

}

}

uses ConditionData;

container tscaiInputUl {

description "It contains transports TSCAI input parameters for TSC traffic at the ingress interface of the DS-TT/UE (uplink flow direction).";

uses TscaiInputContainer;

}

container tscaiInputDl {

description "It contains transports TSCAI input parameters for TSC traffic at the ingress of the NW-TT (downlink flow direction).";

uses TscaiInputContainer;

}

}

grouping PredefinedPccRuleSetGrp {

description "Represents the PredefinedPccRuleSet IOC.";

list PredefinedPccRules {

description "The list of predefined PCC rules.";

key "pccRuleId";

uses PccRule;

}

}

grouping PredefinedPccRuleSetSubtree {

description "It specifies the PredefinedPccRuleSet IOC with inherited attributes.";

list PredefinedPccRuleSet {

description "Specifies the predefined PCC rules.";

key "id";

uses top3gpp:Top_Grp;

container attributes {

description "It contains the attributes defined specifically in the PredefinedPccRuleSet IOC.";

uses PredefinedPccRuleSetGrp;

}

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

description "It specifies the containment relation of PredefinedPccRuleSet MOI with SMFFunction MOI.";

uses PredefinedPccRuleSetSubtree;

}

augment "/me3gpp:ManagedElement/pcf3gpp:PCFFunction" {

description "It specifies the containment relation of PredefinedPccRuleSet MOI with PCFFunction MOI.";

uses PredefinedPccRuleSetSubtree;

}

}

<CODE ENDS>

*** END OF CHANGE 9 ***