3GPP TSG-SA5 Meeting #160	S5-251299

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0610 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct bindings information for 5G ProSe Charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-03-26 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The ProSe charging-related attributes are included in the data type named prose charging information and in the corresponding APIs. However, the binding of ProSe information is referred to as prose information, resulting in an inconsistency in naming. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the data type name in the binding table to ensure consistency. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent naming may lead to confusion and implementation errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

7.9	Bindings for 5G ProSe charging

Table 7.9-1: Bindings of 5G ProSe charging CDR field, Information Element and Resource Attribute

| Information Element | CDR Field | Resource Attribute |
| --- | --- | --- |
|  |  | ChargingDataRequest |
| Multiple Unit Usage | List of Multiple Unit Usage | /multipleUnitUsage |
| Used Unit Container | Used Unit Container | /multipleUnitUsage/usedUnitContainer |
| PC5 Container Information | PC5 Container Information | /multipleUnitUsage/usedUnitContainer/pC5 ContainerInformation |
| Coverage Info | Coverage Info | /multipleUnitUsage/usedUnitContainer/pC5 ContainerInformation/coverage Info |
| Radio Parameter Set Info | Radio Parameter Set Info | /multipleUnitUsage/usedUnitContainer/pC5 ContainerInformation/radioParameterSet Info |
| Transmitter Info | Transmitter Info | /multipleUnitUsage/usedUnitContainer/pC5 ContainerInformation/transmitterInfo |
| Time of First Transmission | Time of First Transmission | /multipleUnitUsage/usedUnitContainer/pC5 ContainerInformation/timeOfFirst Transmission |
| Time of First Reception | Time of First Reception | /multipleUnitUsage/usedUnitContainer/pC5 ContainerInformation/TimeOfFirstReception |
| ProSe Information | ProSe Information | /proseChargingInformation |
| Announcing PLMN ID | Announcing PLMN ID | /proseChargingInformation/announcingPlmnId |
| Announcing UE HPLMN Identifier | Announcing UE HPLMN Identifier | /proseChargingInformation/announcingUeHplmnId |
| Announcing UE VPLMN Identifier | Announcing UE VPLMN Identifier | /proseChargingInformation/announcingUeVplmnId |
| Monitoring UE HPLMN Identifier | Monitoring UE HPLMN Identifier | /proseChargingInformation/monitoringUeHplmnId |
| Monitoring UE VPLMN Identifier | Monitoring UE VPLMN Identifier | /proseChargingInformation/monitoringUeVplmnId |
| Discoverer UE HPLMN Identifier | Discoverer UE HPLMN Identifier | /proseChargingInformation/discovererUeHplmnId |
| Discoverer UE VPLMN Identifier | Discoverer UE VPLMN Identifier | /proseChargingInformation/discovererUeVplmnId |
| Discoveree UE HPLMjN Identifier | Discoveree UE HPLMN Identifier | /proseChargingInformation/discovereeUeHplmnId |
| Discoveree UE VPLMN Identifier | Discoveree UE VPLMN Identifier | /proseChargingInformation/discovereeUeVplmnId |
| Monitored PLMN Identifier | Monitored PLMN Identifier | /proseChargingInformation/monitoredPlmnIdentifier |
| ProSe Application ID | ProSe Application ID | /proseChargingInformation/proseApplicationId |
| Application ID | Application ID | /proseChargingInformation/applicationId |
| Application Specific Data | Application Specific Data | /proseChargingInformation/applicationSpecificData |
| ProSe functionality | ProSe functionality | /proseChargingInformation/proSeFunctionality |
| ProSe Event Type | ProSe Event Type | /proseChargingInformation/proSeEventType |
| Direct Discovery Model | Direct Discovery Model | /proseChargingInformation/directDiscoveryModel |
| Validity Period | Validity Period | /proseChargingInformation/validityPeriod |
| Role of UE | Role of UE | /proseChargingInformation/roleOfUe |
| ProSe Request Timestamp | ProSe Request Timestamp | /proseChargingInformation/proSeRequestTimestamp |
| PC3 Protocol Cause | PC3 Protocol Cause | /proseChargingInformation/pC3ProtocolCause |
| Monitoring UE Identifier | Monitoring UE Identifier | /proseChargingInformation/monitoringUEIdentifier |
| Requestor PLMN Identifier | Requestor PLMN Identifier | /proseChargingInformation/requestorPlmnIdentifier |
| Requested Application Layer User ID | Requested Application Layer User ID | /proseChargingInformation/requestedApplicationLayerUserId |
| Requested PLMN Identifier | Requested PLMN Identifier | /proseChargingInformation/requestedPlmnIdentifier |
| Time Window | Time Window | /proseChargingInformation/timeWindow |
| Range Class | Range Class | /proseChargingInformation/rangeClass |
| Proximity Alert Indication | Proximity Alert Indication | /proseChargingInformation/proximityAlertIndication |
| Proximity Alert Timestamp | Proximity Alert Timestamp | /proseChargingInformation/proximityAlertTimestamp |
| Proximity Cancellation Timestamp | Proximity Cancellation Timestamp | /proseChargingInformation/proximityCancellationTimestamp |
| Relay IP address | Relay IP address | /proseChargingInformation/relayIpAddress |
| ProSe UE-to-Network Relay UE ID | ProSe UE-to-Network Relay UE ID | /proseChargingInformation/proSeUeToNetworkRelayUeId |
| ProSe Destination Layer-2 ID | ProSe Destination Layer-2 ID | /proseChargingInformation/proSeDestinationLayer2 Id |
| PFI Container Information | PFI Container Information | /proseChargingInformation/pFIContainerInformation |
| PC5 QoS Flow ID | PC5 QoS Flow ID | /proseChargingInformation/pFIContainerInformation/pC5QosFlowId |
| Time of First Usage | Time of First Usage | /proseChargingInformation/pFIContainerInformation/ timeOfFirstUsage |
| Time of Last Usage | Time of Last Usage | /proseChargingInformation/pFIContainerInformation/ timeOfLastUsage |
| QoS Information | QoS Information | /proseChargingInformation/pFIContainerInformation/qosInformation |
| QoS Characteristics | QoS Characteristics | /proseChargingInformation/pFIContainerInformation/ qoSCharacteristics |
| User Location Information | User Location Information | /proseChargingInformation/pFIContainerInformation/ userLocationInformation |
| UE Time Zone | UE Time Zone | /proseChargingInformation/pFIContainerInformation/ueTimeZone |
| Presence Reporting Area Information | Presence Reporting Area Information | /proseChargingInformation/pFIContainerInformation/ presenceReportingAreaInformation |
| Report Time | Report Time | /proseChargingInformation/pFIContainerInformation/ reportTime |
| Transmission Data Container | Transmission Data Container | /proseChargingInformation/transmissionDataContainer |
| Local Sequence Number | Local Sequence Number | /proseChargingInformation/transmissionDataContainer/localSequenceNumber |
| Change Time | Change Time | /proseChargingInformation/transmissionDataContainer/changeTime |
| Coverage status | Coverage status | /proseChargingInformation/transmissionDataContainer/coverageStatus |
| User Location Information | User Location Information | /proseChargingInformation/transmissionDataContainer/userLocationInformation |
| Data Volume Transmitted | Data Volume Transmitted | /proseChargingInformation/transmissionDataContainer/dataVolumeTransmitted |
| Change Condition | Change Condition | /proseChargingInformation/transmissionDataContainer/changeCondition |
| VPLMN Identifier | VPLMN Identifier | /proseChargingInformation/transmissionDataContainer/vplmnIdentifier |
| Usage Information Report Sequence Number | Usage Information Report Sequence Number | /proseChargingInformation/transmissionDataContainer/usageInformationReportSequenceNumber |
| Radio Resources Indicator | Radio Resources Indicator | /proseChargingInformation/transmissionDataContainer/radioResourcesId |
| Radio Frequency | Radio Frequency | /proseChargingInformation/transmissionDataContainer/radioFrequency |
| PC5 Radio Technology | PC5 Radio Technology | /proseChargingInformation/transmissionDataContainer/pC5RadioTechnology |
| Reception Data Container | Reception Data Container | /proseChargingInformation/receptionDataContainer |
| Local Sequence Number | Local Sequence Number | /proseChargingInformation/receptionDataContainer/ localSequenceNumber |
| Change Time | Change Time | /proseChargingInformation/receptionDataContainer/ changeTime |
| Coverage Status | Coverage Status | /proseChargingInformation/receptionDataContainer/ coverageStatus |
| User Location Information | User Location Information | /proseChargingInformation/receptionDataContainer/ userLocationInformation |
| Data Volume Received | Data Volume Received | /proseChargingInformation/receptionDataContainer/ dataReceived |
| Change Condition | Change Condition | /proseChargingInformation/receptionDataContainer/ changeCondition |
| VPLMN Identifier | VPLMN Identifier | /proseChargingInformation/receptionDataContainer/ vplmnIdentifier |
| Usage Information Report Sequence Number | Usage Information Report Sequence Number | /proseChargingInformation/receptionDataContainer/ usageInformationReportSequenceNumber |
| Radio Resources Indicator | Radio Resources Indicator | /proseChargingInformation/receptionDataContainer/ radioResourcesId |
| Radio Frequency | Radio Frequency | /proseChargingInformation/receptionDataContainer/ radioFrequency |
| PC5 Radio Technology | PC5 Radio Technology | /proseChargingInformation/receptionDataContainer/ pC5RadioTechnology |
|  |  | ChargingDataResponse |

# * * * * End of change * * * *