3GPP TSG-SA5 Meeting #160	S5-251536

Goteborg, Sweden, 07 - 11 April 2025

Source:	Huawei, Deutsche Telekom

Title:	pCR TS 28.567 Add use case and requirements for network performance problem recovery

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS 28.567

Version:	V0.1.0

Work Item:	CCLM

Comments

The use case and requirements for closed control loop for problem recovery are documented in TR 28.867 and recommended for normative work. It proposes to add use case and requirements for closed control loop for network problem recovery in TS 28.567.

Proposed Changes

* * * First Change * * * *

5.X	Closed Control Loops for network performance problem recovery

5.X.1		Description

Based on the concept in 3GPP TS 28.104 [3], MDA reports may contain root cause analysis of ongoing issues, predictions of potential issues and corresponding relevant causes and recommended actions for preventions, and/or prediction of network and/or service demands. For example:

	MDA for Coverage problem analysis can provide the following information in the MDA report:

-	coverageProblemId;

-	coverageProblemType;

-	coverageProblemAreas; and

-	recommendedActions.

MnS consumer may decide to resolve the observed performance problems based on the analytics reports (e.g. provided by MDA) and other management data (e.g. historical decisions made previously) if necessary. It can be possible that one MnF (e.g. Domain MnF) is responsible for network performance problem observation and recovery, while another MnF (e.g. Cross Domain MnF) is responsible for decision on whether the network performance problem needs to be resolved. In this scenario, The Cross Domain MnF can decide whether needs the Domain MnF to recovery the observed network performance problems (e.g. coverage problem) based on MDA report (e.g. root cause information, recommended solutions) and other information (e.g. user experience information, information from other domains). If decides to recover the observed network performance problems, Cross Domain MnF needs to request Domain MnF to recover the specified network performance problems observed from the MDA report by using closed control loop. MnS consumer may specifies the time window for network performance problem recovery, which means the MnS producer needs to recover the problem at the specified time window. During problem recovery phase, as process for network performance problem recovery is complex and time-consuming, the MnS consumer needs to obtain the progress of the recovery process. When the last step of the network performance problem process is completed, MnS producer needs to send the result of this network performance problem recovery process to the MnS consumers.

If a closed control loop instance can be used to resolve network performance problem, the MnS consumer may need to know the result of resolving the network performance problem by the closed control loop instance, including the network performance problems which are resolved by the closed control loop as well as network performance problem resolution statistics (e.g. the number of network problem resolved by the closed control loop in the specified period).

5.X.2	Requirements

| Requirement label | Description | Related use case(s)/ Motivation |
| --- | --- | --- |
| REQ-CCLPA-01 | The 3GPP management system should have the capability to allow the MnS consumer to request a CCL for resolving the network performance problems. | Closed Control Loops for network performance problem recovery |
| REQ-CCLPA-02 | The 3GPP management system should have the capability to allow the MnS consumer to obtain the result of network performance problem resolved by the closed control loop. | Closed Control Loops for network performance problem recovery |
| REQ-CCLPA-03 | The 3GPP management system should have the capability to allow the MnS consumer to obtain the progress information of network performance problem recovery. | Closed Control Loops for network performance problem recovery |

* * * End of Changes * * * *