3GPP TSG-SA5 Meeting #160	S5-251693

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0693 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS28.552 Add missing EP_N9 for GTP capacity related measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add missing EP_N9 for GTP capacity related measurements |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add missing EP_N9 for GTP capacity related measurements - Correct unit and cleanup |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing EP_N9 may cause measurement loss |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.44.1, 5.4.12.1, 5.4.12.2, 5.4.12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

##### 5.1.1.44.1	DL GTP capacity between UPF and NG-RAN

a)	This measurement provides the DL GTP capacity between PSA UPF and NG-RAN. This measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	The measurement is obtained by the following method: The gNB measures the maximum achievable GTP transmission rate between PSA UPF and NG-RAN for each 5QI or S-NSSAI, by counting the maximum achievable data volume for the measured 5QI or S-NSSAI for each time interval ([t, t + Δt]) during the collection period, taking the arithemetic peak value and then dividing it by Δt.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1Mbits=1000*1000 bits).

e)	GTP.CapMaxDlPsaUpfNgran.5QI, where 5QI identifies the 5QI; 
GTP.CapMaxDlPsaUpfNgran.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by GNBCUUPFunction).

g)	Valid for packet switched traffic.

h)	5GS.

2nd change

### 5.4.12	GTP capacity

#### 5.4.12.1	UL GTP capacity between PSA UPF and NG-RAN

a)	This measurement provides the UL GTP capacity between PSA UPF and NG-RAN. This measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF measures the maximum achievable GTP transmission rate between PSA UPF and NG-RAN for each 5QI or S-NSSAI, by counting the maximum achievable data volume for the measured 5QI or S-NSSAI for each time interval ([t, t + Δt]) during the collection period, taking the arithemetic peak value and then dividing it by Δt.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1Mbits=1000*1000 bits).

e)	GTP.CapMaxUlPsaUpfNgran.5QI, where 5QI identifies the 5QI; 
GTP.CapMaxUlPsaUpfNgran.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by UPFFunction);

EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.4.12.2	DL GTP capacity between PSA UPF and UE

a)	This measurement provides the capacity DL GTP capacity between PSA UPF and UE. This measurement is split into subcounters per S-NSSAI.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF measures the maximum achievable DL GTP transmission rate between PSA UPF and UE for each S-NSSAI, by counting the maximum achievable data volume for the measured S-NSSAI for each time interval ([t, t + Δt]) during the collection period, taking the arithemetic peak value and then dividing it by Δt.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1Mbits=1000*1000 bits).

e)	GTP.CapMaxDlPsaUpfUe.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by UPFFunction);

EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.4.12.3	UL GTP capacity between PSA UPF and UE

a)	This measurement provides the UL GTP capacity between PSA UPF and UE. This measurement is split into subcounters per S-NSSAI.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF measures the maximum achievable UL GTP transmission rate between PSA UPF and UE for each S-NSSAI, by counting the maximum achievable data volume for the measured S-NSSAI for each time interval ([t, t + Δt]) during the collection period, taking the arithemetic peak value and then dividing it by Δt.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1Mbits=1000*1000 bits).

e)	GTP.CapMaxUlPsaUpfUe.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by UPFFunction);

EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

End of changes

1st change

##### 5.1.1.44.1	DL GTP capacity between UPF and NG-RAN

a)	This measurement provides the DL GTP capacity between PSA UPF and NG-RAN. This measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	The measurement is obtained by the following method: The gNB measures the maximum achievable GTP transmission rate between PSA UPF and NG-RAN for each 5QI or S-NSSAI, by counting the maximum achievable data volume for the measured 5QI or S-NSSAI for each time interval ([t, t + Δt]) during the collection period, taking the arithemetic peak value and then dividing it by Δt.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1Mbits=1000*1000 bits).

e)	GTP.CapMaxDlPsaUpfNgran.5QI, where 5QI identifies the 5QI; 
GTP.CapMaxDlPsaUpfNgran.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by GNBCUUPFunction).

g)	Valid for packet switched traffic.

h)	5GS.

2nd change

### 5.4.12	GTP capacity

#### 5.4.12.1	UL GTP capacity between PSA UPF and NG-RAN

a)	This measurement provides the UL GTP capacity between PSA UPF and NG-RAN. This measurement is split into subcounters per 5QI and subcounters per S-NSSAI.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF measures the maximum achievable GTP transmission rate between PSA UPF and NG-RAN for each 5QI or S-NSSAI, by counting the maximum achievable data volume for the measured 5QI or S-NSSAI for each time interval ([t, t + Δt]) during the collection period, taking the arithemetic peak value and then dividing it by Δt.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1Mbits=1000*1000 bits).

e)	GTP.CapMaxUlPsaUpfNgran.5QI, where 5QI identifies the 5QI; 
GTP.CapMaxUlPsaUpfNgran.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by UPFFunction);

EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.4.12.2	DL GTP capacity between PSA UPF and UE

a)	This measurement provides the capacity DL GTP capacity between PSA UPF and UE. This measurement is split into subcounters per S-NSSAI.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF measures the maximum achievable DL GTP transmission rate between PSA UPF and UE for each S-NSSAI, by counting the maximum achievable data volume for the measured S-NSSAI for each time interval ([t, t + Δt]) during the collection period, taking the arithemetic peak value and then dividing it by Δt.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1Mbits=1000*1000 bits).

e)	GTP.CapMaxDlPsaUpfUe.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by UPFFunction);

EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

#### 5.4.12.3	UL GTP capacity between PSA UPF and UE

a)	This measurement provides the UL GTP capacity between PSA UPF and UE. This measurement is split into subcounters per S-NSSAI.

b)	DER (n=1).

c)	The measurement is obtained by the following method:

The UPF measures the maximum achievable UL GTP transmission rate between PSA UPF and UE for each S-NSSAI, by counting the maximum achievable data volume for the measured S-NSSAI for each time interval ([t, t + Δt]) during the collection period, taking the arithemetic peak value and then dividing it by Δt.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1Mbits=1000*1000 bits).

e)	GTP.CapMaxUlPsaUpfUe.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	EP_N3 (contained by UPFFunction);

EP_N9 (contained by UPFFunction).

g)	Valid for packet switched traffic.

h)	5GS.

End of changes