3GPP TSG-SA5 Meeting #160	S5-251729

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.557 | CR | 0017 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.557 Fix wrong references and correct solution for NPN-SC mgmt |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson GmbH, Eurolab |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM_NPN_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Fault supervision service is no longer defined in TS 28.532 from Rel-18; a new specification (TS 28.111, Fault Management) has been defined instead. - The usage of “tenant” concept (and their variants such as “tenant information” and “tenant context”) is confusing in this specification, and it seems to be used interchangeably with the NPN-SC role. Stakeholders and role should not be mixed, and the focus from standard standpoint is on the latter. - The solution for the use case “management of NPN service customer” (clause 6.1.2) is not correct. It includes an editor’s note reflecting that, when the solution was agreed, the discussion on MSAC was still ongoing. Once MSAC specification got published in Rel-18, some of the assumptions captured in this use case solution are no longer valid. For example, Identity, Role and permissionforMnS are not IOCs (so they cannot be operated using CRUD operations), permissionforMnS is an incorrect name, etc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add references to TS 28.111 (FM) and TS 28.319 (MSAC). - Replace TS 28.532 with TS 28.111, for references to fault supervision service. - Remove “tenant” related concepts, replacing them with NPN-SC as needed. - Update the solution for the use case “management of NPN service customer”, to make it aligned with Rel-18 solutions for access control in SA5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference for fault supervision service is incorrect. The solution for the use case “managemnent of NPN service customer” is wrong, building on IOCs that are not defined in 3GPP SA5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.2, 6.1.3, 6.1.5, 6.3.2, B.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[3]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[4]	3GPP TS 22.261: "Service requirements for the 5G system".

[5]	5G-ACIA White paper: "5G Non-Public Networks for Industrial Scenarios", July 31, 2019.

[6]	3GPP TS 23.003: "Numbering, addressing and identification".

[7]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[8]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[9]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[10]	3GPP TS 38.473: "NG-RAN; F1 Application Protocol (F1AP)".

[11]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[12]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[13]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[14]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[15]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)".

[16]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[17]	3GPP TS 28.537: "Management and orchestration; Management capabilities".

[18]	5G-ACIA: Exposure of 5G Capabilities for Connected Industries and Automation Applications, https://5g-acia.org/whitepapers/exposure-of-5g-capabilities-for-connected-industries-and-automation-applications-2/.

[19]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[20]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[21]	3GPP TS 28.533: "Management and orchestration; Architecture framework"

[22]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1"

[23]	3GPP TS 28.536: "Management and orchestration; Management services for communication service assurance; Stage 2 and stage 3"

[X]	3GPP TS 28.111: "Management and orchestration; Fault Management (FM)"

[Y]	3GPP TS 28.319: "Management and orchestration; Access Control for Management services”

Next Change

### 6.1.2	Solution for NPN fault management

In order to provide fault management capabilities scoping NPN and UEs representing 5G industry terminals, an NPN management system should monitor the fault of NPN and large quantity of 5G industry terminals which may be deployed in an enterprise.

The management services and performance data which are reused to achieve NPN fault management are following:

-	Generic fault supervision management service defined in TS 28.111 [X] .

-	Performance assurance management service defined in clause 11.3 of TS 28.532 [14].

-	MDA assisted fault management service defined in clause 7.2.3 of TS 28.104 [19].

-	Trace control and configuration management service defined in clauses 4.1, 4.2 and 6 of TS 32.422 [16].

The performance data collected at 5G industry terminals includes:

-	MDT data (e.g. UL/DL throughput volume, UL/DL throughput time used for calculation of UL/DL throughput, UL/DL packet delay per QoS level, UL/DL packet loss rate per QoS level, etc., defined in clause 4.34.1 of TS 32.423 [20].

Next Change

### 6.1.3	Solution for management of NPN service customer

The NPN service customer (i.e. NPN-SC) is a role defined for NPN management to represent the CSC (e.g. verticals) which consumes communication services for non-public use (see clause 4.2). The management of NPN-SC focuses on the granular access control of NPN-SC,

which is used to specify and authorize the specific  MnS information (e.g., MnS type and MnS components) the NPN-SC is allowed to consume.

The procedure for the management of NPN-SC is described below. The pre-condition of the procedure is that the NPN-SC and NPN-SP/OP have reached a business agreement.

First, the NPN-SC registers into NPN-SP.

Following NPN-SC registration, the NPN-OP hosting access control MnS producer (i.e. Authentication and Authorization MnS producer) identifies the management capabilities that are to be exposed to the NPN-SC, and configure authentication/authorization settings in the system for that tenant. To that end, Management Service Access Control solution defined in TS 28.319 [X] can be used.

Next Change

### 6.1.5	Solution of network slice allocation considering shared/dedicated resources requirements in NPN

To satisfy the shared/dedicated resources demand for NPN service customers, the provisioning MnS producer may provision isolation profiles which contain sets of isolation requirements when configuring managed resources within a PNI-NPN or within an SNPN.

Figure 6.1.5-1 shows the process on network slice allocation considering resource isolation requirements.

Figure 6.1.5-1: Process on network slice allocation considering shared/dedicated resources requirements

1)	NPN-SCidentifies the SLA requirements which includes the shared/dedicated resources demand based on its target NPN-SC applications, such as smart grid applications.

2)	NPN-SC sends the shared/dedicated resources requirements to NPN-SP.

3)	NPN-SP compares received requirements against the defined set of IsolationProfile MOI, and decides to create a new IsolationProfile MOI, or reuse the existing IsolationProfile MOI with modification if necessary.

4)	NPN-SP requests to create/modify instance of IOC IsolationProfile defined in TS 28.541 [7].

5)	NNP-OP returns the isolation profile creation/modification results, which contains the information of IsolationProfile instance if the allocation succeeds.

6)-8)	Same as steps 1)-3) in clause 6.3.1. The NSI allocation request in step 8 can be issued with one of the following operations defined in clause 6.5.1 of TS 28.531 [8]: allocateNsi or CRUD operations with NetworkSliceController IOC.

9)	When allocating NSI (NSSI) for NPN-SC who has the shared/dedicated resource demand, the NPN-OP associates IsolationProfile instance(s) that allocated in Step 5 to NSI (NSSI).

10)	Same as step 5) in clause 6.3.1 with additional association relations with IsolationProfile instance(s).

Since the NSI is allocated for NPN service customer satisfying its dedicated or shared resource demand derived from the targeted running NPN-SC applications, there are several interpretations with different association scenarios between NSI(s) and IsolationProfile MOI (s).

For one IsolationProfile MOI associated to two or more NSIs, see Figure 6.2.1-6: IsolationProfile NRM fragment relationship related to NetworkSlice in TS 28.541 [7], these NSIs are configured with same resource isolation rule:

-	If the resource isolation rule requires the resources to be shared, which means the attribute isolationRule is SHARED, only NSIs associated to this IsolationProfile MOI could share the resources with each other.

-	If the resource isolation rule requires the resources to be dedicated, which means the attribute isolationRule is DEDICATED, NSIs associated to this IsolationProfile MOI are supported by dedicated resources exclusively.

For one IsolationProfile MOI associated to one NSI, see Figure 6.2.1-6: IsolationProfile NRM fragment relationship related to NetworkSlice in TS 28.541 [7], if the isolationRule is SHARED, it means the network slice resources can be shared with other NSI(s), which helps the Provisioning MnS producer make choices when allocating other NSIs.

Figure 6.1.5-2: Example of solutions for shared/dedicated resource demand for NPN-SC

Figure 6.1.5-2 illustrates an example of solutions for shared/dedicated resource demand for NPN-SC.

The dedicated-shared resource requirements come from the use case described in clause 5.1.0.5. Different NSIs are allocated for each L-Group to fulfil the dedicated logical resources requirement. For NSIs allocated for same P-Group, they are associated with the same isolation profile instance with isolationRule equals to SHARED. For NSIs allocated for different P-Group, their associated isolation profile instances are different.

Figure 6.1.5-3: Example of solutions for dedicated resource demand for NPN-SC

Figure 6.1.5-3 illustrates an example of solutions for dedicated resource demand for NPN-SC. Since there is only one L-Group, the NSI allocated for this group is associated with isolation profile instance where isolationRule equals to DEDICATED.

Next change

### 6.3.2	Solution for exposure of management capability of PNI-NPN

The MNO (playing the role of NPN-SP) provides a PNI-NPN for a vertical (playing the role of NPN-SC) in the form of a network slice of a PLMN. This solution can be used by a vertical to obtain certain management capabilities (e.g., provisioning, performance monitoring) exposed by MNO in MNO-Vertical Managed Mode. The vertical consumes the exposed management capabilities to manage the PNI-NPN, including:

-	Using the Network Slice Provisioning exposure management services and Network Slice Provisioning data report exposure management service (see clause 6.1 of TS 28.531 [8]), NPN-SC can request allocating, deallocating modifying an NSI, or obtain notifications about NSI Information model data.

-	Using exposure of generic fault supervision management service (see TS 28.111 [X] ) and generic performance assurance management service (see clause 11.3 of TS 28.532 [14]), NPN-SCs can create certain measurement jobs and select the type of data analytics and performance to be monitored, e.g., performance related to various traffic types, geographical areas, different device types, for a specific group of devices, for certain traffic congestion situation and analytical KPIs related to performance predictions.

Figure 6.3.2-1: Exposure of management capability of PNI-NPN in MNO-Vertical Managed Mode

Next change

## B.5	Void

End of Changes