3GPP TSG-SA5 Meeting #160	S5-251795

# Goteborg , Sweden, 7-11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0197 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel19 CR TS28.104 solution on management data correlation analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The use cases of “Measurement data correlation analytics for ML training” have been discussed and agreed upon in the study (clause 5.3.2 in TR 28.866 and TS 28.104 clause 7.2.9.1). This CR is to normatively propose the solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding solution for the use case description and requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution for the use case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.x (new), 8.5.x(new), 8.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-251440 |  |  |  |  |  |  |  |  |

Start of First change

### 8.4.x	Correlation analytics

#### 8.4.x.1	Measurement data correlation analytics for ML training

##### 8.4.x.1.1	MDA type

The MDA type for ML training data Correlation analysis is: CorrelationAnalytics.TrainingDataAnalysis.

##### 8.4.x.1.2	Enabling data

The enabling data for CorrelationAnalytics.TrainingDataAnalysis MDA type are provided in table 8.4.x.1.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.x.1.2-1: Enabling data for ML training data Correlation analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | The performance metric or KPI data used for ML training. | The performance metric data defined in TS 28.552 [4], and KPI data defined in TS 28.554 [5] |

##### 8.4.x.1.3	Analytics output

The specific information elements of the analytics output for management data correlation analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.1.3-1.

Table 8.4.x.1.3-1: Analytics output for correlation analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| measurementDataCorrelationRecommendation | The attribute indicates measurement data correlation analytics recommendation. This attribute may carry null value, which may indicate no recommendation. allowedValues: N/A | M | type: MeasurementDataCorrelationRecommendation multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |

Next change

### 8.5.x	MeasurementDataCorrelationRecommendation <<dataType>>

#### 8.5.x.1	Definition

This data type specifies the measurement data correlation analytics recommendation. The data type can be the contents of the analytics report representing the recommendations from MDA for the measurement data correlation analytics for ML training.

#### 8.5.x.2	Information elements

Table 8.5.x.2-1

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| recommendedMeasurementDataToCollect | The attribute indicates the measurement data which is recommended to be collected. allowedValues: The list may include metrics or set of metrics defined in TS 28.552 [4], TS 28.554 [5] and TS 32.422 [6]. For performance measurements defined in TS 28.552 [4] the name is constructed as the bullet e) of measurement definition with allowed measurement type. For trace metrics (including trace messages, MDT measurements (Immediate MDT, Logged MDT, Logged MBSFN MDT), RRC, RLF and RCEF reports) defined in TS 32.422 [6], the name (metric identifier) is defined in clause 10 of TS 32.422 [6]. For non-3GPP specified managment data the name is defined elsewhere. | M | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendedMeasurementDataNotToCollect | The attribute indicates the measurement data which is recommended not to be collected. allowedValues: refer to allowed values in attribute recommendedMeasurementDataToCollect | M | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| modelPerformanceImpact | The attribute indicates the model performance impact. It is a percentage indicates the loss the model performance from trained MLModel with generated measurement data comparison to the performance trained with full measurement data. E.g., 3% means the model performance for the MLModel trained with generated measurement data is 3% worse than the performance trained with full measurement data. The consumer may use the value of this attribute to help decide on whether to accept or not accept the recommendation. allowedValues: 0..100 | O | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

Next change

### 8.6.1	MDAType <<enumeration>>

Table 8.6.1-1: <<enumeration>> MDAType

| Enumeration value | Description |
| --- | --- |
| "COVERAGEANALYTICS_COVERAGEPROBLEMANALYSIS" | Indicates that the MDA type for the Coverage problem analysis defined in clause 8.4.1.1 |
| "COVERAGEANALYTICS_PAGINGOPTIMIZATION" | Indicates that the MDA type for the Paging Optimization defined in clause 8.4.1.2 |
| "SLSANALYSIS_SERVICEEXPERIENCEANALYSIS" | Indicates that the MDA type for the Service experience analysis defined in clause 8.4.2.1 |
| "SLSANALYSIS_NETWORKSLICETHROUGHPUTANALYSIS" | Indicates that the MDA type for the Network slice throughput analysis defined in clause 8.4.2.2 |
| "SLSANALYSIS_NETWORKSLICETRAFFICANALYSIS" | Indicates that the MDA type for the Network slice traffic prediction defined in clause 8.4.2.3 |
| "SLSANALYSIS_E2ELATENCYANALYSIS" | Indicates that the MDA type for the E2E latency analysis defined in clause 8.4.2.4 |
| "SLSANALYSIS_NETWORKSLICELOADANALYSIS" | Indicates that the MDA type for the Network slice load analysis defined in clause 8.4.2.5 |
| "MDAASSISTEDFAULTMANAGEMENT_FAILUREPREDICTION" | Indicates that the MDA type for the MDA assisted failure prediction defined in clause 8.4.3.1 |
| "MDAASSISTEDENERGYSAVING_ENERGYSAVINGANALYSIS" | Indicates that the MDA type for the Energy saving analysis defined in clause 8.4.4.1 |
| "MOBILITYMANAGEMENTANALYTICS_MOBILITYPERFORMANCEANALYSIS" | Indicates that the MDA type for the Mobility performance analysis defined in clause 8.4.5.1 |
| "MOBILITYMANAGEMENTANALYTICS_HANDOVEROPTIMIZATION" | Indicates that the MDA type for the Handover Optimization analysis defined in clause 8.4.5.2 |
| "MAINTENANCE_MAINTENANCEANALYTICS" | Indicates that the MDA type for the Maintenance.MaintenanceAnalytics defined in clause 8.4.6.1 |
| "RESOURCEANALYTICS_VIRTUALIZEDRESOURCEUTILIZATIONANALYSISNF" | Indicates that the MDA type for the Virtualized resource utilization analysis defined in clause 8.4.7.1.1 |
| "RESOURCEANALYTICS_PHYISCALRESOURCEUTILIZATIONANALYSISNF" | Indicates that the MDA type for the Physical resource utilization analysis defined in clause 8.4.7.1.2 |
| "RESOURCEANALYTICS_5GCCONTROLPLANECONGESTIONANALYSIS" | Indicates that the MDA type for the 5GC Control plane congestion analysis defined in clause 8.4.7.1.3 |
| "PREDICTIONS_PMDATA" | Indicates that the MDA type for the MDA assisted PM predictions defined in clause 8.4.8.1 |
| "UETHROUGHPUTANALYTICS_UETHROUGHPUTPROBLEMANALYSIS" | Indicates that the MDA type for the UE throughput analysis defined in clause 8.4.2.6 |
| “CORRELATIONANALYTICS_TRAININGDATAANALYSIS” | Indicates that the MDA type for the Correlation analysis defined in clause 8.4.x.1 |

End of change