3GPP TSG-SA5 Meeting #160	S5-251908

Goteborg, Sweden, 07 - 11 April 2025	Revision of S5-251243

Source:	Nokia

Title:	Pseudo-CR on CCL network & time scope conflicts

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.1.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add use case and requirements for CCL scope & action-execution conflicts handling as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 5	Management Capabilities

## 5.A	 CCL Conflict management Capability - CONF

### 5.A.2	Use Cases

#### 5.A.2.K	CCL network scope conflicts handling – CONF_0x

Each CCL should have specific scopes for which it is responsible. The network may be assumed to be a p-dimensional space SP from which subregions dP Є D maybe created. Accordingly, SP is the full scope space whose dimension may include time, geography, while dp Є D can be CCL's scope. Scope assignment is the mapping of CCLs to regions dЄD that are part of the network's full scope S.

The 3GPP management system should support the capability to coordinate the scope assignment to enable detection and avoidance of potential scope conflicts. It may be desirable to define the full scope space Sp and a set of scope rules to be used to derive the best scope to be assigned to each CCL. An example rule may be that the defined CCL scope should not overlap. The rules may for example be defined by an operator or can be implementation specific depending on the types of CCLs that are to be configured.

Scope conflicts are only considered actual if their use results in negative outcomes. The management system should support the capability to coordinate the scope assignment to detect and resolve actual scope conflicts. The CCLs monitor changes in their scope. If the scope is changed, it is desirable for the CCLs to notify the scope assignment MnS consumer of the changes or differences between what was configured and the actual scopes. The scope assignment MnS consumer may then trigger scope conflict evaluation based on the actual scope.

#### 5.A.2.Y	CCL action-execution-time conflicts handling - CONF_0y

Several CCLs may want to execute actions onto the network. It may not be desirable that their actions are executed within the same time frame. For example, if executed so close to one another, their effects will be super-imposed and neither CCL can identify the effect of its actions on the network.

The management system should support the capability for detection of potential action-execution-time conflicts. An MnS consumer acting as a supervisory action-critic oversees the actions of the different CCLs may need to receive information enabling the detection of such conflicts. The action-critic functionality takes the for the end-to-end performance of the Network enabling evaluation of cases when the actions of multiple CCLs collide.

For a given CCL, the MnS consumer may need to receive the recommended changes from the CCLs, to evaluates them and see if they overlap with other proposed changes from other CCLs. Where there are likely conflicts and expected undesired impacts, the MnS consumer may propose to the CCLs, the changes that should be undertaken to minimize concurrent changes on the same network resources. The MnS consumer may need to provide feedback to the CCL instance (s) regarding their recommended actions.

In some instances, the conditions in the network may be such that it is not clear which CCL should be triggered, requiring to trigger multiple CCL in sequence. The CCLs may operate in a hierarchy with each CCL having an operational profile indicating the specific level of hierarchy. The MnS consumer that coordinates the execution times of the CCLs needs to configure the appropriate hierarchy for the CCLs. The triggering by a coordination capability based on information from the CCL allows resolution of CCL Trigger-time conflicts.

### 5.A.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-CONF_0x-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL scope conflicts. | CONF-CONF_0x |
| REQ-CONF_0x-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL scope conflict after it is confirmed. | CONF-CONF_0x |
| REQ-CONF_0x-03 | The 3GPP Management System should support a capability to avoid or resolve a CCL scope conflict that has been detected | CONF-CONF_0x |
| REQ-CONF_0x-04 | The 3GPP Management System should support a capability to coordinate the resolution of CCL scope conflicts among multiple CCLs | CONF-CONF_0x |
| REQ- CONF_0y-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential or actual CCL action-execution-time targets conflicts. | CONF-CONF_0y |
| REQ-CONF_0y-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about a detected CCL action-execution-time targets conflict after it is confirmed. | CONF-CONF_0y |
| REQ-CONF_0y-03 | The 3GPP Management System should support a capability to avoid or resolve a CCL action-execution-time targets conflict that has been detected | CONF-CONF_0y |
| REQ-CONF_0y-04 | The 3GPP Management System should support a capability enabling the MnS consumer to configure a hierarchy of a CCL | CONF-CONF_0y |

* * * End of Changes * * * *