3GPP TSG-SA5 Meeting #160	S5-251337

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0680 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.552 Add a new measurement related to efficiency of data collection from DCCF or NWDAF hosting DCCF performed by NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NWDAF_OAM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-3-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NWDAF data collection may be performed via DCCF, when DCCF is deployed or via NWDAF hosting DCCF. The amount of data collected by NWDAF from each notification and/or response from DCCF/NWDAF hosting DCCF varies. The number of these notification and/or response received by NWDAF is proportional to the overhead of NWDAF data collection via DCCF/NWDAF hosting DCCF directly. For a given time length during the observation, the average amount of data collected from the DCCF/NWDAF hosting DCCF per notification and response can be measured. This measurement shows the relationship between total amount of data collected by NWDAF via DCCF/NWDAF hosting DCCF directly and the corresponding transmission overhead. It reflects the efficiency on the transmission aspect of one NWDAF collecting data from DCCF/NWDAF hosting DCCF directly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new measurement “Efficiency of data collection from DCCF or NWDAF hosting DCCF performed by NWDAF” is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator will not be able to monitor the data efficiency of NWDAF performing data collection from DCCF/NWDAF hosting DCCF and to take management actions for configuration and resource allocation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

5.18	Performance measurements for NWDAF

5.18.5	Measurements related to the NWDAF Data Collection

5.18.5.x Efficiency of NWDAF data collection from DCCF/NWDAF hosting DCCF

a) 	This measurement provides the estimation of the efficiency of NWDAF data collection via DCCF/NWDAF hosting DCCF

b) 		Gauge

c)	This measurement is obtained by averaging the estimation of the total amount of data collected from one or multiple DCCF instance(s) or NWDAF instance(s) hosting DCCF with the total number of notification and response corresponding to the subscription and request invoked by NWDAF to collect data from the aforesaid one or multiple DCCF instance(s) or NWDAF instance(s) hosting DCCF. The measurement can be splited into sub-counters per DCCF instance or NWDAF instance hosting DCCF.

d) 	A real value

e) 		The measurement name has the form DANS.DataCollectEfficiencyWithDCCF.DataSourceID, where the DataSourceID identifies a DCCF instance or a NWDAF instance hosting DCCF

f) 		NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

End of Changes