3GPP TSG-SA5 Meeting #160	S5-252030

Goteborg, Sweden, 07 - 11 April 2025 	Revision of S5-251249

Source:	Nokia, Samsung

Title:	Pseudo-CR on CCL General NRM

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.1.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add NRM for the CCL as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 6	Model

## 6.1	Imported and associated information entities

TBD

### 6.1.1	Imported information entities and local labels

TBD

### 6.1.2	Associated information entities and local labels

TBD

## 6.2	Class diagram

#### 6.2.1	Relationships

Figure 6.2.1.1-1: Relations for common information models for AI/ML management

Note: The handling of Goal, targets or objectives for the general closed control loops is FFS

#### 6.2.2	Inheritance

Figure 6.2.1.2-1: Inheritance Hierarchy for Closed Control Loops

### 6.3	Class definitions

#### 6.3.1	ClosedControlLoop

6.3.1.1	Definition

This IOC represents the closed control loop. It represents the information for controlling and monitoring a CCL associated with a stated scope.

The ClosedControlLoop is name-contained by SubNetwork or ManagedElement and is associated with a CCLreport that contains reported  information about the CCL. Accordingly, the report about a CCL can exist even when the CCL is deleted.

The capabilities of the CCL are contained in one or more CCLPurposes that describe what the CCL is capable of doing or can be configured to do - including information the network resources for which the CCL can execute decisions and actions. So, the ClosedControlLoop is associated with one or more CCLPurpose(s) that indicate(s) a list of characteristics that describe what a CCL can/is expected to be able to do. The purpose describes the type of functionality that can be executed including problem recovery and fault management .

The operational information about the CCL is contained in the CCLScope(s), so the ClosedControlLoop is associated with one or more CCLScope(s). The CCLScope defines what the CCL has been configured to read, evaluate, control, etc.

6.3.1.2	Attributes

The CCLControlLoop IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.1.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attribute related to role |  |  |  |  |  |
| cCLPurposeRefList | M | T | T | T | T |

6.3.1.3	Attribute constraints

None

6.3.1.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

##### 6.3.2	CCLScope

6.3.2.1	Definition

It indicates a scope of a CCL. It may be the measurement scope, control scope or impact scope.

The CCLScope includes the attribute scopeType that indicates the type of scope that represented by the particular scope instance.

6.3.2.2	Attributes

The CCLScope IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.2.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| scopeType | O | T | F | F | T |

6.3.2.3	Attribute constraints

None.

6.3.2.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

##### 6.3.3	CCLReport

6.3.3.1	Definition

This class represents the reported outcomes on a CCL instance, e.g., the information about the outcomes on one or the executing of the CCL. An CCLReport is contained by the entity containing the CCL, since the CCLReport can exist beyond the life of the CCL on which it is reporting.

There is one CCLReport per CCL for an observation time. The content of the CCLReport may be different for different observation time.

6.3.3.2	Attributes

The CCLReport IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.3.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attributes related to role |  |  |  |  |  |

6.3.3.3	Attribute constraints

None.

6.3.3.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

##### 6.3.4	CCLPurpose <<dataType>>

6.3.4.1	Definition

This data type represents a single purpose that describes what a CCL can do. The purpose is alist of characteristics that describe the capabilities of the CCL.

6.3.4.2	Attributes

Table 6.3.4.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attributes related to role |  |  |  |  |  |

6.3.4.3	Attribute constraints

None.

6.3.4.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions..

6.4	Attribute definitions

6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| scopeType | It indicates the type of scope that represented by the particular scope instance. allowedValues: CCL_MEASUREMENT_SCOPE, CCL_TARGET_SCOPE, CCL_CONTROL_SCOPE, CCL_IMPACT_SCOPE Note: The allowed values will be revisited | type: Enum multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * Next Change * * * *

Annex A (informative):
UML code for model diagrams

# A.X	UML code for CCL management model diagrams

This annex contains the PlantUML source code for the NRM diagrams defined in clause 6.2 of the present document.

# A.X.1	CCL NRM fragment (Figure 6.2.1.1-1)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class ManagedEntity <<ProxyClass>>

class ClosedControlLoop <<InformationObjectClass>>

class CCLPurpose << ProxyClass >>

class CCLScope << InformationObjectClass >>

class CCLReport <<InformationObjectClass>>

ManagedEntity "1" *-- "*" ClosedControlLoop: <<names>>

ClosedControlLoop "1" *--> "*" CCLPurpose

ClosedControlLoop "1" *-- "*" CCLScope: <<names>>

ClosedControlLoop "1" *-- "*" CCLReport: <<names>>

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedElement

end note

note top of CCLPurpose

Can be any of these CCL purposes:

NetworkProblemRecovery

FaultManagement

...

end note

@enduml

# A.X.2	CCL inheritance relationships (Figure 6.2.1.1-2)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top << InformationObjectClass >>

class ClosedControlLoop <<InformationObjectClass>>

class CCLReport <<InformationObjectClass>>

class CCLScope <<InformationObjectClass>>

Top <|-- ClosedControlLoop

Top <|-- CCLScope

Top <|-- CCLReport

@enduml

* * * End of Changes * * * *