3GPP TSG-SA5 Meeting #160	S5-252089

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.560 | CR | 0006 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addressing EN for supporting type of signalling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone, Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Monstra-OAM |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The contribution aims to address the following editors note: “Editor's Note: To further define the type of signalling traffic/messages that need to be monitored.” The SA1 requirements related to Monstra state the follwing: “The 5G network shall enable the MNO to configure network monitoring, e.g., switching on/off per network element, selecting what type of elements and what type of signalling from these elements is the target for monitoring.” The present specification supports monitoring only the signalling traffic of selected interface(s) of selected NF(s). This will result in all the signalling traffic on a particular interface (networkInterfaceTypeList) from a particular NF (reportingNFList). For illustration purpose, the following table shows the amount of signalling traffic that will be sent for N11 from AMF. As stated in Clause 4.2.7 of 23.501, N11 is realized by corresponding NF service-based interfaces i.e Namf and Nsmf. Hence, traffic related with all the service operation shown in following two figures will be copied and sent to the external entity. This huge amount of data being sent may not be useful for a specific purpose. Hence, there is a need to specify further filtrations criteria (signalling type) to identify the traffic that should be sent. For example, the monitoring may be required only for EBIAssignment of N11. Rev.1: The new attribute is changed to Optional, and allowed values are added to the network interface type |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new requirement for identifying the type of signalling messages to be copied. Add related stage 2 and stage 3 content to fulfill the requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4, 6.1.1, 6.2.1.1, 6.2.1.2, 6.2.x (new), 6.2.y (new), 6.3.1, A.1, 7.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1637 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev. 1: new attribute changed to optional and list of interface names added as allowed values. Minor typo fixed in the name of the OpenAPI document. Cleaned cover sheet |  |  |  |  |  |  |  |  |

First Modified Section

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.532: "Generic management services".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 33.501: " Security architecture and procedures for 5G System ".

[5]	3GPP TS 28.622: "Generic Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)".

[6]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[7]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[8]	IETF RFC8086: GRE-in-UDP Encapsulation.

[9]	IETF draft-ietf-opsawg-pcapng-04: PCAP Next Generation (pcapng) Capture File Format.

[10]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[11]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[X]		3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2"

Next Modified Section

# 4	Signalling traffic monitoring management capabilities (stage 1)

The 3GPP management system shall have the capabilities as listed in the following table.

Table 4-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-SM-FUN-1 | The 3GPP system shall have the capability to send copies of the 5G Core control plane signalling messages, of selected interface(s) of selected NF(s), to an external entity that collects these signalling messages. | Signalling Traffic Monitoring Streaming |
| REQ-SM-FUN-2 | Authorised consumers shall have the capability to control (to configure, to enable, and to disable) sending copies of the 5G Core control plane signalling messages to the external collecting entity. | Signalling Monitoring Activation Signalling Monitoring Termination |
| REQ-SM-FUN-3 | The 3GPP system shall have the capability to send the copies of the 5G Core control plane signalling messages in a reliable or unreliable transport manner. | Signalling Traffic Monitoring Streaming |
| REQ-SM-FUN-4 | Authorised consumers shall have the capability to indicate the type of signalling messages, the copies of which is to be sent to an external entity that collects these signalling messages. Note: The type of signalling messages is identified by the “Service Operations” (e.g EBIAssignment) (see clause 5.2 of 23.502[x]) of the services exposed by the NF(s). | Signalling Traffic Monitoring Activation |

Next Modified Section

# 6	Signalling traffic monitoring management Information Model (stage 2)

## 6.1 	Imported and associated information entities

### 6.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 28.622 [5], IOC, Subnetwork | SubNetwork |
| 3GPP TS 28.622 [5], IOC, ManagedElement | ManagedElement |
| 3GPP TS 28.622 [5], IOC, ManagedFunction | ManagedFunction |

### 6.1.2	Class diagram

#### 6.1.2.1	Relationships

This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes. The following figure shows the containment/naming hierarchy and the associations of the classes defined in the present document.

Figure 6.1.2.1-1: STM control NRM fragment

#### 6.1.2.2	Inheritance

This clause depicts the inheritance relationships.

Figure 6.1.2.2-1: STM control NRM inheritance relationships

## 6.2	Class definitions

### 6.2.1	StmCtrl <<IOC>>

#### 6.2.1.1	Definition

This IOC represents STM jobs. It can be name-contained by SubNetwork, ManagedElement, or ManagedFunction.

Editor's note: To check whether we need an identifier for an object instance beyond DN.

reportingNFList specifies the network function whose signalling traffic is to be monitored. If this parameter is not present or it is empty, then all Network Functions within the SubNetwork or ManagedElement shall be monitored. This parameter shall be omitted if the STM control object is specified under a ManagedFunction.

networkInterfaceTypeList specifies the target network interface type to be monitored. If this parameter is not present or it is empty, then all applicable interface types from the target NF shall be monitored.

Editor's note: To further define how specific interface instances is to be identified.

stmTargetUri is the URI of the STM consumer that shall receive the monitored signalling message copies.

administrativeState is used by the STM consumer to lock or unlock the StmCtrl instance in order to stop or start the signalling traffic monitoring.

operationalState is used by STM consumer to report its working state.

targetSignallingType is used by the STM consumer to specify the type of signalling traffic that need to be copied and sent to the external entity. This will indicate service operation(s) exchanged over the interface(s) specified in networkInterfaceTypeList, see clause 5.2 of 23.502 [x]

#### 6.2.1.2	Attributes

The StmCtrl IOC includes attributes inherited from Top IOC (defined in 3GPP TS 28.622 [5] subclause 4.3.29) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| reportingNFList | CM | T | T | F | T |
| networkInterfaceTypeList | M | T | T | F | T |
| stmTargetUri | M | T | T | T | T |
| administrativeState | M | T | T | F | T |
| operationalState | M | T | F | F | T |
| targetSignallingType | O | T | T | F | T |

Editor's note: To check whether we need an identifier for an object instance beyond DN.

#### 6.2.1.3	Attribute constraints

None.

#### 6.2.1.4	Notifications

The common notifications defined in 3GPP TS 28.622 [5] subclause 4.5 are valid for this IOC, without exceptions or additions.

### 6.2.x	TargetSignallingType <<Data Type>>

#### 6.2.x.1	Definition

This defines the type of signalling traffic that need to be copied and sent to the external entity. This will include service operation(s), see clause 5.2 of 23.502 [x].

#### 6.2.x.2	Attributes

The TargetSignallingType contains the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceOperation | M | T | T | F | T |
| networkInterface | M | T | T | F | T |

#### 6.2.x.3	Attribute constraints

None.

#### 6.2.x.4	Notifications

The common notifications defined in 3GPP TS 28.622 [5] subclause 4.5 are valid for this IOC, without exceptions or additions.

### 6.2.y	ServiceOperation <<choice>>

#### 6.2.y.1	Definition

This <<choice>> defines the service operation(s) which is requested. It is a choice between

- 	All service operation

- 	a list of required service operations

#### 6.2.y.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| CHOICE_1.1 aLL | M | T | T | T | N/A |
| CHOICE_2.1 serviceOperationList | M | T | T | T | N/A |

#### 6.2.y.3	Attribute constraints

None.

#### 6.2.y.4	Notifications

The clause 4.5 of the <<IOC>> using this <<choice>> as one of its attributes, shall be applicable.

Next Modified Section

## 6.3	Attribute definitions

### 6.3.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| reportingNFList | List of Network Function Distinguished Name. allowedValues: N/A | Type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| networkInterfaceTypeList | List of network interface type. The applicable network interface type names are specified based on subclause 4.2.3 of 3GPP TS 23.501 [7]. The value "ALL" is specified for the case if all the applicable interface type of the network function shall be monitored. allowedValues: ALL, N4, N5, N7, N8, N10, N11, N12, N13, N14, N15, N22, N58, N59, N80, N81. | Type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: ALL isNullable: False |
| stmTargetUri | It specifies the Uniform Resource Identifier (URI) of the streaming target where the signalling traffic shall be sent. The detailed URI structure is defined in clause 4.4 of 3GPP TS 32.158 [6]. | type: Uri multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| administrativeState | Administrative state of a managed object instance. The administrative state describes the permission to use or prohibition against using the object instance. The administrative state is set by the STM consumer. allowedValues: LOCKED, UNLOCKED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| operationalState | Operational state of managed object instance. The operational state describes if an object instance is operable ("ENABLED") or inoperable ("DISABLED"). This state is set by the object instance or the STM producer and is hence READ-ONLY. allowedValues: ENABLED, DISABLED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLED isNullable: False |
| targetSignallingType | This defines the type of signalling traffic, exchanged over each network interface listed in networkInterfaceTypeList, that need to be copied and sent to the external entity. | Type: TargetSignallingType multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TargetSignallingType. networkInterface | This defines the interface on which the serviceOperation(s) is requested. allowedValues: N2, N4, N5, N7, N8, N10, N11, N12, N13, N14, N15, N16, N17, N18, N20, N21, N22, N23, N26, N28, N29, N30, N33, N34, N35, N36, N37, N40, N41, N42, N51, N52, N58, N59, N60, N61, N62, N63, N80, N81, N82, N83, N84, N85, N86, N87, N88, N89, N96, NL1, NL2, NL5, NL6, NL8, NL9. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TargetSignallingType. serviceOperation | This defines required service operation(s) exchanged over the interface specified by networkInterface. | Type: ServiceOperation multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ServiceOperation.aLL | This will indicate if all the service operation are to be copied and sent to the external entity. The value TRUE indicate that all the service operation shall be copied and sent to the external entity. | Type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| ServiceOperation. serviceOperationList | This will indicate the list of service operation(s) that are to be copied and sent to the external entity, see clause 5.2 of 23.502 [x] | Type: String multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

## 6.4	Common notifications

This clause presents a list of notifications, defined in 3GPP TS 28.533 [3], that a STM consumer can receive.

| Name | S | Notes |
| --- | --- | --- |
| notifyMOICreation | O |  |
| notifyMOIDeletion | O |  |
| notifyMOIAttributeValueChanges | O |  |
| notifyMOIChanges | O |  |

Last Modified Section

Annex A (informative):
Use Cases

# A.1 	Signalling Monitoring Activation

External monitoring systems are often used by mobile network operators (MNOs) to track network activity for analysis and troubleshooting purposes, and subsequently to perform diagnosis and analysis of their system. Such monitoring system(s) are fully under the control of the MNOs, and the monitoring is performed at signalling level.

The goal of this use case is to enable the authorized consumer (signalling monitor consumer) to activate the streaming of packets from a specific NF or entity acting on behalf of the NF (signalling monitor producer) for a specific interface and service operation. In this activation it will be requested the desired reliability for the streaming of the signalling traffic, along with the targeted interface(s).

Next Modified Section

## 7.x	OpenAPI document for StmCtrl NRM

The OpenAPI/YAML definitions for StmCtrl NRM are specified in 3GPP Forge, refer to clause 4.3 of TS 28.623 [19] for the Forge location. An example of Forge location is: "https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel18_SA104/".

Directory: OpenAPI

File: TS28560_StmCtrl.yaml

End of Modified Section