| 3GPP TS 28.572 V0.4.0 (2025-04) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Management of planned configurations (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Concept and overview	8

5	Requirements	8

6	Solution description	10

6.1	Planned configurations	10

6.1.1	Definition	10

6.1.2	Format of planned configurations	11

6.1.3	Meta data for planned configurations	11

6.1.4	Planned configuration descriptor	12

6.2	Planned configuration groups	12

6.2.1	Definition	12

6.2.2	Planned configuration group members	12

6.2.3	Conflicts between planned configuration group members	13

6.2.4	Meta data for planned configuration groups	13

6.2.5	Planned configuration group descriptor	13

6.3	Fallback configurations	13

6.3.1	Definition	13

6.3.2	Fallback configuration preparation	14

6.3.3	Meta data for fallback configurations	15

6.3.4	Fallback configuration descriptor	15

6.4	Trigger conditions	15

6.4.1	Definition	15

6.4.2	Usage	15

6.4.3	Specification of trigger conditions	15

6.4.4	Meta data for trigger conditions	16

6.4.5	Control parameters for trigger conditions	16

6.4.6	Monitor parameters for trigger conditions	16

6.4.7	Trigger condition descriptor	16

6.5	Validation of planned configurations	16

6.5.1	Definition	16

6.5.2	Potential problems	16

6.5.3	Validation process	17

6.5.4	Validation of planned configuration groups	18

6.5.5	Validation of fallback configurations	18

6.5.6	Validation control parameters	18

6.6	Activation of planned configurations	18

6.6.1	Definition	18

6.6.2	Potential problems	18

6.6.3	Activation process	19

6.6.4	Activation of planned configuration groups	19

6.6.5	Activation control parameters	19

6.7	Conditional activation of planned configurations	19

6.8	Asynchronous configuration updates	19

6.9	Fallback	20

6.10	Planned configuration lifecycle	20

6.11	Managing planned configurations	20

6.13	Managing fallback configurations	20

6.14	Managing trigger conditions	20

6.15	Managing validations	20

6.16	Managing activations	21

6.17	Notifications	22

7	Information model	22

7.1	PlannedConfigurationDescriptor	22

7.1.1	Definition	22

7.1.2	Information Elements	22

7.2	PlannedConfigurationGroupDescriptor	24

7.2.1	Definition	24

7.2.2	Information Elements	24

7.3	FallbackConfigurationDescriptor	26

7.3.1	Definition	26

7.3.2	Information Elements	26

7.4	TriggerConditionDescriptor	27

7.4.1	Definition	27

7.4.2	Information Elements	28

7.4.3	Data types	30

7.4.3.1	Hysteresis	30

7.5	ValidationJob	30

7.5.1	Definition	30

7.5.2	Information Elements	30

7.6	ActivationJob	32

7.6.1	Definition	32

7.6.2	Information Elements	33

Annex A (normative):  Solution sets	36

A.1	RESTful HTTP-based solution set	36

Annex B (informative): PlantUML for figures	40

Annex C (informative): Change history	42

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document …

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

[3]	IETF RFC 8072: " YANG Patch Media Type".

[4]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[5]	3GPP TS 32.161: "Management and orchestration; JSON expressions (Jex)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Concept and overview

# 5	Requirements

Requirements related to planned configurations

Req-PCG-1: The 3GPP management system shall support creating, reading and deleting planned configurations for managed systems.

Req-PCG-2: The 3GPP management system should support updating planned configurations for managed systems.

Req-PCG-3: The 3GPP management system shall support creating, reading, updating, and deleting meta data for planned configurations.

Requirements related to control parameters for validation and activation of planned configurations

Req-PCC-1: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer if validation and activation shall be processed in best effort or atomic mode.

Requirements related to planned configuration groups

Req-PGG-1: The 3GPP management system should allow MnS consumers to group planned configurations into planned configuration groups.

Req-PGG-2: The 3GPP management system should allow MnS consumers to group planned configuration groups into planned configuration groups.

Req-PGG-3: The 3GPP management system shall support creating, reading and deleting planned configuration groups.

Req-PGG-4: The 3GPP management system should support updating planned configuration groups.

Req-PGG-5: The 3GPP management system shall support creating, reading, updating, and deleting meta data for planned configuration groups.

Requirements related to control parameters for validation and activation of planned configuration groups

Req-PGC-1: The 3GPP management system shall support creating, reading, updating, and deleting control parameters for planned configuration groups.

General plan validation requirements

Req-PVG-1: The 3GPP management system shall support a capability allowing a MnS consumer to request a MnS producer to validate a planned configuration.

Req-PVG-2: The 3GPP management system should support a capability allowing a MnS consumer to retrieve the status of a plan validation.

Req-PVG-3: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to cancel an ongoing plan validation.

Req-PVG-4: The 3GPP management system shall support a capability allowing a MnS consumer to retrieve the result of a plan validation.

Req-PVG-5: The 3GPP management system should allow for multiple sequential validation of the same plan.

Req-PAG-6: The 3GPP management system shall support a capability allowing a MnS consumer to find out if a plan has been previously validated.

Requirements related to the validation of multiple plans

Req-PVM-1: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to validate multiple planned configurations with a single request.

Req-PVM-2: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the type of relationship between the plans, for the case where multiple plans are requested to be validated with a single request.

Req-PVM-3: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the behavior if plan conflicts are detected, for the case where multiple plans are requested to be validated with a single request.

General plan activation requirements

Req-PAG-1: The 3GPP management system shall support a capability allowing a MnS consumer to request a MnS producer to activate a planned configuration.

Req-PAG-2: The 3GPP management system should support a capability allowing a MnS consumer to retrieve the status of a plan activation.

Req-PAG-3: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to cancel an ongoing plan activation.

Req-PAG-4: The 3GPP management system shall support a capability allowing a MnS consumer to retrieve the result of a plan activation.

Req-PAG-5: The 3GPP management system should allow for multiple sequential activations of the same plan.

Req-PAG-6: The 3GPP management system shall support a capability allowing a MnS consumer to find out if a plan has been previously activated.

Req-PAG-7: The 3GPP management system shall validate a planned configuration automatically when activation of the planned configuration is requested without prior validation.

Requirements related to control parameters for activation

Req-PAC-1: The 3GPP management system should support a capability allowing a MnS consumer to indicate to a MnS producer if activation should be done with the least possible service impact or within the shortest possible time.

Req-PAC-2: The 3GPP management system should support a capability allowing a MnS consumer to indicate to a MnS producer if fallback shall be enabled.

Requirements related to the activation of multiple plans

Req-PAM-1: The 3GPP management system should support a capability allowing a MnS consumer to request a MnS producer to activate multiple planned configurations with a single request.

Req-PAM-2: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the type of relationship between the plans, for the case where multiple plans are requested to be activated with a single request.

Req-PAM-3: The 3GPP management system shall support a capability allowing a MnS consumer to indicate to a MnS producer the behavior if plan conflicts are detected, for the case where multiple plans are requested to be activated with a single request.

Requirements related to conditional activation of plans

Req-CAP-1: The 3GPP management system should allow MnS consumers to create conditions on MnS producers that specify when planned configurations are activated.

# 6	Solution description

Editor's note: Clarify that annotations are only for the user and are not processed by the MnS producer.

## 6.1	Planned configurations

### 6.1.1	Definition

The current configuration is the configuration currently used by the managed system. It is represented by (a tree of) configuration data nodes. State data nodes are not included. The format of the configuration data node tree is defined by (stage 2) NRM definitions and (stage 3) schema definitions.

Note that the data node tree representing a manged system typically comprises besides configuration data nodes also (read-only) state data nodes, that represent the current state of the managed system. Configuration data nodes and state data nodes may be separated in dedicated subtrees, but they may be also contained in one tree.

A planned configuration is a configuration that can be manipulated without impacting the current configuration of the system. It relates to configuration data nodes and not to state data nodes. It is maintained side-by-side with the current configuration.

### 6.1.2	Format of planned configurations

A planned configuration is represented by a set of operations to be applied to the current configuration. The operations are described by the input parameters of the "changeMOIs" operation specified in TS 28.532 [4], with the addition that each operation in the operation set has a key for identifying the operation.

The following guidelines may ease the processing of operation sets during validation and activation:

- Parent objects should be created before child objects.

-	Objects that are referenced should be created before the reference.

### 6.1.3	Meta data for planned configurations

Each planned configuration is described by meta data. This includes the name, the version, and a human readable textual description. It is also possible to specify additional MnS consumer defined properties (key value pairs) to further describe and qualify the planned configuration, for example to specify who created the planned configuration. The aforementioned annotations are for usage by a (human) MnS consumer only. The are not processed by the MnS producer.

Editor's note: Clarify that "version" may help the consumer to realize a version management system (use multiple plans with different version numbers).

Furthermore, it includes the content type of the planned configuration and a pointer to the current configuration. The pointer allows for deployments where the planned configuration is on another MnS producer than the current configuration.

Furthermore, it is specified if the operations of the operations set are an atomic set, where all operations need to be validated and activated successfully, or if some operations are allowed to fail (non-atomic or best effort set). In other words, for atomic sets, all operations of the set must be processed successfully or no operation at all. Operations, that are already applied when an error occurs, must be rolled back. For best effort sets, the operations that can be processed successfully are processed, and those that for whatever reason cannot be processed successfully are not processed. In this mode the validation or activation process continues on the occurrence of an error. In a third mode the validation or activation process stops on the occurrence of an error.

For example, subscriptions to alarm notifications on multiple Network Functions may have value also in case the subscription on some Network Functions fails. When analytics are computed based on measurements collected at some well selected Network Functions, then all measurement collection jobs must be created. If one job cannot be created it does not make sense to collect the remaining measurements.

The date and time at which information in the planned configuration was modified the last time is provided as well.

### 6.1.4	Planned configuration descriptor

The operation set and related meta data are specified in a planned configuration descriptor. Each descriptor has a unique identifier.

## 6.2	Planned configuration groups

### 6.2.1	Definition

A planned configuration group is a set of planned configurations or planned configuration groups that shall be processed (validated and activated) together.

For example, one plan may create a new cell, and another plan may start the collection of measurements for the new cell, or one plan may configure one base station and another plan may configure another base station, when both base stations need to be consistently configured to enable handovers between them.

### 6.2.2	Planned configuration group members

A planned configuration (or planned configuration group) belonging to a planned configuration group is also called a member of that planned configuration group. A planned configuration (or planned configuration group) can be a member of more than one planned configuration group.

The members of a planned configuration group shall be considered as atomic or non-atomic (best effort), in the same way as the operations of an operation set may form an atomic or a non-atomic (best effort) set.

Furthermore, the members of a planned configuration group can be ordered or not ordered. If ordered, the planned configuration group members are processed in the order as specified in the planned configuration group. If not ordered, the members can be processed in any order or in parallel.

For example, when the configuration for a new Network Function is in one plan and the creation of a performance metric collection job in another plan, then the plans must be ordered (assuming the job object is contained under a configuration object).

Not ordered are for example plans adding performance metric collection jobs to already configured Network Functions.

There shall not be any circular membership relationships between two or more planned configuration groups. E.g. group-A is a member of group-B then group-B is not allowed to be a member of group-A.

Figure 6.2.2-1: NOT allowed circular planed configuration group membership relation (with 3 groups)

A planned configuration group may not include any individual planned configuration more than once.The following is not allowed because Group_A cannot contain Plan2 twice, It is also invalid because Group_B contains Plan1 twice, once via Group_C and once via Group_A.

Figure 6.2.2-2: NOT allowed multiple containment of planed configuration

All member planned configurations shall have the same configuration content type.

### 6.2.3	Conflicts between planned configuration group members

Conflicting operations may be contained in planned configuration group members. For example, a planned configuration may set an attribute to some value and another planned configuration sets the same attribute to some other value.

If the configuration group members are ordered, the result is deterministic. The conflicting operations are processed in the order as they appear in the ordered plans. The last processed operation from the ordered set of operations manipulating the same node determines the result. If the configuration group members are not ordered, the result is not deterministic. Depending on which operation from the unordered set of operations manipulating the same node is processed last, the result may be different.

It is configurable if a detected conflict should invalidate the planned configuration group.

### 6.2.4	Meta data for planned configuration groups

Each planned configuration group is described by meta data. This includes annotations such as the name, the version, and a human readable textual description.

Furthermore, it includes the information if the group members are atomic or non-atomic, if they are ordered or unordered, and if a conflict between planned configuration group members shall invalidate the planned configuration group and prevent its activation.

The date and time at which information in the planned configuration group was modified the last time is provided as well.

### 6.2.5	Planned configuration group descriptor

The planned configuration group members and its meta data are specified in a planned configuration group descriptor. Each descriptor has a unique identifier.

## 6.3	Fallback configurations

### 6.3.1	Definition

Fallback is the process of returning to the configuration that was used by the managed system before the activation of a planned configuration or planned configuration group. A fallback configuration is the configuration describing this configuration.

Fallback configurations are generated by the system. Fallback may be applied when the activation of a planned configuration or a planned configuration group does not yield the desired results. Use cases include:

- A wrong planned configuration was activated. The operator wants to return to the old configuration.

- The new configuration does not perform as expected. The operator or the automation system wants to return to the old configuration.

- The new configuration was deployed on existing Network Functions in anticipation of some new Network Functions being rolled out. The roll out does not happen. The existing Network Functions are rolled back to the configuration state at the time the activation was triggered.

Fallback may be implemented by one of the following methods:

-	"undo operations": This method only reverses the changes resulting from the activation of a planned configuration or planned configuration group. It has no effect on configuration data nodes that are not affected by the activation of the planned configuration or planned configuration group. Therefore, changes to other data nodes, that occur after activation and before fallback, are not undone.

-	"restore complete configuration": This method reverts all configuration changes done after the activation of a planned configuration or planned configuration group, i.e. those resulting from the activation of a planned configuration or planned configuration group and those done via some other means. This method restores the complete old configuration. Therefore, changes to data nodes, that occur after activation and before fallback, are undone as well. Note that the complete data node tree is restored in this mode. Therefore, this mode might not be supported in deployment scenarios with large data node trees.

### 6.3.2	Fallback configuration preparation

A fallback configuration is created by the system upon the start of an activation process when enable fallback is requested. The format of the fallback configuration is dependent on the fallback method.

In case of "undo operations" the fallback configuration shall be visible to the consumer as a read-only planned configuration. A fallback configuration is always a single configuration (not a planned configuration group) and is atomic. For example, for a planned configuration that adds a new object to the current data node tree the fallback configuration simply removes this object. Note that the fallback operations are not always simply the opposite of the operations contained in the activated planned configuration. The fallback configuration is created on effective changes only, for example:

- Only successful operations shall be included in the fallback configuration.

- For removed objects, the fallback configuration needs to include the representation of the object in the current data node tree at the time of removal.

- The removal of objects that do not exist cannot result in the addition of an object, leave alone that there would be no representation for that object.

- The addition of an object that already exists shall not result in the removal of that object.

Editor's note: For "changeMOIs" it is not specified if the addition (operation "add") of an object or attribute, that already exists, is an error. Furthermore, it is not specified if the removal (operation "remove") of an object, that does not exist, is an error. This may have to be clarified. Maybe it is also dependent on the protocol.

In case of "restore configuration" the fallback configuration is implementation specific.

### 6.3.3	Meta data for fallback configurations

Each fallback configuration is described by meta data. This includes annotations such as the name, the version, and a human readable textual description.

Furthermore, an identifier of the activation process whose result is to be rolled back must be provided.

A fallback is always atomic.

### 6.3.4	Fallback configuration descriptor

Editor's note: This descriptor is ffs. A fallback configuration has the following differences compared to a normal planned configuration:

- apply mode is always atomic (plan descriptor)

- must include the id of the activation job (plan descriptor)

- planConfigContentType is set by the MnS producer and is read-only (plan descriptor)

- currentConfigAddress is set by the MnS producer and is read-only (plan descriptor)

- cannot be a member of a planned configuration group (group descriptor)

- cannot be activated by trigger conditions (trigger conditions)

- restore flavour cannot be validated (validation)

- no fallback is possible (activation)

- no cancelling is possible (activation)

- a pointer to the fallback configuration descriptor shall be included in the related activation job (activation)

## 6.4	Trigger conditions

### 6.4.1	Definition

A trigger condition is a set of definitions controlling the points in time when associated planned configurations and planned configuration groups are activated. The set of definitions typically includes expressions on data node trees that evaluate to true or false. Trigger conditions are evaluated permanently.

### 6.4.2	Usage

Trigger conditions are provided for conditional activation of planned configurations and planned configuration groups. The planned configuration is activated automatically when the provided conditions are met.

For example, additional performance metric collection jobs or assurance functions shall be created when metrics that are always collected cross certain thresholds.

### 6.4.3	Specification of trigger conditions

The trigger for activation is specified as follows:

- a condition expression evaluating to true or false.

- an evaluation period.

- a hysteresis that may require the evaluation to yield true for a specified time or a specified number of times before the trigger is activated.

The condition expression works on the data node tree of the current configuration. The notation used to express the condition depends on the MnS protocol used. The transition of the evaluation result of the trigger condition from false to true is considered as a trigger. The transition from true to false is not considered as a trigger.

The evaluation period determines how often the trigger condition is evaluated.

The hysteresis may be used to ensure that the trigger is activated only when the trigger condition evaluates to true for a certain time or a certain number of times. This prevents the trigger from being activated also when the evaluation result toggles between false and true.

Editor's note: Time based triggers are ffs.

Editor's note: It is ffs if a hysteresis is not for the first triggering but only for subsequent triggers.

### 6.4.4	Meta data for trigger conditions

Each trigger condition is described by meta data. This includes annotations such as the name, the version, and a human readable textual description.

### 6.4.5	Control parameters for trigger conditions

The trigger may be activated each time the result of the trigger condition evaluation changes from false to true or only once. A control parameter allows to configure this behavior.

### 6.4.6	Monitor parameters for trigger conditions

Monitor parameters are provided that display the current result of the evaluation, the data and time at which the trigger was last activated, and a Boolean indicating if the trigger is inactive for the case that only one activation is requested.

### 6.4.7	Trigger condition descriptor

The trigger condition descriptor includes the specification of the trigger condition itself, and associated meta-data, control parameters and monitor parameters.

## 6.5	Validation of planned configurations

### 6.5.1	Definition

Planned configurations may have problems and it may be impossible to activate them. Validation is the process to unveil potential problems with planned configurations before activation of the planned configuration.

The validation function has two input arguments: the operation set representing the planned configuration and a snapshot of the current configuration. The planned configuration is applied to the snapshot of the current configuration. The resulting configuration data node tree must comply to the (stage 2) NRM definitions and (stage 3) NRM schema definitions.

### 6.5.2	Potential problems

Validation discovers problems of planned configurations. These problems may be classified as follows:

- Information model problems

-	Problems of the planned configuration itself

-	Problems of the planned configuration in relationship to the current configuration

- Application layer problems

Examples for problems of the planned configuration itself:

- Update of an attribute with a value that does not match the data type defined for the value of the attribute

- Creation of an attribute name value pair that is not defined

- Update of an existing attribute that is defined as read-only

- Creation of an object with an object class that is not defined

- Creation of an object with a representation does not match the object class defined for the object

- Creation of an object under an object whose class does not support this containment

Examples for problems of the planned configuration in relationship to the current configuration:

- Creation of objects under parents that do not exist

- Deletion of leaf objects that do not exist

- Deletion of objects that are no leaves

- Violation of the cardinality property of an object

- Violation of the multiplicity property of an attribute

Examples for application layer problems

-	An attribute value in the planned configuration is in the value range allowed by the attribute properties but the value is not allowed in the current state of the application.

### 6.5.3	Validation process

The purpose of validation is to detect problems of the planned configuration. As described in the previous clause, problems can be categorized into three groups.

Problems of the operations itself are detected by analyzing the operations together with the NRM schema. The current configuration is not involved in this process. Problematic operations are always invalid. If an atomic operation set contains at least one invalid operation, the complete planned configuration is invalid. If a non-atomic operation set contains a bad operation, only the operation is invalid. The operation set with the other operations may still be valid, dep ending on the validation result for the other operations.

Problems of the planned configuration in relationship to the current configuration are detected by applying the operations of the operation set one by one to a snapshot of the current configuration. This may result in intermediate states that do not comply to the constraints defined by the NRM schema. For example, an object of class A shall contain always exactly five objects of class B. To replace an object of class B, the object to be replaced may be deleted in the first step. In the second step the new object is added. The other possibility is to add the new object first and delete the object to be removed afterwards. In both cases the cardinality has a temporary invalid state. The validation process needs to take this into account: Problems of the planned configuration in relationship to the current configuration may be temporary only and be healed later when more operations are processed. A final assessment on the validity of operations can be made only after processing all operations.

In addition to problems of the planned configuration in relationship to the current configuration, problems of the planned configuration at application layer may occur. These problems are detected by evaluating the updates specified by the operations in the context of the configuration that is produced by applying the planned configuration against the current configuration. As for problems of the planned configuration in relationship to the current configuration, a final assessment on the validity of operations can be made only after processing all operations.

In the atomic mode for validation all operations must be valid for the planned configuration to be valid. If only one operation is invalid the complete planned configuration is invalid. The former case is also referred to as a successful validation and the latter as a failed validation.

In the best effort mode for validation all operations must be valid for the planned configuration to be valid. If at least one operation is invalid (but not all), the planned configuration is partially valid (or partially invalid). If all operations are invalid the planned configuration is invalid. These three cases are also referred to as successful validation, partially successful validation (or partially failed validation), and failed validation.

All constraint violations shall be reported.

A successful validation does not guarantee a successful activation.

### 6.5.4	Validation of planned configuration groups

The planned configuration group members are validated in the order specified. If no order is specified, they can be validated in any order. Each member is validated as specified in this and the previous clause. The validation of atomic members concludes with either validated or failed. The validation of best effort members concludes with either validated, partially validated or failed.

An atomic group is considered as valid, if atomic members are valid, and best effort members are valid or partially valid. If the validation of one (atomic or best effort) member fails, the validation of the complete group shall fail.

A best effort group is considered as valid, if atomic members are valid, and best effort members are valid or partially valid. If the validation of at least one (atomic or best effort) member fails, the group is considered as partially valid. If the validation of all members fails, the validation of the complete group shall fail.

### 6.5.5	Validation of fallback configurations

When the fallback configuration is the complete old configuration, the complete current configuration is replaced when activating the fallback configuration. In this case the concept of validation has no meaning. The complete old configuration is always valid. Therefore, the fallback configuration should not be validated when the fallback configuration is represented by the complete old configuration.

When the fallback configuration is a set of undo operations, the fallback configuration should be validated like a planned configuration prior to activation.

### 6.5.6	Validation control parameters

Control parameters for validation are not specified.

Editor's note: Add "cancelRequest". A cancellation request stops an ongoing validation process. The state is set to "NOT_VALIDATED".

## 6.6	Activation of planned configurations

### 6.6.1	Definition

Activation is the process of applying a planned configuration to the managed system.

### 6.6.2	Potential problems

A planned configuration is validated before activation. Therefore a planned configuration that is activated should not have any of the problems that are discovered by validation. Only valid or partially valid planned configurations or planned configuration groups should be activated. Therefore, other types of problems are encountered during activation. These include

- A Network Function cannot be updated, because it is not reachable.

- A Network Function cannot be updated, because it is locked.

- A Network Function cannot be updated, because it does not exist.

- A Network Function cannot be updated, because it lacks necessary resources.

- A Network Function cannot be updated, because necessary access rights are not in place.

### 6.6.3	Activation process

Activation is the process of applying the updates specified by the operations contained in a planned configuration to the managed system. After the managed system is updated, the updates are reflected in the current configuration. Note that the system may not apply the operations to the managed system exactly as specified. For example, to change the configuration of a function, the function may have to be locked. Locking operations may not be included in the planned configuration and therefore the configuration update operation is augmented with locking and unlocking operations.

In atomic mode, the system may have to roll back to the current configuration that was effective when the activation process started. Therefore, the system needs to take and store a (conceptual) snapshot of the impacted part of the current configuration upon starting the activation process.

If not in atomic mode, the activation process may stop or continue when the first error occurs.

The activation process may run for quite some time, for example when large configurations are downloaded to the Network Functions as part of the activation process.

The planned configuration to be activated should be validated first.

Problems related to the activation of a planned configuration shall be reported.

### 6.6.4	Activation of planned configuration groups

The planned configuration group members are activated in the order specified. If no order is specified, they can be activated in any order. Each member is activated as specified in the previous clause. The activation of atomic members concludes with either activated or failed. The activation of best effort members concludes with either activated, partially activated or failed.

An atomic group is considered as activated, if atomic members can be activated, and best effort members can be activated or partially activated. If the activation of one (atomic or best effort) member fails, the activation of the complete group shall fail (and all updates must be undone).

A best effort group is considered as activated, if atomic members can be activated, and best effort members can be activated or partially activated. If the activation of at least one (atomic or best effort) member fails, the group is considered as partly activated. If the activation of all members fails, the activation of the complete group shall fail (and all updates must be undone).

### 6.6.5	Activation control parameters

Editor's note: Explain "enable-fallback", "service-impact and "immediate-activation". Also "cancel-request".

## 6.7	Conditional activation of planned configurations

## 6.8	Asynchronous configuration updates

Updating the current configuration may take some time, more time than an update operation based on a synchronous request response pattern, where the request provides the desired updates and the response information about the outcome of the requested updates, may be able to cope with. In a synchronous request-response pattern, the response is sent shortly after reception of the request.

This problem is addressed by so-called asynchronous update operations, where the request response pattern just provides the desired updates to the MnS producer. The response does not contain information about the outcome but a confirmation of reception of the desired updates. The information about the outcome of the update operations is provided asynchronously, i.e. later when the information is available.

Management of planned configuration includes as a special case also an asynchronous update operation. It is not mandatory to create a planned configuration before validation or activation. A new configuration can be provided also together with the validation or activation request. This is conceptually equivalent to an asynchronous update operation.

## 6.9	Fallback

## 6.10	Planned configuration lifecycle

## 6.11	Managing planned configurations

Planned configuration descriptors are created, updated and deleted by MnS consumers on MnS producers.

Multiple planned configuration descriptors can be maintained on a MnS producer. Each planned configuration is identified by an identifier. The identifier is assigned by the MnS producer.

Planned configuration group descriptors are created, updated and deleted by MnS consumers on MnS producers. The planned configurations and planned configuration groups referred to as members in a planned configuration group shall exist.

Multiple planned configuration group descriptors can be maintained on a MnS producer. Each planned configuration group is identified by an identifier. The identifier is assigned by the MnS producer.

## 6.12	Managing planned configuration groups

Planned configuration group descriptors are created, updated and deleted by MnS consumers on MnS producers. The planned configurations referred to in a planned configuration group shall exist.

Multiple planned configuration group descriptors can be maintained on a MnS producer. Each planned configuration group is identified by an identifier. The identifier is assigned by the MnS producer.

## 6.13	Managing fallback configurations

Fallback configuration descriptors are not directly managed by MnS consumers. They are created automatically by MnS producers when the "enableFallback" attribute in an activation job is set to true. Only the "name", "version", and "description" can be set by MnS consumers once the object is created.

When many fallbacks are enabled, the situation can become soon complex and the risk for misconfigurations increases. MnS producers may therefore apply implementation specific strategies to reduce the risk for misconfigurations. For example, they may allow only for one fallback configuration to exist; if another fallback is enabled, the existing fallback configuration is overwritten. Or they may allow for multiple fallback configurations only if they affect different parts of the network.

Fallback configurations can be deleted by MnS consumers, or by the MnS producer based on an implementation specific policies.

No fallback on the activation of a fallback configuration is possible.

## 6.14	Managing trigger conditions

Conditions are created, updated and deleted by MnS consumers on MnS producers. For each condition shall be specified which activation jobs are triggered by this condition.

## 6.15	Managing validations

Validation of a planned configuration is requested by MnS consumers and performed by MnS producers. To request validation the MnS consumer needs to create a validation job on the MnS producer. Each job is identified by an identifier. The identifier is assigned by the MnS producer.

The planned configuration to be validated is specified by including the identifier of the corresponding planned configuration descriptor in the job creation request. Alternatively, for validating a planned configuration group, a planned configuration group descriptor can be specified.

In case no planned configuration descriptor or no planned configuration group descriptor has been created for the planned configurations to be validated, a descriptor may be specified directly in the validation job creation request.

When only one planned configuration is requested to be validated no control parameter needs to be specified in the request. For validating planned configuration groups, the "isFailOnConflict" parameter needs to be specified.

The MnS producer adds the following information elements to the job: "state", "startedAt", "stoppedAt", "result", "memberConflicts", and "errors".

The information element "errors" is a structured data type with the following properties:

-	The mandatory "type" property that provides high level error information.

-	The optional "title" that provides a short, human-readable summary of the problem type. It shall not change from occurrence to occurrence of the problem.

-	The optional "reason" property" that provides more details on the error conditions than the "type" property.

-	The mandatory "badOp" property that specifies the operation, that cannot be satisfied.

Further details can be found in TS 32.158 [2].

Multiple validation jobs can run in parallel, even if the planned configurations target the same portions of the current configuration.

Editor's note: Clarify that the validation result is valid only when it is produced. Wheen the current configuration changes or the plan is updated the result is outdated.

Editor's note: Clarify that planned configurations and planned configuration groups should be locked during validation.

## 6.16	Managing activations

Activation of a planned configuration is requested by MnS consumers and performed by MnS producers. To request activation the MnS consumer needs to create an activation job on the MnS producer. Each job is identified by an identifier. The identifier is assigned by the MnS producer.

The planned configuration to be activated is specified by including the identifier of the corresponding planned configuration descriptor in the job creation request. Alternatively, for validating a planned configuration group, a planned configuration group descriptor can be specified.

In case no planned configuration descriptor or no planned configuration group descriptor has been created for the planned configurations to be activated, a descriptor may be specified also directly in the activation request.

For falling back to an old configuration, the identifier of a fallback configuration descriptor needs to be specified.

Activation shall start immediately upon creation of the activation job, unless the "isImmediateActivation" parameter is set to false. In this case the activation is triggered by a trigger condition that points to the activation job. Note that the activation job does not know the related conditions.

For immediate activation an activation job can activate a planned configuration only once. If the planned configuration is to be activated a second time a new job needs to be created. For conditional activation, in contrast, an activation job can activate the same planned configuration multiple times. For example, if a planned configuration is for weekdays and another planned configuration is for weekends, two activation jobs can be used for activating both planned configurations. One job is triggered by a trigger condition triggering the activation when the weekend starts, the other job is triggered by a trigger condition when the weekdays start.

Multiple activation jobs running can run in parallel if the planned configurations do not update the same portions of the current configuration.

Furthermore, the following control parameters need to be specified in the job creation request: "isEnableFallback", and "serviceImpact".

The MnS producer adds the following information elements to the job: "state", "startedAt", "stoppedAt", "result", and "errors".

Only planned configurations, that are successfully validated, should be activated. Therefor a MnS consumer should always create a validation job first. Only after successful validation of a planned configuration, a job for activating the planned configuration should be created. In case the planned configuration was not validated the MnS producer shall create a validation job for the planned configuration. The activation is started only if the planned configuration is valid. If the planned configuration is invalid the activation is not started and the result attribute of the activation indicates that the activation failed.

A MnS consumer can request a MnS producer to cancel a running activation process. The behavior depends on the application mode:

- For "ATOMIC" the configuration is rolled back.

- For "BEST_EFFORT" the activation is stopped.

- For "STOP_ON_ERROR" the activation is stopped

Editor's note: Clarify that planned configurations and planned configuration groups should be locked during activation.

## 6.17	Notifications

# 7	Information model

Editor's note: Add statements about uniqueness of ids.

## 7.1	PlannedConfigurationDescriptor

### 7.1.1	Definition

This definition represents a planned configuration descriptor.

### 7.1.2	Information Elements

The following table specifies the information elements of a planned configuration descriptor.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| id | M | The identifier of the planned configuration. | type: String multiplicity: 1 isInvariant: True isWritable: False |
| name | M | The name of the planned configuration. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| version | M | The version of the planned configuration. Its format is implementation specific. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| description | M | The textual human-readable description of the planned configuration. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| customProperties | M | The container allowing to specify additional consumer defined properties (key value pairs) describing and qualifying the planned configuration. | type: String multiplicity: * isOrdered: False isUnique: True isInvariant: False isWritable: True |
| planConfigContentType | M | The format of the planned configuration. | type: String multiplicity: 1 isInvariant: True isWritable: True |
| currentConfigAddress | O | The address of the current configuration. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| applyMode | M | Specifies if the operations in the operation set are treated as atomic, best effort, or if processing shall stop on the occurrence of the first error. allowedValues: - ATOMIC - BEST_EFFORT - STOP_ON_ERROR | type: ENUM multiplicity: 1 isInvariant: False isWritable: Truei |
| lastModifiedAt | M | The date and time at which the planned configuration descriptor was modified the last time by a MnS consumer. Upon creation of the planned configuration descriptor the value of the information element is set to the date and time at which the descriptor is created. | type: DateTime multiplicity: 1 isInvariant: False isWritable: False |
| planConfig | M | The operation set specifying the planned configuration. The operations are described by the input parameters of the "changeMOIs" operation specified in TS 28.532 [4]. The node to be modified is identified by the concatenation of the "currentConfigAddress" (if available) and the "path" component in "planConfig". Editor's note: The exact format is still ffs. | type: String multiplicity: 1 isInvariant: False isWritable: True |
| validationState | M | The outcome of the last validation of the planned configuration, or "NOT_VALIDATED" if the planned configuration has not been validated yet. The state is reset to "NOT_VALIDATED", if in another state, when "planConfig" is updated. allowedValues: - NOT_VALIDATED - VALID - PARTIALLY_VALID - INVALID | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |
| lastValidatedAt | M | The date and time at which the planned configuration was validated (the last time). The information element is absent, when the planned configuration has not been validated yet. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |

## 7.2	PlannedConfigurationGroupDescriptor

### 7.2.1	Definition

This definition represents a planned configuration group descriptor.

### 7.2.2	Information Elements

The following table specifies the information elements of a planned configuration group descriptor.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| id | M | The identifier of the planned configuration group descriptor | type: String multiplicity: 1 isInvariant: True isWritable: False |
| name | M | The name of the planned configuration group. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| version | M | The version of the planned configuration group. Its format is implementation specific. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| description | M | The textual human-readable description of the planned configuration group. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| customProperties | O | The container allowing to specify additional consumer defined properties (key value pairs) describing and qualifying the planned configuration. | type: String multiplicity: * isOrdered: False isUnique: True isInvariant: False isWritable: True |
| members | M | The identifiers of planned configurations or the planned configuration groups that are members of this planned configuration group. | type: String multiplicity: 1..* isOrdered: True isUnique: True isInvariant: False isWritable: True |
| isOrdered | M | Specifies if the members of the planned configuration group are ordered. When ordered, the planned configuration group members shall be validated/activated in the specified order. When not ordered the planned configuration group members can be validated/activated in any order. | type: Boolean multiplicity: 1 isInvariant: False isWritable: True |
| applyMode | M | Specifies if the members of the planned configuration group are treated as atomic, best effort, or if processing shall stop on the occurrence of the first error. allowedValues: - ATOMIC - BEST_EFFORT - STOP_ON_ERROR | type: ENUM multiplicity: 1 isInvariant: False isWritable: True |
| isFailOnConflict | M | Specifies if the activation shall fail on detection of conflicts, or if the operations shall be processed as if there were no conflicts. | type: Boolean multiplicity: 1 isInvariant: False isWritable: True |
| lastModifiedAt | M | The date and time at which the planned configuration group descriptor was modified the last time by a MnS consumer. Upon creation of the planned configuration group descriptor the value of the information element is set to the date and time at which the descriptor is created. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| validationState | M | The outcome of the last validation, or "NOT_VALIDATED" if the planned configuration group has not been validated yet. The state is reset to "NOT_VALIDATED", if in another state, when "members" is updated. allowedValues: - NOT_VALIDATED - VALID - PARTIALLY_VALID - INVALID | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |
| lastAalidatedAt | M | The date and time at which the planned configuration group was validated (the last time). The information element is absent, when the planned configuration has not been validated yet. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |

## 7.3	FallbackConfigurationDescriptor

### 7.3.1	Definition

This definition represents a fallback configuration descriptor.

### 7.3.2	Information Elements

The following table specifies the information elements of a fallback configuration descriptor.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| id | M | The identifier of the fallback configuration descriptor | type: String multiplicity: 1 isInvariant: True isWritable: False |
| name | M | The name of the fallback configuration. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| version | M | The version of the fallback configuration. Its format is implementation specific. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| description | M | The textual human-readable description of the fallback configuration. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| customProperties | M | The container allowing to specify additional consumer defined properties (key value pairs) describing and qualifying the planned configuration. | type: String multiplicity: * isOrdered: False isUnique: True isInvariant: False isWritable: True |
| ctivationJob | M | The identifier of the related activation job. | type: String multiplicity: 1 isInvariant: True isWritable: False |
| configurationContentType | M | The format of the fallback configuration. | type: String multiplicity: 1 isInvariant: True isWritable: False |
| fallback-config | M | The fallback configuration. | type: String multiplicity: 1 isInvariant: True isWritable: False |

## 7.4	TriggerConditionDescriptor

### 7.4.1	Definition

This definition represents a trigger condition descriptor.

### 7.4.2	Information Elements

The following table specifies the information elements of a trigger condition descriptor.

| Information element name |  | S | Documentation and Allowed Values |  |  | Properties |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| id |  | M | The identifier of the trigger condition descriptor. |  |  | type: String multiplicity: 1 isInvariant: True isWritable: False |  |  |
| name | M |  |  | The name of the trigger condition. |  |  | type: String multiplicity: 0..1 isInvariant: False isWritable: True |  |
| version | M |  |  | The version of the planned configuration. Its format is implementation specific. |  |  | type: String multiplicity: 0..1 isInvariant: False isWritable: True |  |
| description |  | M | The textual human-readable description of the trigger condition. |  |  | type: String multiplicity: 0..1 isInvariant: False isWritable: True |  |  |
| customProperties |  | M | The container allowing to specify additional consumer defined properties (key value pairs) describing and qualifying the planned configuration. |  |  | type: String multiplicity: * isOrdered: False isUnique: True isInvariant: False isWritable: True |  |  |
| conditionExpression |  | M | The condition expression. |  |  | type: String multiplicity: 1 isInvariant: False isWritable: True |  |  |
| evaluationPeriod |  | M | The evaluation period specifies the interval of time between two consecutive condition expression evaluations. |  |  | type: Integer multiplicity: 1 isInvariant: False isWritable: True |  |  |
| hysteresis |  | M | The hysteresis, when present, specifies that the trigger shall not be activated immediately, when the evaluation result changes from false to true, but only when the evaluation results is true for a specified time (which must be a multiple of the evaluation period) or a specified number of times. |  |  | type: Hysteresis multiplicity: 0..1 isInvariant: False isWritable: True |  |  |
| isTriggerOnce |  | M | The boolean indication, if the trigger is disarmed after the first firing. |  |  | type: Boolean multiplicity: 1 isInvariant: False isWritable: True |  |  |
| activationJobs |  | M | The identifiers of one or more activation jobs that shall be triggered by this condition. |  |  | type: String multiplicity: * isOrdered: False isUnique: True isInvariant: False isWritable: True |  |  |
| startEvaluationAt |  | M | The date and time at which the evaluation of the condition expression shall start. The evaluation result is set to "False" before that date and time. If the information element is not specified, evaluation of the trigger condition shall start immediately. |  |  | type: DateTime multiplicity: 0..1 isInvariant: True isWritable: True |  |  |
| stopEvaluationAt |  | M | The date and time at which the evaluation of the condition expression shall stop. The evaluation result is set to "False" after that date and time. If the information element is not specified, evaluation of the trigger condition shall continue until the deletion of the trigger condition descriptor. |  |  | type: DateTime multiplicity: 0..1 isInvariant: True isWritable: True |  |  |
| lastModifiedAt |  | M |  |  | The date and time at which the trigger condition was modified the last time by a MnS consumer. Upon creation of the trigger condition descriptor the value of the information element is set to the date and time at which the descriptor is created. |  |  | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| currentEvaluationResult |  | M | The current result of evaluating the "condition-expression". |  |  | type: Boolean multiplicity: 1 isInvariant: False isWritable: False |  |  |
| lastTriggeredAt |  | M | Th date and time at which the evaluation result of the trigger condition changed the last time from "False" to "True". |  |  | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |  |  |
| isTriggerActive |  | M | The indication if the trigger can start activation jobs (trigger is active), or if the trigger cannot start activation jobs (trigger is inactive). |  |  | type: Boolean multiplicity: 1 isInvariant: False isWritable: False |  |  |

### 7.4.3	Data types

#### 7.4.3.1	Hysteresis

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| timeOfTrueEvaluations | M | The hysteresis, when present, specifies that the trigger shall not be activated immediately, when the evaluation result changes from false to true, but only when the evaluation results is true for a specified time (which must be a multiple of the evaluation period). Unit is seconds | type: Integer multiplicity: 0..1 isInvariant: False isWritable: True |
| numberOfTrueEvaluations | M | This information element, when present, specifies that the trigger shall not be activated immediately, when the evaluation result changes from false to true, but only when the evaluation results is true for a specified time (which must be a multiple of the evaluation period) or a specified number of times. | type: Integer multiplicity: 0..1 isInvariant: False isWritable: True |

## 7.5	ValidationJob

### 7.5.1	Definition

This definition represents a validation job.

### 7.5.2	Information Elements

The following table specifies the information elements of a validation job.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| id | M | The identifier of the validation job. | type: String multiplicity: 1 isInvariant: True isWritable: False |
| name | M | The name of the validation job. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| description | M | The textual human-readable description of the validation job. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| planConfigDescrId | M | The identifier of the planned configuration descriptor, whose operation set is requested to be validated. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigGroupDescrId | M | or, alternatively, the identifier of the planned configuration group descriptor, whose operation sets are requested to be validated. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigDescr | O | or, alternatively, the planned configuration descriptor to be activated, if no planned configuration descriptor was created earlier. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigDescrGroup | O | or, alternatively, the planned configuration group descriptor to be validated, if no planned configuration group descriptor was created earlier. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| state | M | The validation job state. allowedValues: - NOT_STARTED - RUNNING - CANCELLING - CANCELLED - COMPLETED - FAILED Editor's note: It is ffs if more detailed state and progress info needs to be added | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |
| startedAt | M | The date and time at which the validation process started, i.e. the time when the job state transition from "NOT_STARTED" to "RUNNING" occurred. In the "NOT_STARTED" state the information element is absent or carries no information. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| stoppedAt | M | The date and time at which the validation process stopped, i.e. the job state transition from "RUNNING" to "COMPLETED" or "FAILED", or from "CANCELLING" to "CANCELLED". In the "NOT_STARTED", "RUNNING" or "CANCELLING" state the information element is absent or carries no information. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| result | M | The validation result, either "VALID" or "INVALID". When the activated operation set is non-atomic or the activated planned configuration group is non-atomic, also "PARTIALLY_VALID". allowedValues: - VALID - PARTIALLY_VALID - INVALID | type: ENUM multiplicity: 0..1 isInvariant: False isWritable: False |
| memberConflicts | M | In case the validation process detects a conflict between the members of the planned configuration group to be validated, details about the conflict are specified here. | type: MemberConflict multiplicity: 0..* isInvariant: False isWritable: False |
| errors | M | In case validation failed or partially failed, error details are specified here.Editor's note: This parameter is ffs. It may have to be merged with "memberConflicts" and "result". | type: Error multiplicity: 0..1 isInvariant: False isWritable: False |

## 7.6	ActivationJob

### 7.6.1	Definition

This definition represents an activation job.

### 7.6.2	Information Elements

The following table specifies the information elements of an activation job.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| id | M | The identifier of the activation job. | type: String multiplicity: 1 isInvariant: True isWritable: False |
| name | M | The name of the activation job. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| description. | M | The textual human-readable description of the activation job. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| planConfigDescrId | M | The identifier of the planned configuration descriptor, whose operation set is requested to be activated. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigGroupDescrId | M | or, alternatively, the identifier of the planned configuration group descriptor, whose operation sets are requested to be activated. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| fallbackConfigDescrId | M | or, alternatively, the identifier of the fallback configuration descriptor, whose operation sets are requested to be activated. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigDescr | M | or, alternatively, the planned configuration descriptor to be activated, if no planned configuration descriptor was created earlier. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| planConfigGroupDescr | M | or, alternatively, the planned configuration group descriptor to be activated, if no planned configuration group descriptor was created earlier. | type: String multiplicity: 0..1 isInvariant: True isWritable: True |
| isEnableFallback | O | This boolean attribute allows to specify if the MnS producer shall create a plan with the current configuration to allow for a later fallback. Default value is "False", e.g. no fallback is enabled. The plan-id of the fallback plan shall be returned to the MnS consumer. | type: Boolean multiplicity: 1 isInvariant: True isWritable: True |
| serviceImpact | O | The service impact mode. allowedValues: - LEAST_SERVICE_IMPACT - SHORTEST_TIME | type: ENUM multiplicity: 1 isInvariant: True isWritable: True |
| isImmediateActivation | O | This Boolean attribute specifies if the activation job shall start immediately or, alternatively, by conditional activation. | type: Boolean multiplicity: 1 isInvariant: True isWritable: True |
| cancelRequest | O | This boolean attribute allows to request to cancel the activation process. | type: Boolean multiplicity: 1 isInvariant: False isWritable: True |
| state | M | The activation job state. allowedValues: - NOT_STARTED - RUNNING - CANCELLING - CANCELLED - COMPLETED - FAILED Editor's note: It is ffs if more detailed state and progress info needs to be added | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |
| startedAt | M | The date and time at which the activation process started, i.e. the time when the job state transition from "NOT_STARTED" to "RUNNING" occurred. In the "NOT_STARTED" state the information element is absent or carries no information. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: FalseType: DateTime multiplicity: 0..1 isOrdered: NA isUnique: NA |
| stoppedAt | M | The date and time at which the activation process stopped, i.e. the job state transition from "RUNNING" to "COMPLETED" or "FAILED", or from "CANCELLING" to "CANCELLED". In the "NOT_STARTED", "RUNNING" or "CANCELLING" state the information element is absent or carries no information. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |
| result | M | The activation result, either "ACTIVATED" or "ACTIVATION_FAILED". When the activated operation set is non-atomic or the activated planned configuration group is non-atomic, also "ACTIVATION_PARTIALLY_FAILED". allowedValues: - ACTIVATED - ACTIVATION_FAILED - ACTIVATION_FAILED_ROLLED_BACK - ACTIVATION_FAILED_ROLLBACK_FAILED - PARTIALLY_ACTIVATED | type: ENUM multiplicity: 0..1 isInvariant: False isWritable: False |
| errors | M | In case validation failed or partially failed, error reasons are specified here. Editor's note: This parameter is ffs. It may have to be merged with "result". | type: multiplicity: 0..1 isInvariant: False isWritable: False |

# Annex A (normative):
Solution sets

## A.1	RESTful HTTP-based solution set

Format of planned configurations

A planned configuration might target MnS producers whose data node tree is described using YANG or OpenAPI.

When described in OpenAPI, the value of the "configuration-content-type" is either "application/vnd.3gpp.json-patch+json" defined in TS 32.158 [2] or "application/json-patch+json".

When described in YANG, the value of the "configuration-content-type" is "application/yang-patch+json", defined in RFC 8072 [3].

A single planned configuration shall only include one configuration-content-type. If some items are modelled in YANG while others in OpenAPI these should be placed in separate planned configurations and made members of a plan configuration group.

Editor's note: Can the content types above be used given that a key is added for identifying an operation in the operation set? What is the content type for a restore configuration?

Editor's note: It is ffs if the key should be added only when we deal with YANG defined current configurations.

Note that JSON Patch is not used together with HTTP Patch. Therefore, the atomicity and synchronicity properties of HTTP Patch do not apply here.

CRUD operations

Editor's note: It is ffs if the CRUD operations defined in TS 32.158 are used or if dedicated CRUD operations are specified.

Format of condition expressions

Conditions expressions shall be expressed using Jex as defined in TS 32.161 [5].

Resource structure

The resource structure on the MnS producer is as follows:

…/{MnSName}/{/MnSVersion}/plan-descriptors/{id}

…/{MnSName}/{/MnSVersion}/plan-group-descriptors/{id}

…/{MnSName}/{/MnSVersion}/plan-validation-jobs/{id}

…/{MnSName}/{/MnSVersion}/plan-activation-jobs/{id}

Creating, updating and deleting planned configurations

The data node tree on the MnS producer is as follows upon system start up.

{ "mopc": { "plan-descriptors": {}, "plan-group-descriptors": {}, "fallback-config-descriptors": {}, "trigger-condition-descriptors": {}, "plan-validation-jobs": {}, "plan-activation-jobs": {} } }

A new item of the collection resource "plan-descriptors" is created by MnS consumers using HTTP POST.

POST 3gpp/ProvMnS/v1/plan-descriptors HTTP/1.1 Host: example.org Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "planConfigContentType": "application/vnd.3gpp.json-patch+json", "currentConfigAddress": "http://example.org/3gpp/ProvMnS/v1", "applyMode": "ATOMIC", "planConfig": [ { "key": "op1", "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": {1 "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] }

The MnS producer allocates the identifier "p1" for the new resource and returns the response. The location header contains the URI of the new resource. The response body contains the representation of the new resource which is equal to the representation received in the request with the "lastModifiedAt" and "validationState" properties added.

HTTP/1.1 201 Created Date: Wed, 21 Aug 2024 15:39:57 GMT Location: http://example.org/3gpp/ProvMnS/v1/plan-descriptors/p1 Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "planConfigContentType": "application/vnd.3gpp.json-patch+json", "currentConfigAddress": "http://example.org/3gpp/ProvMnS/v1", "applyMode": "ATOMIC", "lastModifiedAt": " 2025-03-06T16:50:26-08:00 ", "validationState": "NOT_VALIDATED", "planConfig": [ { "key": "op1", "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] }

The resource structure on the MnS producer contains the new resource.

{ "plan-descriptors": { "p1": { "name": "NewBts10Plan", "version": "2.0", "description": "This is the plan for the new BTS 10.", "planConfigContentType": "application/vnd.3gpp.json-patch+json", "currentConfigAddress": "http://example.org/3gpp/ProvMnS/v1", "applyMode": "ATOMIC", "lastModifiedAt": " 2025-03-06T16:50:26-08:00 ", "validationState": "NOT_VALIDATED", "planConfig": [ { "key": "op1", "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] } }, "plan-group-descriptors": {}, "fallback-config-descriptors": {}, "trigger-condition-descriptors": {}, "plan-validation-jobs": {}, "plan-activation-jobs": {} }

The next example shows how the value of the "location" attribute in the operation can be changed from "Castle Charlottenburg" to "Summer palace Charlottenburg".

PATCH 3gpp/ProvMnS/v1/plans/p1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "replace", "path": "/planConfig/0/value/attributes/location", "value": "Summer palace Charlottenburg" } ]

To delete the planned configuration "p1" a MnS consumer might send.

DELETE 3gpp/ProvMnS/v1/plan-descriptors/p1 HTTP/1.1 Host: example.org

In case of success, the MnS producer returns the following message.

HTTP/1.1 204 No Content Date: Wed, 21 Aug 2024 16:12:45 GMT

The next example shows how a planned configuration can be created for the case where the current configuration data node tree is specified with YANG.

POST 3gpp/ProvMnS/v1/plan-descriptors HTTP/1.1 Host: example.org Content-Type: application/json Accept : application/json,application/problem+json { "name" : "planxyz", "activation-mode" : "BEST_EFFORT", # Execute in a best effort manner trying all operations "configuration-content-type" : "application/yang-patch+json" "configuration": { "ietf-yang-patch:yang-patch" : { "patch-id" : "NRCellDU", "edit" : [ { "operation": "create", "edit-id" : "opId-001", "target": "/_3gpp-common-subnetwork:SubNetwork=Irl/3gpp-common-mecontext:MeContext=Dublin-1/3gpp-common-managed-element:ManagedElement=Dublin-1/3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1", "value": { "NRCellDU": [ { "id": "1", "attributes": { "typeISinglePanelRiRestriction": "15", "ailgDlPrbLoadLevel": "0", "drxOnDurationTimer": "ONDURATIONTIMER_8MS", "trsResourceShifting": "DEACTIVATED", "ulStartCrb": "0", ... } } ] } } } }

# Annex B (informative):
PlantUML for figures

@startuml

left to right direction

object Group_A

object Group_B

object Group_C

Group_A ---o Group_B : is_member_of

Group_B ---o Group_C : is_member_of

Group_C ---o Group_A : is_member_of

@enduml

Figure 6.2.2-1: NOT allowed circular planned configuration group membership relation (with 3 groups)

@startuml

hide empty members

object Group_A

object Group_B

object Group_C

object Plan1

object "Plan1 "

Object Plan2

Object "Plan2 "

Group_A -up-o Group_B : is_member_of

Group_B -down-o Group_C : is_member_of

Plan1 -up-o Group_A : is_member_of

Plan2 -up-o Group_A : is_member_of

"Plan2 " -up-o Group_A : is_member_of

"Plan1 " -up-o Group_C : is_member_of

@enduml

Figure 6.2.2-2: NOT allowed multiple containment of planned configuration

Annex C (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-10 | SA5#157 | S5-245426 |  |  |  | Input Draft TS 28.572 Management of planned configurations | 0.0.0 |
| 2024-10 | SA5#157 | S5-246080 |  |  |  | pCR 28.572 Add skeleton | 0.1.0 |
| 2024-10 | SA5#157 | S5-246081 |  |  |  | pCR 28.572 Add requirements for plans | 0.1.0 |
| 2024-10 | SA5#157 | S5-246082 |  |  |  | pCR 28.572 Add requirements for transaction | 0.1.0 |
| 2024-10 | SA5#157 | S5-246083 |  |  |  | pCR 28.572 Add requirements for validation | 0.1.0 |
| 2024-10 | SA5#157 | S5-246084 |  |  |  | pCR 28.572 Add requirements for activation | 0.1.0 |
| 2024-10 | SA5#157 | S5-246085 |  |  |  | pCR 28.572 Add content for plans | 0.1.0 |
| 2024-10 | SA5#157 | S5-246086 |  |  |  | pCR 28.572 Add content for transactions | 0.1.0 |
| 2024-10 | SA5#157 | S5-246087 |  |  |  | pCR 28.572 Add content for validation | 0.1.0 |
| 2024-10 | SA5#157 | S5-246088 |  |  |  | pCR 28.572 Add content for activation | 0.1.0 |
| 2024-11 | SA5#158 | S5-247183 |  |  |  | pCR 28.572 Add conditional activation of plans | 0.2.0 |
| 2024-11 | SA5#158 | S5-247184 |  |  |  | pCR 28.572 Improve description of clause Planned configurations | 0.2.0 |
| 2024-11 | SA5#158 | S5-247185 |  |  |  | pCR 28.572 Improve description of clause Planned configuration groups | 0.2.0 |
| 2024-11 | SA5#158 | S5-247186 |  |  |  | pCR 28.572 Improve description of clause Validation of planned configurations | 0.2.0 |
| 2024-11 | SA5#158 | S5-247187 |  |  |  | pCR 28.572 Improve description of clause Activation of planned configurations | 0.2.0 |
| 2024-11 | SA5#158 | S5-247188 |  |  |  | pCR 28.572 Improve description of clauses about managing planned configurations | 0.2.0 |
| 2024-11 | SA5#158 | S5-247189 |  |  |  | pCR 28.572 Add examples for REST | 0.2.0 |
| 2025-02 | SA5#159 | S5-250908 |  |  |  | pCR TS 28.572 Improve clause Planned configuration groups | 0.3.0 |
| 2025-02 | SA5#159 | S5-250909 |  |  |  | pCR TS 28.572 Add clause Fallback configurations and Managing fallback configurations | 0.3.0 |
| 2025-02 | SA5#159 | S5-251126 |  |  |  | pCR TS 28.572 Improve clause Trigger conditions | 0.3.0 |
| 2025-02 | SA5#159 | S5-250911 |  |  |  | pCR TS 28.572 Improve clause Validation | 0.3.0 |
| 2025-02 | SA5#159 | S5-250912 |  |  |  | pCR TS 28.572 Improve clause Activation | 0.3.0 |
| 2025-02 | SA5#159 | S5-250913 |  |  |  | pCR TS 28.572 Add clause Asynchronous configuration updates | 0.3.0 |
| 2025-02 | SA5#159 | S5-250914 |  |  |  | pCR TS 28.572 Improve clauses on Managing planned configurations | 0.3.0 |
| 2025-02 | SA5#159 | S5-250915 |  |  |  | pCR TS 28.572 Add information model | 0.3.0 |
| 2025-04 | SA5#160 | S5-251950 |  |  |  | Rel-19 pCR TS 28.572 Improve clauses on Planned configuration concepts | 0.4.0 |
| 2025-04 | SA5#160 | S5-251951 |  |  |  | Rel-19 pCR TS 28.572 Improve clauses on Managing planned configurations | 0.4.0 |
| 2025-04 | SA5#160 | S5-251952 |  |  |  | Rel-19 pCR TS 28.572 Improve clause Information model | 0.4.0 |
| 2025-04 | SA5#160 | S5-251953 |  |  |  | Rel-19 pCR TS 28.572 Improve clause RESTful HTTP-based solution set | 0.4.0 |
| 2025-04 | SA5#160 | S5-251954 |  |  |  | Rel-19 pCR TS 28.572 Allow recursive Planned configuration groups | 0.4.0 |