3GPP TSG-SA5 Meeting #160 	S5-251222

Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

# Source:	Ericsson, AT&T, CMCC

# Title:	Support Continuous MDT on reply to RAN3 incoming LS (R3-250788)

# Document for:	Discussion and Endorsement

Agenda Item:	6.1

# 1	Decision/action requested

The group is asked to discuss and endorse to this proposal.

# 2	References

[1]	3GPP TS 32.422: "Subscriber and equipment trace; Trace data definition and management".

# 3	Rationale

## 3.1	Problem statement

In the incoming LS (R3-250788), SA5 has been asked to provide guidance on possible solutions for Continuous MDT to RAN3.

- Reuse the Management Based MDT framework. A solution should minimise impacts on the current Management Based MDT framework.

- Rely on the management system to configure the RAN with a Management Based MDT configuration to be used for Continuous MDT

- Be network based and avoid impacts on legacy UEs. The Continuous MDT functionality should be enabled by a solution applicable to legacy UEs and not only to Rel-19 UEs.

- Continuous MDT collection targeting the same UE across RRC states

- Support Trace Correlation, that the TCE receiving the Trace Record files containing MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection process from the same UE

## 3.2	Potential Solution

Considering the Continuous management-based MDT solutions presented by RAN3 LS and observations listed above, the following solution is proposed:

- The scope of Continuous management-based MDT

  - According to stage 2 defined in 3GPP TS 32.422[1], the measurement of a management-based MDT is based on the specified areaScope which is specified by management system via TraceJob. The TraceJob includes an assigned TR and MDT configurations.

  - The same principle shall be applied for Continuous management-based MDT. The Continuous management-based MDT is still a management-based MDT with a bigger areaScope, i.e., more than one RAN nodes. And the management system shall define which RAN nodes are participating in this Continuous management-based MDT.

  - In this TraceJob, a specific identifier is used to indicate this is a Continuous management-based MDT. When UE moves between the participating source and target RAN nodes, the UE shall be re-selected by the target RAN node for Continuous management-based MDT.

- Trace Measurement continuity is ensured by

  - A TraceJob is sent to all participating RAN nodes at the time of Continuous management-based MDT initiation. The TraceJob includes a specific Trace Reference identifier (i.e. TR) to indicate this is a Continuous management-based MDT procedure. A dedicated TRSR range is assigned to each RAN node, which is used by RAN node during Trace Recording Session Reference (TRSR) assignment.

  - When a UE is selected for the continuous management-based MDT, and a Trace Recording session is initiated, the gNB allocates a TRSR from the received range. The same TRSR will be used for the whole duration of the Continuous MDT trace of a UE for correlation purpose.

  - If the selected UE is handed over to a participating RAN node, the target RAN node recognize the UE as a Continuous management-based MDT UE based on its TR. Therefore, the UE will be reselected in the target RAN node for the Continuous management-based MDT.

    - If the UE has been configured with Immediate Continuous management-based MDT and the UE moves to a different RAN node, the source RAN node may signal to the target RAN node, e.g. as part of the Handover Request message, the TR and TRSR assigned to the UE. If the target RAN node is a participating RAN node, it understands that the UE was selected for Continuous management-based MDT. Hence the target RAN would select the UE again for further Continuous management-based MDT measurements.

    - If the UE has been configured with Logged Continuous management-based MDT, and the UE moves to RRC_Idle mode or RRC_Inactive state, while in RRC_Idle mode, the UE will collect measurements corresponding to the measurement configuration the UE has received for the logged continuous management-based MDT process. Such measurements will be stored by the UE together with the assigned TR, and TRSR. 	
Once the UE moves to RRC_Connected mode, the UE will report to the serving RAN node the logs of measurements collected while in RRC_Idle or RRC_Inactive state. If the serving RAN node is a participating RAN node, it will therefore identify that the UE was selected for Continuous management-based MDT by looking at the TR and/or TRSR that is reported with them in the received logs. The serving RAN would therefore further select the UE for Continuous management-based MDT, i.e., the UE will be selected for further continuous management-based MDT measurements.

  - If the selected UE is handed over to a non-participating RAN node, the normal MDT procedures will be followed.

    - If the UE has been configured with Immediate Continuous management-based MDT and the UE moves to a non-participating RAN node, the Immediate Continuous management-based MDT is stopped.

    - If the UE has been configured with Logged Continuous management-based MDT, and the UE moves to RRC_Idle mode or RRC_Inactive state, while in RRC_Idle mode, the UE will collect measurements corresponding to the measurement configuration the UE has received for the Logged Continuous management-based MDT process. Such measurements will be stored by the UE together with the TR, and TRSR. 	
When the UE moves to RRC_Connected, the UE will report to the newly serving RAN node the logs of measurements collected while in RRC_Idle or RRC_Inactive.

- Trace Correlation is achieved by:

  - Assigned TR/TRSR will identify that the trace records belong to the same selected UE. The same TRSR will be used for the whole duration of the Continuous management-based MDT trace of a UE.

# 4	Detailed proposal

The proposed solution, as specified in section 3.2, is based on the same principle of using management system to configure the RAN with a Management Based MDT configuration to be used for Continuous MDT. It is reused the current Management Based MDT framework with some minor changes. Such changes have no impacts on current UE mechanism for management-based MDT, i.e., no impacts on legacy UEs. This solution allows Continuous MDT collection targeting the same UE across RRC states. It also supports Trace Correlation of the same UE by TR/TRSR combination.

It is proposed to endorse the proposal solution as specified in section 3.2, and response to RAN3 accordingly.