3GPP TSG-SA5 Meeting #160	S5-251277

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0311 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Update 5.3.1 Intent handling capability obtaining |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2024-3-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. The policy should be given by MnS consumer, which cannot be modelled as supported capabilities provided by MnS producer. 2. The original statement specifically refers to "RAN or Core network service" which may limit the scope to specific domains of network services. This is generic use case, it is better to make it generic. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Delete “policy context” related content in the table, fill in the missing entries in "units" column, and clarify the possible values for "objectType" can include all network services, not just RAN and 5GC. 2. Replace "RAN or Core network service" with "network service". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The policy and context are not the description for supported description of supported expectation targets. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1.1.2, 5.3.1.1.3, 5.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.3.1	Intent handling capability obtaining

5.3.1.1	Introduction

5.3.1.1.1	Information on supported expectation object and corresponding expectationTargets.

Clause 4.2.2 described that an Intent-driven MnS producer has the following capabilities: fulfil the received intent and report the result/information about the intent fulfilment, and clause 6.2.2 defined different scenario specific intent expectations with different expectation objects and expectation targets to support different use cases. In a network, multiple intent handling functions may be deployed to support different kinds of intents. Different intent handling functions may be deployed to support different intent expectation object domains, e.g. intent handling function A is deployed to handle the radio network related intents, intent handling function B is deployed to handle the 5GC network related intents, while intent handling function C is deployed to handle the service related intents. Or different intent handling functions are deployed to support different areas of the same intent expectation object domain, e.g. intent handling function D is deployed to support to handle the intent for radio network in Area#1, while intent handling function E is deployed to support to handle the intent for radio network in Area#2.

Before MnS consumer expresses the intent expectation targets and expectation objects to MnS producer, MnS consumer may want to know what expectation targets and expectation objects can be supported by MnS producer. Based on such supported expectation targets information and expectation objects information, the MnS consumer may use such information to select the proper intent handling function to express the intent.

In case the MnS producer updates the intent handling capabilities for one or more intent handling functions, the MnS producer may inform these updates to the MnS consumer.

5.3.1.1.2	Description of supported expectation targets

Besides the list of supported expectation objectType and expectationTargets, the intent handling capability needs to also describe the supported value ranges for the targets or the sets of targets that are supported. For instance, the intent driven MnS producer for network service management intent may want to expose the description of RAN services that they can offer. This means the intent driven MnS producer exposes the supported intents or intent expectations having a specific set of targets and a specific set of values for those targets as illustrated by Table 5.3.1.1.2-1. The IDMS producer should support expose of such capabilities.

| Object Type |  |  |  |  | network service |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Targets or Contexts | Units | Value Options |  |  |  |  |  |  |  |  |
| End-to-end latency | ms | 0.5 | 1 | 2 |  | 5 | 10 | 25 | 50 | 100 |
| Jitter | ms | .001 | 0.01 | 0.1 |  | 1 | 2 | 5 | 10 | 20 |
| Survival Time | ms | 0 | 1 | 2 |  | 5 | 10 | 25 | 50 | 100 |
| Availability | % | 99 | 99.5 | 99.9 |  | 99.995 | 99.999 | 99.9995 | 99.9999 | .99999 |
| Reliability | % | 99 | 99.5 | 99.9 |  | 99.995 | 99.999 | 99.9995 | 99.9999 | .99999 |
| End user Data rate | Mbps | 0.1 | 0.2 | 0.5 |  | 1.0 | 2.0 | 5.0 | 10.0 | 20.0 |
| Payload | Bytes | 1 | 2 | 5 |  | 10 | 20 | 100 | 500 | 1000 |
| Traffic density | Gbps/km2 | 1 | 2 | 5 |  | 10 | 20 | 50 | 100 | 1000 |
| Connection density | '000/km2 | 0.1 | 0.5 | 1 |  | 5 | 10 | 20 | 50 | 100 |
| Service area size | A km x B km | 0.1x10 | 0.1x100 | 0.1x500 |  | 0.1 x 0.1 | 1 x1 | 10 x10 | 50 x50 | 100x100 |
| : | : | … |  |  |  |  |  |  |  |  |

Table 5.3.1.1.2-1: An example of a service offer description indicating the possible characteristics of
network services that can be exposed via an intent handling capability as the service characteristic offered by
 an intent driven MnS producer for service management intents

5.3.1.1.3	Describing alternative expectations

The IDMS may be used by the network service management MnS producer to enable the design of a specific network service management workflow between the MnS producer and the MnS consumer.

Based on the extended content of intent handling capability as described in 5.3.1.1.2, the network service MnS consumer can generate an artifact, say called a service-offer description that describes the different candidate characteristics of the desired network service from the MnS consumer's point of view. The MnS consumer provides a service-offer description to the network service management system for validation. The service-offer description may be viewed as a network service management intent that contains a list of candidate intent expectations, i.e., the expectations are candidates that may be separately submitted by the MnS consumer for fulfilment.

On receiving the service-offer description the network service Management MnS producer validates the service-offer description and confirms to what extent the network service intent as described by the service-offer description can be supported. For that, the network service management MnS producer returns the service-offer description but indicates only the supported combinations of network service features and values with contexts under which the network service may be offered.

Figure 5.3.1.1.3-1: Example interaction between an MnS producer and MnS consumer on
the exposure of capabilities for service management intents

The service offer description can use the capability of exposing alternative expectations to express the services that can be offered e.g. by a RAN service or Edge service intent handling function. It is up to the MnS producer how description information is provided. It is not indicative of ability to fulfil an intent as many factors can change over time depending on network conditions.

5.3.1.2	Requirements

REQ-IDMS_IHCO-CON-1 The intent driven MnS producer shall have capabilities enabling an MnS consumer to obtain intent handling capabilities of each intent handling function, including supported expectation object and targets.

REQ-IDMS_IHCO-CON-2: The intent driven MnS producer should support a capability to provide a description of the supported scenario specific intents to the authorized MnS consumer, including value ranges for the supported expectation targets.

REQ-IDMS_IHCO-CON-3: The intent driven MnS producer should support a capability enabling an MnS consumer to list the set of potential alternative expectations that the MnS consumer would like to be evaluated for whether it is feasible or fulfillable.

End of Change