3GPP TSG-SA5 Meeting #160	S5-251339

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0682 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.552 Add a new measurement related to transmission rate related to the data collection from DCCF or NWDAF hosting DCCF performed by NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NWDAF_OAM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-3-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The NWDAF data collection may be performed via DCCF, when DCCF is deployed or via NWDAF hosting DCCF. NWDAF may obtain data using Ndccf_DataManagement_Subscribe/Nnwdaf_DataManagement_Subscribe services operations directly. Therefore, in such cases, the DCCF(s) or the NWDAF(s) hosting DCCF can be considered as the source of NWDAF data collection. Since the NWDAF and the data sources may not be deployed close to each other from transmission point of view, the time used for transmission from different data sources to the NWDAF varies, too. Therefore, the transmission rate can be used to evaluate the NWDAF data collection efficiency. For a given time length during the observation, the transmission rate can be measured as the ratio between the total amount of data collected by NWDAF from DCCF/NWDAF hosting DCCF and the corresponding accumulated time used for transmitting those amount of data from the DCCF/NWDAF hosting DCCF to the NWDAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new measurement “Transmission rate related to the data collection from DCCF or NWDAF hosting DCCF performed by NWDAF” is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator will not be able to monitor the transmission rate of NWDAF performing data collection from DCCF/NWDAF hosting DCCF and to take management actions for configuration and resource allocation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

5.18	Performance measurements for NWDAF

5.18.5	Measurements related to the NWDAF Data Collection

5.18.5.x Transmission rate of NWDAF performing data collection from DCCF/NWDAF hosting DCCF

a) This measurement provides the transmission rate related to the data collection from DCCF/NWDAF hosting DCCF performed by NWDAF

b)	CC

c)	This measurement is provided by averaging the total amount of data collected from one or multiple DCCF(s)/NWDAF(s) hosting DCCF with the transmission time used for each notification and response corresponding to the subscription and request for NWDAF to perform data collection from the aforesaid one or multiple DCCF(s) /NWDAF(s) hosting DCCF.

d)  A real value

e)	The measurement name has the form DANS.DataCollectTransmissionRate.withDCCF.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

End of Changes