3GPP TSG-SA5 Meeting #160	S5-251568

Goteborg, Sweden, 07 - 11 April 2025

Source:	Huawei

Title:	Pseudo-CR on TS 28.561 Remove abbreviations in clause 5.1

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS 28.561

Version:	0.1.0

Work Item:	NDT

Comments

The abbreviation of each clause title is only used for associating requirements with use cases, which can be easily achieved by using clause number. In addition, the abbreviation decreases the readbility of the sepc and doesn’t align with clause 5.2.

Proposed Changes

* * * First Change * * * *

5.1	Control and life cycle management of NDTs

5.1.1	Description

This describes uses on control and life cycle management of NDT instances.

5.1.2	Use cases

5.1.2.1	Life cycle management of NDT instances

NDTs can be used for many application use cases. In each case, the NDT needs to implement a defined network scenario, run a simulation or emulation of that scenario, and subsequently provide an output representing the statuses of different network for the simulated network scenario. The lifecycle management of Network Digital Twins (NDT) refers to the management processes that control and transition an NDT instance through the processes of creation, activation, operation, deactivation, deletion and any updates during these processes. At each step, it is critical to ensure that the NDT instance accurately reflects the status of physical entities and support efficient simulation/emulation.

-	For creation, the NDT MnS Consumer requests the creation of an NDT instance based on the capabilities of the NDT producer. The NDT MnS consumer may request the NDT producer to create a new NDT from scratch, to update existing NDT models or to combine existing NDT models to generate a new NDT.

-	At any point during the life of an NDT instance, the MnS consumer may refine and adjust the NDT instance to accurately reflect the status of the physical entity and adapt it to specific application scenarios. For example, the consumer may trigger data collection from the network and synchronization of the state with the NDT.

5.1.2.2	Control of NDT instances

The 3GPP management system should support capabilities enabling consumers to define the network scenario, to control (start, suspend, resume or stop) the simulation or emulation of that network scenario and to provide outcomes representing measurements and counters from metrics in the simulated or emulated network scenario.

An NDT may have dependencies on other NDTs or DTs, each of which implements a specific network scope. The MnS consumer should also be enabled to configure or modify the dependencies e.g., to select which environment or radio propagation DT to be applied.

5.1.2.3	Synchronization with network

The NDT needs to obtain information from network functions and entities. The collected information includes the following:

-	Performance data: Performance measurement or the KPI as defined in 3GPP TS 28.552 [2] and 3GPP TS 28.554 [3].

-	MDT/Trace data: MDT measurements as defined in 3GPP TS 32.422 [4].

-	Fault management data: Fault management data as defined in 3GPP TS 28.111[5]

-	Configuration data: Configuration data defined in 3GPP TS 28.541 [6] and 3GPP TS 28.622 [7], any of the available MOIs;

Mobile networks are generally composed of numerous network elements. The data about these numerous network elements can potentially be vast. When collecting this vast amount of data to NDT, the collection might cause congestion in the network bandwidth used for data collection. The MnS producer should enable the MnS consumer to control the frequency of data to be collected by the NDT to minimize unnecessary collection of data and minimal congestion.

5.1.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-DTLCM-DTLCM-01: | The NDT should support a capability enabling an MnS consumer to define the network scenario that should be modelled and simulated/emulated. | Life cycle management of NDT instances (See clause 5.1.2.1) |
| REQ-DTLCM-DTLCM-02: | The NDT should support a capability enabling an MnS consumer to control the simulation/emulation of a network scenario in the gNB, e.g. to start, or stop the simulation/emulation. | Life cycle management of NDT instances (See clause 5.1.2.1) |
| REQ-DTLCM-DTLCM-03: | The NDT should support a capability to provide to the MnS consumer an output of a modelling or simulation process, the output representing the statues of the network for the simulated scenario. | Life cycle management of NDT instances (See clause 5.1.2.1) |
| REQ-DTLCM- DTSYNC-01: | The NDT shall have the capability enabling the MnS consumer to control the frequency of network data collection. | Synchronization with network (See clause 5.1.2.3) |

* * * End of Change * * * *