3GPP TSG-SA5 Meeting #160	S5-251601

Goteborg, Sweden, 07 - 11 April 2025

# Source:	China Mobile, FiberCop

# Title:	TS28.561 Add the NDT concept and scope

# Document for:	Approval

Agenda Item:	6.19.5.1

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

None.

3	Rationale

Add the NDT concept and scope in clause 4.

4	Detailed proposal

Start of Change

4	Concepts and overview

4.x	Introduction and Overview

4.x.1	Digital Twins and Network Digital Twins

A Digital twin is a representation of an object that models the characteristics and behaviors of the physical object. A digital twin can be created for any physical object, including any objects in communication networks. The digital twin may also be created for a group of objects, e.g. for the sets of network objects that form the RAN or NFs in Core network.

Accordingly, a digital twin modelling an object of a communication network may be called Network Digital twin.

Network Digital Twin (NDT) can be used as a replica of a mobile network, in order to learn how an actual mobile network would behave in certain scenarios, without causing any changes to the actual mobile network. To provide meaningful results, the Network digital twin needs to emulate (or simulate) the behavior of the mobile network, so that the result of the operations on the virtual replica are a good approximation to similar operations on the actual network. The standardization for an NDT focuses on the implementation of independent aspects of a network.

3GPP uses the Network Resource Model (NRM) to model the attributes of a mobile network. The concept of Network Digital Twin also adds the ability to model the behavior of a mobile network. This behavior is modelled by emulating or simulating a complete mobile network or parts of a mobile network.

Existing automation capabilities include analytics services and decision-making capabilities with the assumption that the knowledge about network behavior is available within the automation functions. Accordingly, the unaddressed gap for network automation is the capabilities for modelling the behavior of the network. This knowledge on how the network behaves or will behave is the new value provided by digital twins that understands and models the behavior of the network.

The implementation of NDT modelling rely on simulation, emulation or other modelling technologies that enables the NDT to mimic the behaviour of the network. Simulation relies on mathematical models to abstract network functions and management behavior. By parameterizing configurations and state data, it constructs a virtual network environment and models network traffic to predict system dynamics. Emulation uses actual algorithms or functional modules to replicate real network behavior in an NDT environment. It requires virtualization of network equipment, functions, and management components, along with complete configuration and state data. It may be possible to combine emulation and simulation to create an integrated solution. The specific choice between these approaches depends on implementation and deployment consideration, which will not distinguish in this specification,

4.1	NDT Management Service

The NDT MnS can be requested by the NDT MnS consumer to model the behaviour of the network and generate the simulation/emulation output. The simulation/emulation capabilities provided by NDT MnS Producer can be invoked by the network intelligence and automation management functions (e.g. MDAF, AIML Inference Function, Intent Handling Function, etc.) playing the role of NDT MnS Consumer to support their intelligence and automation functionality.

Figure 4.1-1