# 3GPP TSG SA WG5 Meeting #160                 	           S5-251644

# Goteborg, SWEDEN 7 - 11 April 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.251 | CR | 0523 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add charging information for store and forward satellite operation with UP CIoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-CH |  |  |  |  |  | Date: |  |  | 2025-3-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the conclusion of the TR 28.846, in order to support the store and forward satellite operation with UP CIoT, the charging information for S&F in the MME-CDR need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add charging information for store and forward satellite operation charging with UP CIoT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The store and forward feature and UE-satellite-UE communication feature are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.1.X	UP CIoT EPS Optimisation charging data in MME (MME-CDR)

If the collection of MME data is enabled then the MME data specified in table 6.1.x.1 shall be available for the UP CIoT EPS Optimisation bearer with store and forward satellite operation.

Table 6.1.x.1: MME UP CIoT record (MME-CDR)

| Field | Category |  | Description |
| --- | --- | --- | --- |
|  | SGSN | MME |  |
| Record Type | M | M | MME UP CIoT |
| Retransmission | - | OC | This parameter, when present, indicates that information from retransmitted Charging Data Requesthas been used in this CDR. |
| Served IMSI | M | M | The IMSI of the subscriber. |
| Served IMEI | OC | OC | The IMEI or IMEISV of the ME, if available. |
| Served MSISDN | OC | OC | The primary MSISDN of the subscriber, if available. For MSISDN-less subscription, this field holds the Dummy MSISDN as defined in TS 23.003 [103]. |
| Serving Node Type | M | M | Type of node handling the MME. |
| Recording Entity | OM | OM | The E.164 number of the MME. |
| Serving Node Address | - | OM | The address of the MME used. |
| Serving Node IPv6 Address Address | - | OC | The IPv6 address, in case of IPv4v6 dual stack, of the MME. |
| MME Name | - | OM | The operation Identity of the MME. |
| MME Realm | - | OM | The operation Realm Identity of the MME. |
| User Location Information | - | OC | This field contains the User Location (User location Information as defined in TS 29.274 [210]) from which the message originated. |
| User Location Information Time | - | OC | This field contains the time at which the user location information was acquired. |
| Message Reference | M | M | A reference provided by the MS uniquely identifying this message. |
| Event Time Stamp | M | M | The time at which the message was received by the MME from the subscriber. |
| Node ID | OM | OM | Name of the recording entity. |
| Local Record Sequence Number | OM | OM | Consecutive record number created by this node. The number is allocated sequentially including all CDR types. |
| Charging Characteristics | M | M | The Charging Characteristics flag set used by the MME. |
| Serving Node PLMN Identifier | Oc | Oc | This field holds the PLMN Identifier (MCC and MNC) serving the UE during this record. Only one value is included in case multiple PLMN Ids are supported by the MME. |
| CN Operator Selection Entity | OC | OC | This field indicates whether the Serving Network identified by the "Serving Node PLMN Identifier” has been selected by the UE or by the network, as defined in TS 29.060 [203] for GPRS, and in TS 29.274 [210] for EPC, if available. |
| RAT Type | OC | OC | This field indicates the Radio Access Technology (RAT) type currently used by the Mobile Station as defined in TS 29.061 [205], when available. |
| Charging Characteristics Selection Mode | OM | OM | Holds information about how Charging Characteristics were selected. |
| Record Extensions | OC | OC | A set of network operator/ manufacturer specific extensions to the record. Conditioned upon the existence of an extension. |
| Satellite Access Indicator |  | OC | This field holds the use of 4G Satellite Access. |
| Satellite information |  | OC | This field holds all the satellite information that support to finish the Store and Forward Satellite operation. |
| Store and Forward indicator |  | OC | This field holds the use of Store and Forward Satellite operation. |
| S&F Monitoring list (1..max) |  | OC | This field holds the satellite IDs that used for the Store and Forward Satellite operation. |

End of change