3GPP TSG-SA5 Meeting #160	S5-251814

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.240 | CR | 0511 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.240 CAPIF Logical Charging Architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_CAPIF_EXT |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Enable CHF support for CAPIF service |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add CAPIF to the Logical ubiquitous charging architecture |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CCF is not visible in the Charging architecture |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3, 4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-251435 |  |  |  |  |  |  |  |  |

First change

### 4.2.3	Common architecture – service based interface

The following figures provide an overview of the logical ubiquitous charging architecture and the information flows for converged offline and online charging in service based interface variant for 5G systems and Edge Computing enabling sub-systems.

Figure 4.2.3.1 provides the overview in service based representation:

Figure 4.2.3.1: Logical ubiquitous charging architecture and information flows for 5G systems – service based representation

Figure 4.2.3.2 provides the overview in reference point representation:

Figure 4.2.3.2: Logical ubiquitous charging architecture and information flows for 5G systems – reference point representation

The reference points are defined in clause 4.4.3.

For the sake of simplicity, the SMF+PGW-C is not explicitly added in Figure 4.2.3.1 and Figure 4.2.3.2, and is represented by the SMF. The SMF+PGW-C uses Nchf for 5GS and EPC interworking as well as when enhanced to support GERAN/UTRAN.

The Nchf_SpendingLimitControl service exposed by CHF and consumed by the PCF is specified in 3GPP TS 23.502 [214] and 3GPP TS 23.503 [297].

Third change

### 4.4.3	Charging services Reference point

The common charging architectures are mapped into the specific domain/subsystem/service charging architectures in the respective middle tier TSs, which contain in their reference point representation, the following reference points:

N28:	Reference point between PCF and CHF defined in TS 23.501[215].

N40:	Reference point between SMF and the CHF in the same PLMN defined in clause 4.2 of TS 32.255 [15].

N41:	Reference point between AMF and CHF in different PLMNs defined in clause 4.2 of TS 32.256 [16].

N42:	Reference point between AMF and CHF in the same PLMN defined in clause 4.2 of TS 32.256 [16].

N44:	Reference point between NEF and CHF defined in clause 4.4 of TS 32.254 [14].

N45:	Reference point between IMS Node and CHF defined in clause 4.4 of TS 32.260 [20].

N46:	Reference point between SMSF and CHF defined in clause 4.4 of TS 32.274 [34].

N47: 	Reference point between SMF and the CHF in different PLMNs defined in clause 4.2 of TS 32.255 [15].

N48: 	Reference point between 5G DDNMF and the CHF in different PLMNs defined in clause 4.4 of TS 32.277 [37].

N49: 	Reference point between EES and CHF defined in clause 4.2.3 of TS 32.257 [17].

N100:	Reference point between MMS node and CHF defined in clause 4.4 of TS 32.270 [30].

N101:	Reference point between MB-SMF and the CHF in the same PLMN defined in clause 4.2 of TS 32.279 [39].

N102:	Reference point between NSACF and the CHF defined in clause 4.2.1 of TS 28.203 [72].

N103:	Reference point between NSSAAF and the CHF defined in clause 4.2.1 of TS 28.204 [73].

N104:	Reference point between TSN AF and CHF defined in clause 4.2 of TS 32.282 [42].

N105:	Reference point between TSCTSF and CHF defined in clause 4.2 of TS 32.282 [42].

N106:	Reference point between GMLC and CHF defined in clause 4.4 of TS 32.271 [31].

N107:	Reference point between two CHFs (V-CHF and H-CHF) defined in clause 4.2 of TS 32.255 [15] and clause 4.2 of TS 32.256 [16].

N108:	Reference point between two CHFs (C-CHF to B-CHF) defined in clause 4.3 of TS 32.255 [15] and clause 4.3 of TS 32.256 [16].

N109:	Reference point between CCF and the CHF defined in clause 4.x.y of TS 32.254 [14].

End of changes