3GPP TSG-SA5 Meeting #160	S5-251835

Goteborg, Sweden, 07 - 11 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.271 | CR | 0034 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extend charging information and bindings for 5G LCS converged charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is no charging information and bindings for 5G LCS converged charging |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend charging information and bindings to support 5G LCS converged charging |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G LCS converged charging will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3, 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.291 CR 0616 TS 32.298 CR 1034 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.3	LCS charging specific parameters

6.3.1	Definition of LCS charging information

6.3.1.0	General

The LCS Information parameter used for LCS charging is provided in the Service Information parameter.

6.3.1.1	LCS charging information assignment for Service Information

The components in the Service Information that are use for LCS charging can be found in Table 6.3.1.1.

Table 6.3.1.1.1: Service Information used for LCS Charging

| Information Element | Category | Description |
| --- | --- | --- |
| Service Information | OM | A set of fields hold the 3GPP specific parameter as defined in TS 32.299 [50]. |
| Subscriber Identifier | OM | Used as defined in TS 32.260 [20]. |
| LCS Information | OM | This field holds a set of MMTel services with theirs specific parameters. The details are defined in clause 6.3.1.2. |

6.3.1.2	Definition of the LCS Information

LCS specific charging information is provided within the LCS Information, and the detailed structure of the LCS Information can be found in table 6.3.1.2.1.

Table 6.3.1.2.1: Structure of the LCS Information

| Information Element | Category | Description |
| --- | --- | --- |
| LCS Client ID | OC | This field holds the ID of the LCS client that invoked the LR, if available. |
| Location Type | OC | This field holds the type of location information being requested in case of MT-LR. |
| Location Estimate | OC | This field denotes the location of an MS in universal coordinates and the accuracy of the estimate upon succesful MO-LR. |
| Positioning Data | Oc | This field indicates the positioning method that was attempted to determine the location estimate for MO-LR, if available. |
| Target UE ID | OC | The identity of Target UE in LCS, which is applicable to 5GS LCS. |

6.3.1.3	Definition of Ranging and Sidelink Positioning Charging Information

Ranging and Sidelink Positioning specific charging information is provided within the Ranging and Sidelink Positioning Charging Information, and the detailed structure of the Ranging and Sidelink Positioning Charging Information can be found in table 6.3.1.3.1.

Table 6.3.1.3.1: Structure of the Ranging and Sidelink Positioning Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| Target UE ID | OC | The identity of Target UE in Ranging/Sidelink positioning |
| SL Reference UE ID | OC | The identity of SL Reference UE in Ranging/Sidelink positioning |
| SL Positioning Server UE ID | OC | The identity of SL Positioning Server UE in Ranging/Sidelink positioning |
| Located UE ID | OC | The identity of Located UE in Ranging/Sidelink positioning |
| Location Type | OC | This field holds the type of location information being requested. |
| Location Estimate | Oc | This field denotes the location of a Target UE and the requested accuracy of the estimate. |

6.3.2	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data for 5GS LCS converged charging.

The Operation types are listed in the following order: I [Initial] / U (Update)/T [Termination]/E [event]. Therefore, when all Operation types are possible it is marked as IUTE. If only some Operation types are allowed for a node, only the appropriate letters are used (e.g. IUT or E) as indicated in the table heading. The omission of an Operation type for a particular field is marked with "-" (e.g. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.3.2.1 illustrates the basic structure of the supported fields in the Charging Data Request for 5GS LCS converged charging.

Table 6.3.2.1: Supported fields in Charging Data Request message

| Information Element | Node Type | GMLC |
| --- | --- | --- |
|  | Supported Operation Types | E |
| Subscriber Identifier |  | E |
| Tenant Identifier |  | E |
| NF Consumer Identification |  | E |
| NF Functionality |  | E |
| NF Name |  | E |
| NF Address |  | E |
| NF PLMN ID |  | E |
| Charging Identifier |  | E |
| Invocation Timestamp |  | E |
| Invocation Sequence Number |  | E |
| One-time Event |  | E |
| One-time Event Type |  | E |
| Retransmission Indicator |  | E |
| Supported Features |  | E |
| Service Specification Information |  | E |
| Ranging and Sidelink Positioning Information |  | E |
| LCS Information |  | E |

Table 6.3.2.2 illustrates the basic structure of the supported fields in the Charging Data Response for Ranging and Sidelink Positioning converged charging.

Table 6.3.2.2: Supported fields in Charging Data Response Message

| Information Element | Node Type | GMLC |
| --- | --- | --- |
|  | Supported Operation Types | E |
| Session Identifier |  | E |
| Invocation Timestamp |  | E |
| Invocation Result |  | E |
| Invocation Result |  | E |
| Failed parameter |  | E |
| Failure Handling |  | E |
| Invocation Sequence Number |  | E |
| Session Failover |  | E |
| Supported Features |  | E |

6.3.3	Formal Ranging and Sidelink Positioning converged charging parameter description

6.3.3.1	Ranging and Sidelink Positioning charging CHF CDR parameters

The detailed definitions, abstract syntax and encoding of the Ranging and Sidelink Positioning charging CHF CDR parameters are specified in TS 32.298 [51].

6.3.3.2	5GS LCS charging resources attributes

The detailed definitions of resources attributes used for 5GS LCS charging are specified in TS 32.291 [56].

Next change

6.4	Bindings for 5GS LCS converged charging

This mapping between the Information Elements, resource attributes and CHF CDR parameters for 5GS LCS converged charging is described in clause 7 of TS 32.291 [56].

End of changes