3GPP TSG-SA5 Meeting #160	S5-251906

Goteborg, Sweden, 07 - 11 April 2025 	Revision of S5-251242

Source:	Nokia, Samsung

Title:	Pseudo-CR on CCL Goal-targets and actions conflicts

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.1.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add use case and requirements for CCL Goal-targets conflicts handling as was agreed in the CCL study in TR28. 867

Proposed Changes

* * * First Change * * * *

# 5	Management Capabilities

## 5.A	 CCL Conflict management Capability - CONF

### 5.A.1			Description

A CCL may experience direct conflicts on its goals, targets, scopes, trigger time and execution time. The management system needs to support capabilities to avoid, detect and resolve the conflicts.

### 5.A.2	Use Cases

#### 5.A.2.1	CCL action conflicts handling –CONF_0z

When two (or more) CCLs attempt to adjust the same network parameter but with different and contradicting values, the desired actions of the 2 CCL will be in conflict. For example, a CCL assuring throughput of a slice may be scaling-out the virtual resources of the slice. Whereas a CCL minimizing the energy consumption may be scaling-in the virtual resource of the same slice. It can be when the CCLs execute actions at the same time. However, it also happens when the CCLs execute at different times, and the scenario for actions to be separated in time is the more likely than actions occurring simultaneously. casein these conflict scenarios, the network parameter continuously ping-pongs between the two values. Such a conflict may be called an action conflict.

NOTE:	A potential conflict can for example be detected if a CCL observed that PMs on a certain object keep flipping between two values. The constant flipping can be an indication that 2 CCL instances are attempting to change the same scope.

The CCL may detect or observe events that identify the conflicts. The conflict can be avoided using some information or the policies (e.g. priority) provided by the consumer. If the conflict actually occurs, the CCL MnS producer should support services to inform MnS consumers the confirmed detected conflicts. It is needed to maximize the avoidance of conflict, including “requesting” information from MnS consumer and to inform MnS consumer about the potential conflict. CCL MnS Producer may also provide recommendations, for updating/deleting the conflicting CCLs, that would result in the resolution of detected conflict. The recommendation for update may include suggestions for modified targets.

Note: the exact information that can be exchanged is FFS

### 5.A.3	Requirements

| Requirement label | Description | Related use case(s)/ Motivation |
| --- | --- | --- |
| REQ-CONF-CONF_0z-01 | The 3GPP Management System should support a capability to detect and inform an authorized MnS consumer about a potential action conflict. | CONF-CONF_0z |
| REQ-CONF- CONF_0z-02 | The 3GPP Management System should support a capability to confirm and inform an authorized MnS consumer about an actual action conflict. | CONF-CONF_0z |
| REQ-CONF- CONF_0z-03 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to resolve a CCL action conflict. | CONF-CONF_0z |
| REQ-CONF- CONF_0z-04 | The 3GPP Management System should enable authorized MnS consumers to provide information that can be used to avoid the action conflict. | CONF-CONF_0z |
| REQ-CONF- CONF_0z-05 | The 3GPP Management System should support a capability to coordinate the resolution of CCL action conflicts among multiple CCLs | CONF-CONF_0z |

* * * End of Changes * * * *