3GPP TSG-SA5 Meeting #160	S5-251922

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0310 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Update procedures for intent report management to support implicit intent report subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The intentReportControl is introduced in the Intent IOC to support the implicit intent report subscription mechanism. However, in Annex E.1.2 Intent report subscription and notification, only explicit procedures for intent report subscription and notification is described. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updat the procedure for intent report subscription and notification to support implicit intent report subscription mechanism |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The implicit intent report subscription mechanism is missing in procedure for intent report subscription and notification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.1.2, A.1.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

E.1.2	Intent report subscription and notification

Figure E.1.2-1 illustrates the procedure for intent report subscription and notification.

Figure E.1.2-1: Procedure for intent report subscription and notification

The following step 1a-1c describes the procedures for explicit intent report subscription.

1a.	The MnS Consumer sends a request to create a NtfSubscriptionControl instance (see createMOI operation defined in TS 28.532 [3]) to the MnS Producer with intent report subscription information (see attributes of NtfSubscriptionControl IOC in TS 28.622 [6]) to establish a subscription for the attribute value change notification for IntentReport MOI. The "objectInstance" of IntentReport MOI is specified in the attribute "scope" of NtfSubscriptionControl IOC.

- If the MnS Consumer wants to subscribe the Intent Fulfilment information, the attribute name 'intentFulfilmentReport' needs to be specified in the attribute 'notificationFilter' of NtfSubscriptionControl IOC.

- If the MnS Consumer wants to obtain Intent conflict information, the attribute name 'intentConflictReports' needs to be specified in the attribute 'notificationFilter' of NtfSubscriptionControl IOC..

- If the MnS Consumer wants to obtain Intent fulfilment feasibility check information, the attribute name 'intentFeasibilityCheckReport' needs to be specified in the attribute 'notificationFilter' of NtfSubscriptionControl IOC.

- If the MnS Consumer wants to obtain intent exploration report, the attribute name 'intentExplorationReport' needs to be specified in the attribute 'notificationFilter' of NtfSubscriptionControl IOC.

- If the MnS Consumer wants to obtain intent negotiation report, the attribute name 'intentNegotiationReport' needs to be specified in the attribute 'notificationFilter' of NtfSubscriptionControl IOC.

1b.	Based on the request, the MnS Producer creates and configures NtfSubscriptionControl MOI to establish a subscription for the attribute value change notification for IntentReport MOI.

1c.	The MnS Producer sends a response (see createMOI operation defined in TS 28.532[3]) to the MnS Consumer with attribute "objectInstance" of the created NtfSubscriptionControl MOI intent instance.

The following steps 2a-2c describes the procedures for implicit intent report subscription.

2a. The MnS Consumer sends a request to configure the IntentReportControl of the intent instance to the MnS Producer with intent report control information (see IntentReportControl <<dataType>> in clause 6.2.1.3.16) to establish a subscription for the attribute value change notification for IntentReport MOI. The request can be intent instance creation (see createMOI operation defined in TS 28.532 [3]) or modification request (see modifyMOIAttributes operation defined in TS 28.532 [3]).

2b. Based on the request, the MnS Producer configures NtfSubscriptionControl of the Intent MOI to establish a subscription for the attribute value change notification for IntentReport MOI. If the request is intent instance creation request, MnS producer needs to create the intent MOI before configuring the NtfSubscriptionControl of the Intent MOI.

2c. The MnS Producer sends a response (see createMOI operation or modifyMOIAttributes defined in TS 28.532[3]) to the MnS Consumer with attribute "objectInstance" of the created or modified Intent MOI.

The following steps are executed until NtfSubscriptionControl MOI is deleted.

3.	The MnS Producer configures the value of attributes for IntentReport MOI at the end of each observation period.

4.	The MnS Producer sends a notification (see notifyMOIAttributeValueChanges notification defined in TS 28.532[3]) to the subscribed MnS Consumer about intent report information, including at least one of the attributes 'intentFulfilmentReport', 'intentConflictReports', and 'intentFeasibilityCheckReport'.

2nd Change

A.1.5.3	Intent report subscription and notification

@startuml

title [Intent report subscription and notification]

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

Alt 'Explicit intent report subscription'

MnS_Consumer -> MnS_Producer: 1a. Requst to create NtfSubscriptionControl instance for an intent report instance\n(including objectInstance and list of NtfSubscriptionControl IOC)

MnS_Producer -> MnS_Producer: 1b. Create a NtfSubscriptionControl MOI for the intent report instance \n and configure the created NtfSubscriptionControl MOI

MnS_Producer -> MnS_Consumer: 1c. Reponse for creating NtfSubscriptionControl instance for an intent report instance

else 'Implicit intent report subscription'

MnS_Consumer -> MnS_Producer: 2a.Request to configure intentReportControl of the Intent instance \n (including objectInstance of the Intent MOI and list attributes of IntentReportControl)

MnS_Producer -> MnS_Producer: 2b.Configure the intentReportControl of Intent instance with the \n specified vlaues for the list attributes of IntentReportControl

MnS_Producer -> MnS_Consumer: 2c. Reponse for configuring intentReportControl of the Intent instance

end

loop until the NtfSubscriptionControl MOI or Intent MOI is deleted

MnS_Producer -> MnS_Producer: 3. Configure the value of the atttributes for IntentReport MOI

MnS_Producer -> MnS_Consumer: 4. Notify intent report information \n (objectinstance, attributeListValueChanges of IntentReport MOI)

end

hide footbox

@enduml

End of Changes