3GPP TSG-SA5 Meeting #160	S5-251948

Goteborg, Sweden, 07 - 11 April 2025

# Source:	Nokia, Huawei, AT&T, Ericsson

# Title:	Discussion paper on MSED components, requirements and naming

# Document for:	Endorsement

Agenda Item:	6.19.21.1

# 1	Decision/action requested

The group is asked to endorse the proposal.

# 2	References

[1] 3GPP TS 28.579, "Management services exposure to external consumers through CAPIF".

# 3	Rationale

## 3.1 MSED vs MSEDF naming convention

The group discussed and agreed not to focus on the name whether MSED or MSEDF and put this discussion on pause for Rel-20. For Rel-19, we agreed to proceed with the MSED naming convention.

### 3.2 3GPP management services exposure

In the context of 3GPP management services exposure, there are different exposure scenarios depending on the type of the MnS consumer as illustrated in Figure 3.2-1.

Figure 3.2-1: Management services exposure scenarios

From Figure 3.2-1, the following observations can be made:

##### Observation 1	Types of MnS consumers

- Internal MnS consumer – MnS consumer within the PLMN trust domain. There are two categories of internal MnS consumers:

  - MnS consumer within the 3GPP management system, e.g. SA5 Provisioning MnS consumer, SA5 Fault management MnS consumer and SA5 streaming MnS consumer. This is equivalent to internal MnS consumer X1 in Figure 3.2-1.

  - MnS consumer outside the 3GPP management system but within the PLMN trust domain, e.g. SA2 NWDAF, Operator’s AFs (e.g., EAS). This is equivalent to internal MnS consumer X2 in Figure 3.2-1.

- External MnS consumer – MnS consumer outside the PLMN trust domain. There are two categories of external MnS consumers:

  - MnS consumers outside the PLMN trust domain consuming management services directly from the 3GPP management system, e.g., GSMA OP, TMF and verticals applications. This is equivalent to external MnS consumer Y1 in Figure 3.2-1.

  - MnS consumers outside the PLMN trust domain consuming management services indirectly through CAPIF. TS 28.579 specifies how to expose management services to this category of external MnS consumers, e.g., GSMA OP, TMF, verticals applications. This is equivalent to external MnS consumer Y2 in Figure 3.2-1.

##### Observation 2 Different access control rights for different types of MnS consumers

For MnS consumers consuming management services directly from the 3GPP management system (e.g., internal MnS consumer X1, internal MnS consumer X2 and external MnS consumer Y1), SA5 has defined a unified exposure mechanism to support the registration and discovery of management services and the corresponding management capabilities (Refer to MnSInfo and MnSRegistry IOCs in Ts 28.622). Further, in TS 28.319, SA5 has defined access control model to enable the exposure of management services to internal and external MnS consumers.

For MnS consumers consuming management services indirectly through CAPIF (e.g., external MnS consumer Y2), SA5 has specified how the management services can be exposed through CAPIF in TS 28.579.

# 4	Detailed proposal

The group is asked to endorse the following:

- For Rel-19, we proceed with the MSED naming convention. In Rel-20, we can re-examine if we need to rename MSED to refer to a function instead of a domain.

- The group is asked to endorse the definitions of internal and external MnS consumers in this DP and produce a CR to TS 28.533 in SA5#161 meeting to capture the concepts presented in clause 3.2.