3GPP TSG-SA5 Meeting #160	S5-251987

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.540 | CR | 0036 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.540 Add secure backhaul requirements for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Canada Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NTN_OAM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add new requirements for NTN management to support secure backhaul. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new requirements as recommended in TR 28.874. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NTN requirements would not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 4.10.2.x (new), 5.9.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[6]	3GPP TS 28.531: "Management and orchestration of 5G networks; Provisioning".

[7]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[8]	GSMA NG.116 – "Generic Network Slice Template" v1.0 (2019-05-23).

[9]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[x]	3GPP TS 28.314: “Plug and Connect; Concept and requirements”.

[y]	3GPP TS 28.315: “Plug and Connect; Procedure Flow”.

2nd change

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G Core Network: Defined in 3GPP TS 23.501 [2].

AMF Region: Defined in 3GPP TS 23.501 [2].

AMF Set: Defined in 3GPP TS 23.501 [2].

en-gNB: Defined in 3GPP TS 37.340 [5].

gNB: Defined in 3GPP TS 38.300 [3].

gNB Central Unit (gNB-CU): Defined in 3GPP TS 38.401 [4].

gNB-CU-Control Plane (gNB-CU-CP): Defined in 3GPP TS 38.401 [4].

gNB-CU-User Plane (gNB-CU-UP): Defined in 3GPP TS 38.401 [4].

gNB Distributed Unit (gNB-DU): Defined in 3GPP TS 38.401 [4].

ng-eNB: Defined in 3GPP TS 38.300 [3].

NG-RAN: Defined in 3GPP TS 23.501 [2].

RedCap UE: defined in 3GPP TS 38.331 [X].

eRedCap UE: defined in 3GPP TS 38.331 [X].

Plug and Connect: defined in 3GPP TS 28.314 [y].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core network

5GS	5G System

AMF	Access and Mobility Management Function

EN-DC	E-UTRA-NR Dual Connectivity

EPS	Evolved Packet System

MR-DC	Multi-Radio Dual Connectivity

NG-RAN	NG Radio Access Network

NR	New Radio

PCF	Policy Control Function

PnC	Plug and Connect

SeGW	Security Gateway

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

3rd change

## 4.10	Management of NTN

### 4.10.1	Management of transparent mode feature

Satellite Transparent Mode is a communication mode where a satellite functions as a simple relay or "bent-pipe" that merely forwards signals between the user equipment (UE) and the terrestrial network, without any signal processing or modification by the satellite. The 3GPP management system should provide the satellite ephemeris parameters to the gNB providing non-terrestrial access as outlined in TS 38.300 [3] to support management of NTN function. Satellite assistance information (e.g. ephemeris information) can be used for the handling of coverage holes or discontinuous satellite coverage in a power efficient way.

For a UE using a Non-Terrestrial Network that provides discontinuous coverage, 3GPP management system needs to support UE location verification, NTN access restriction and NTN mobility management.

### 4.10.2	Management of Backhaul feature

The AMF report the satellite backhaul category and indicates the satellite backhaul category change to SMF as outlined in TS 23.501 [2]. And the AMF can determine the Satellite backhaul category based on the information within Global RAN Node IDs associated with satellite backhaul. The 3GPP management system should provide a capability to allow the MnS consumer to configure satellite backhaul information for NTN node.

### 4.10.2.x	Management of secure NTN backhaul feature

For management of secure backhaul between NTN and SeGW see [x].

4th change

## 5.9	Requirements for management of NTN

### 5.9.1	Management of transparent mode feature

#### 5.9.1.1	Management of transparent mode feature for NG-RAN

REQ-NTN_TRANS_NRNRM-001: The NR NRM definitions should support management of NTN NG-RAN node.

REQ-NTN_TRANS_NRNRM-002: The NR NRM definitions should support NTN mobility management.

#### 5.9.1.2	Management of transparent mode feature for 5GC

REQ-NTN_TRANS_5GCNRM-001: The 5GC NRM definitions should support NTN UE location verification.

REQ-NTN_TRANS_5GCNRM-002: The 5GC NRM definitions should support NTN access restriction.

REQ-NTN_TRANS_5GCNRM-003: The 5GC NRM definitions should support NTN mobility management.

### 5.9.2	Management of Backhaul feature

REQ-NTN_BACKHAUL-001: The 5GC NRM definitions should support a capability to allow the MnS consumer to configure satellite backhaul information for NTN node.

#### 5.9.2.x  Management of secure NTN backhaul feature

For management of secure backhaul for NTN see [y].

### 5.9.3	Management of UE-Satellite-UE communication feature

REQ-NTN_USU_5GCNRM-001: The 5GC NRM definitions should support NTN UE-Satellite-UE communication feature.

### 5.9.4	Management of regenerative mode feature

REQ-NTN_REGEN-001: The NR NRM definitions shall have the capability to allow the MnS consumer to configure NTN neighbour cells considering the satellite movement.

End of changes