3GPP TSG-SA5 Meeting #160	S5-251989

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.315 | CR | 0002 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.315 Add secure backhaul for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Canada Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NTN_OAM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add NTN management to support secure backhaul. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add recommendation from TR 28.874. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NTN solution would not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

# 5.x	Management of NTN secure backhaul

In ground based terrestrial networks the connectivity between RAN nodes and SeGW is based on physical connectivity. As a result, the underlying IP network design seldom changes, and the logical connectivity between the RAN nodes and SeGW remains relatively stable.

However, in airborne Non-Terrestrial Networks (NTN) the connectivity between RAN nodes and SeGW is not stable since the RAN nodes are moving. E.g. a satellite in LEO, MEO or GEO orbit. A non-terrestrial node provides a service link and relies on a feeder link to communicate with other nodes comprising the NTN, including the SeGW.

As NTN nodes move the availability of terrestrial connectivity is subject to change. This has potential impact to the security associations between the NTN node, SeGW and CN.

Figure 5.x-1	Secure connectivity between non-terrestrial RAN node and terrestrial CN node(s)

As NTN nodes move their security associations should be updated subject to the availability of new terrestrial connectivity to a new SeGW. In some cases, such SeGW transitions may be able to be performed in anticipation of the upcoming feeder link update (i.e. "make then break") whereas in others the new connectivity may not be available in advance (i.e. "break then make"). Both scenarios should be supported, and different information may be required to setup and maintain the secure associations subject to which security protocols and features are configured.

Figure 5.x-2	Impact of feeder link switchover between NTN node, SeGW and CN

The IP configuration requires correlation with the state of connect/disconnect of the IP link itself. For example, for satellite-based NTN node the following may need to be considered:

- satellite may only ever connect to ground stations allowed by policy. As a result, when the satellite moves out of ground station's catchment location, it loses the ground connectivity via the feeder link and the IP transport during such period, and then reestablishes the IP connectivity.

- when the satellite orbit is passing over the earth surface (e.g. oceans, mountains, deserts, forests etc.) where there is no ground station and supporting infrastructure, the satellite may by-design route data over to Inter Satellite Link (ISL) to other satellites that may have connectivity with the ground infrastructure over a feeder link, but the ground infrastructure may reside in jurisdiction not permitted by regulations.

To further ensure the security associations are maintained, additional information should also be made available to the NTN node about the anticipated terrestrial connectivity based on criteria such as flight path and/or time windows.

In summary, movement of the NTN nodes mean the backhaul connection should traverse multiple feeder links and its security associations should be maintained throughout lifecycle phases of IP connectivity. As a result, NTN nodes require information not only to setup the initial secure communications channel, but to maintain such communications as the NTN moves.

End of changes