3GPP TSG-SA5 Meeting #160	S5-252034

Goteborg, Sweden, 07 - 11 April 2025 	Revision of S5-251248

Source:	Nokia

Title:	Pseudo-CR on Coordinating CCLs with other management functions

Document for:	Approval

Agenda item:	6.19.4.1

Spec:	3GPP TS28.567

Version:	0.1.0

Work Item:	Closed Control Loop Management

Comments

This pCR is to add the use case on Coordinating CCLs with other management functions as agreed in TR 28.867

Proposed Changes

* * * First Change * * * *

# 5	Management Capabilities

## 5.D	 CCL Coordination – COORD

### 5.D.1	Description

This use case related to the coordination of CCL, e.g. with other management capabilities

### 5.D.2	Use Cases

## 5.D.2.1	Coordinating CCLs with other management functions – COORD_01

A CCL can make and execute decisions in different network contexts and for different network functions and parameters. Yet within the network, there may be other management functions or features including MDA functions, SON functions, and AIML Functions, which also make decisions that affect the same network functions and parameters as the CCL. The operation of CCLs needs to be coordinated with the other management functions.

NOTE: This use-case only focuses on coordinating CCLs with other management functions for executing decisions.

For a given context, the CCL should indicate the set of network functions and corresponding parameters which it is interested in changing. Accordingly, the MnS consumer, say responsible for coordinating the CCLs with management functions may subscribe to be notified of changes on network functions and parameters. The MnS consumer should be able to inform the CCL of the latest changes to a network function or its parameter and a management entity/function (e.g. CCL, MDA, SON, AI/ML inference Function) responsible for the change to the parameter.

The CCL may want to obtain the history of previous values of the parameter. The history includes, for each previous value, the identifier of a respective management entity/function responsible for that change to the parameter. The CCL may define a favourable range of values of the parameter based on the received information on the latest change and the history of previous changes to the parameter. The CCL can calculate a new value of the parameter considering the favourable range as a constraint for the new value. The CCL needs then to update the value of the parameter of the network function to the new value.

Note:	The MnS consumer may for example be the functionality that is responsible for coordinating CCL and other management functions.

### 5.D.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-COORD-FNS-01 | The CCL MnS producer should have a capability to indicate to an MnS consumer the set of network functions including their parameters which it is interested in changing | COORD_01 |
| REQ-COORD-FNS-02 | The management system should have a capability enabling an authorized CCL instance acting as MnS consumer to receive information on the latest changes to a network function parameter and an identifier of a management entity/function including MDA Function, a SON Function or an AI/ML inference Function that responsible for the change to the parameter. | COORD_01 |
| REQ-COORD-FNS-03 | The management system should have a capability enabling an authorized MnS consumer to receive the history of previous values of the parameter, including, for each previous value, the identifier of a respective management entity/function responsible for that change to the parameter. | COORD_01 |

* * * End of Changes * * * *