3GPP TSG-SA5 Meeting #160	S5-251257

Goteborg, Sweden, 07 - 11 April 2025

Source:	Nokia

Title:	Pseudo-CR on NDT NRM

Document for:	Approval

Agenda item:	6.19.5.1

Spec:	3GPP TS28.561

Version:	0.1.0

Work Item:	Management aspects of Network Digital Twins

Comments

This pCR is to add NRM for the NDT  following agreements in the NDT study.

Proposed Changes

* * * First Change * * * *

# 6	Model

## 6.1	Imported and associated information entities

TBD

### 6.1.1	Imported information entities and local labels

TBD

### 6.1.2	Associated information entities and local labels

TBD

## 6.2	Class diagram

Figure 6.2.1.1-1: Relations for common information models for AI/ML management

#### 6.2.2	Inheritance

Figure 6.2.1.2-1: Inheritance Hierarchy for Closed Control Loops

### 6.3	Class definitions

#### 6.3.1	NetworkDigitalTwin

6.3.1.1	Definition

This IOC represents the Network Digital Twin. It represents the information for controlling and monitoring a NDT  associated. The NDT may contain one or more Simulation(s) and is capable of simulations for (and is thus associated with) one or more simulationScope(s).

6.3.1.2	Attributes

The NetworkDigitalTwin IOC includes attributes inherited from ManagedFunction IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.1.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedSimulationTypes | M | T | F | T | T |
| Attribute related to role |  |  |  |  |  |

6.3.1.3	Attribute constraints

None

6.3.1.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

##### 6.3.2	Simulation

6.3.2.1	Definition

This IOC represents a single simulation on an NDT .

A simulation is associated with a SimulationScope and a simulationReport in which the outcomes of the simulation are reported.

6.3.2.2	Attributes

The Simulation IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.2.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| simulationType | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| simulationScopeRefList | M | T | T | F | T |
| simulationReportRefList | M | T | F | F | T |

6.3.2.3	Attribute constraints

None.

6.3.2.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

##### 6.3.3	SimulationScope

6.3.3.1	Definition

This IOC represents a single SimulationScope on an NDT .

A SimulationScope is name-contained on a NetworkDigitalTwin which runs simulation over that SimulationScope. A SimulationScope may be associated with zero, one or more Simulation(s.

6.3.3.2	Attributes

The SimulationScope IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.3.2-1

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| scopeType | O | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| simulationRef | M | T | T | F | T |

6.3.3.3	Attribute constraints

None

6.3.3.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

##### 6.3.4	SimulationReport

6.3.3.1	Definition

This IOC represents a single SimulationReport on an NDT .

A SimulationReport is name-contained on a NetworkDigitalTwin which runs simulation and reports on those simulations through the SimulationReport.

6.3.4.2	Attributes

The SimulationReport IOC includes attributes inherited from Top IOC (defined TS 28.622[5]) and the following attributes:

Table 6.3.4.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| Attributes related to role |  |  |  |  |  |
| simulationRef | M | T | T | F | T |

6.3.4.3	Attribute constraints

None

6.3.4.4	Notifications

None

6.3.5.4	Notifications

The common notifications defined in clauses 6.1 are valid for this IOC, without exceptions.

6.4	Attribute definitions

6.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| supportedSimulationTypes | It indicates the different types of simulations which the NDT is capable of undertaking. Candidate values include: “FAILURE_PREDICTION“, “FAULT_INJECTION“, “ML-TRAINING_DATA_GENERATION“, “RISKY-ACTIONS_PREDICTION“, “EVENTS-IMPACTS_VERIFICATION“, “NETWORK_CONFIGURATIONS_VERIFICATION“, “AUTOMATION_CONFIGURATION_VERIFICATION“ "LOCKED", "UNLOCKED". The meaning of these values is as described in clause 5. The detailed implementation is out of scope of the presenting document. The list can be extended with vendor specific values. | type: String multiplicity:1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| simulationType | It indicates the type of simulation that is desired to be executed. allowedValues: “FAILURE_PREDICTION“, “FAULT_INJECTION“, “ML-TRAINING_DATA_GENERATION“, “RISKY-ACTIONS_PREDICTION“, “EVENTS-IMPACTS_VERIFICATION“, “NETWORK_CONFIGURATIONS_VERIFICATION“, “AUTOMATION_CONFIGURATION_VERIFICATION“ "LOCKED", "UNLOCKED". The meaning of these values is as described in clause 5. The detailed implementation is out of scope of the presenting document. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| simulationRef | It indicates an DN of a Simulation Instance. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| simulationScopeRefList | It indicates a list of DN for SimulationScope Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| simulationReportRefList | It indicates a list of DN for SimulationReport Instances. allowedValues: N/A | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * Next Change * * * *

Annex A (informative):
UML code for model diagrams

# A.X	UML code for NDT management model diagrams

This annex contains the PlantUML source code for the NRM diagrams defined in clause 6.2 of the present document.

# A.X.1	NDT NRM fragment (Figure 6.2.1.1-1)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class ManagedEntity <<ProxyClass>>

class NetworkDigitalTwin <<InformationObjectClass>>

class Simulation <<InformationObjectClass>>

class SimulationReport <<InformationObjectClass>>

class SimulationScope <<InformationObjectClass>>

ManagedEntity "1" *-- "*" NetworkDigitalTwin: <<names>>

NetworkDigitalTwin "1" *-- "1..*" Simulation: <<names>>

NetworkDigitalTwin "1" *-- "1..*" SimulationScope: <<names>>

NetworkDigitalTwin "1" *-- "1..*" SimulationReport: <<names>>

Simulation "1" -r-> "1..*" SimulationReport

SimulationScope "1..*" -r-> "1" Simulation

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedElement

end note

@enduml

# A.X.2	NDT inheritance relationships (Figure 6.2.1.1-2)

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

class Top << InformationObjectClass >>

class ManagedFunction << InformationObjectClass >>

class NetworkDigitalTwin <<InformationObjectClass>>

class Simulation <<InformationObjectClass>>

class SimulationReport <<InformationObjectClass>>

class SimulationScope <<InformationObjectClass>>

ManagedFunction <|-- NetworkDigitalTwin

Top <|-- Simulation

Top <|-- SimulationScope

Top <|-- SimulationReport

@enduml

* * * End of Changes * * * *