3GPP TSG-SA5 Meeting #160	S5-251275

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0309 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.312 Add procedure for intent exploration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The use case, requirements and solution for intent exploration are documented in TS 28.312. However, the procedure for intent exploration is missing. It proposes to add procedure for intent exploration in clause 6.3 based on the procedure in TR 28.914. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for intent exploration in clause 6.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.X(new), Annex.1.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.3.X	Intent exploration

Figure 6.3.X-1 illustrates the procedure for intent exploration.

Figure 6.3.X-1: Procedure for intent exploration

1.	MnS Consumer sends a request to create an intent instance (see createMOI operation defined in TS 28.532 [3]) to MnS Producer, specifying the intentMgmtPurpose as "EXPLORE" and providing intent information for the new intent to be created. The detailed intent information sees attributes (attribute which "isWritable" property is "True") of the concrete intent IOC defined in clause 6.2. The value of specific intent targets or contexts that need to be explored is initially set to 'NULL', indicating that the MnS consumer is seeking the best value for these target(s) and/or context(s).

2.	Based on the received request, the MnS Producer creates the concrete intent instance (i.e., instance of intent IOC) and configure the new created intent MOI with the received intent information.

3.	MnS Producer sends a response (see createMOI operation defined in TS 28.532[3]) to the MnS Consumer with attribute "objectInstance" of the created intent instance.

4.	Based on the created intent instance with intentMgmtPurpose as "EXPLORE", MnS Producer performs the exploration process by evaluating and determining the best possible values for the specified targets and/or contexts, considering current resource situation and capabilities of the system. The expected network will not act on the intent exploration request.

5.	MnS Producer should notify (see notifyMOIAttributeValueChanges notification) MnS Consumer about attribute "objectInstance" of intent report instance and corresponding intent exploration report information that contains targetExplorationResults and/or contextExplorationResults which are required by the MnS Consumer.

Next Change

A.1.X	Intent exploration

@startuml

title "[Intent Exploration]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

Collections "NE(s)" as ManagedEntity

MnS_Consumer -> MnS_Producer: 1. Request to create an intent instance (intentMgmtPurpose=EXPLORE, list of attributes of intent IOC, \n    value of specific intent target(s) and/or context(s) to be explored = 'NULL')

MnS_Producer -> MnS_Producer: 2. Create and configure intent MOI

MnS_Producer -> MnS_Consumer: 3. Response for create an intent instance

MnS_Producer -> MnS_Producer: 4. Perform the exploration for the best value of specified target(s) and/or context(s)

MnS_Producer -> MnS_Consumer: 5. Notify of intent exploration report (DN of IntentReport MOI, ExpectationExplorationResult )

hide footbox

@enduml