3GPP TSG-SA5 Meeting #160	S5-251338

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0681 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.552 Add a new measurement related to transmission rate related to the data collection from NFs performed by NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NWDAF_OAM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-3-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For each of the NF services producers listed in table 6.2.2.1-1 in TS 23.288, the data can be provided to a NWDAF by the notification corresponding to the subscription of Event Exposure Service or by the response corresponding to the request to the other services offered by the data source NF. The amount of data collected by NWDAF from each notification and/or response varies. Since the NWDAF and the data sources NFs may not be deployed close to each other from transmission point of view, the time used for transmission from the difference NFs in 5GC to the NWDAF varies, too. Therefore, the transmission rate can be used to evaluate the NWDAF data collection efficiency. For a given time length during the observation, the transmission rate can be measured as the ratio between the total amount of data collected by NWDAF and the corresponding accumulated time used for transmitting those amount of data from the data source NF to the NWDAF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new measurement “Transmission rate related to the data collection from NFs performed by NWDAF” is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator will not be able to monitor the transmission rate of NWDAF performing data collection from NFs and to take management actions for configuration and resource allocation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

5.18	Performance measurements for NWDAF

5.18.5	Measurements related to the NWDAF Data Collection

5.18.5.x Transmission rate of NWDAF performing data collection from NFs

a) This measurement provides the transmission rate related to the data collection from NFs performed by NWDAF

b)	CC

c)	This measurement is provided by averaging the total amount of data collected from one or more NF(s) with the accumulation of the transmission time of each notification and response corresponding to the subscription and request for NWDAF to perform data collection from NFs.

d)  A real value

e)	The measurement name has the form DANS.DataCollectTransmissionRate.withoutDCCF.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

End of Changes