3GPP TSG-SA5 Meeting #160	S5-251467

Goteborg, SWEDEN 7 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0058 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.254 Addition of the CAPIF Charging Architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_CAPIF_EXT |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In order to support API charging for the industry, the Common API Framework is specifed by SA6 to support any service APIs when used by northbound entities, as well as any interactions between the CAPIF and the service APIs themselves. The corresponding charging support should be enhanced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add reference to the SA6 and CT3 specifications about architecture aspect of CAPIF and service operations. Add abbreviations for CAPIF Charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CAPIF service is not supported from the charging aspect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 4.1, 4.X(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2] – [49]	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application".

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[53]	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS): Applications and interfaces".

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55] – [56]	Void.

[57]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[58]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[59] – [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101] – [199]	Void

[200]	3GPP TS 23.501:"System Architecture for the 5G System".

[201]	3GPP TS 23.502:"Procedures for the 5G System".

[202] – [229]	Void

[230]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[231] – [2235]	Void

[236]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[237] – [242]	Void

[243]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[244]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[245]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs".

[246] – [299]	Void

[300] – [399]	Void.

Next Change

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [100] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [100].

AI/ML	Artificial Intelligence/Machine Learning

AEF	API Exposing Function

AF	Application Function

AMF	API Management Function

APF	API Publishing Function

API	Application Program Interface

AS	Application Server

BD	Billing Domain

CAPIF	Common API Framework

CCF	CAPIF Core Function

CDF	Charging Data Function

CGF	Charging Gateway Function

CTF	Charging Trigger Function

ECUR	Event Charging with Unit Reservation

IE	Information Element

IEC	Immediate Event Charging

NEF	Network Exposure Function

PEC	Post Event Charging

SCEF	Service Capability Exposure Function

SCS	Services Capability Server

SGSN	Serving GPRS Support Node

RCAF	RAN Congestion Awareness Function

Next Change

4.1	High-level Service Exposure Function architecture

The overall architecture and detailed description for Service Capability Exposure which enables the 3GPP network to securely expose its services and capabilities provided by 3GPP network interfaces to external 3rd party service provider applications are defined in 3GPP TS 23.682 [243], and figure 4.1.1 is highlighting T8 involved for interworking between SCEF and SCS/AS.

Figure 4.1.1: Northbound API of 3GPP Architecture Service Capability Exposure

The overall architecture and detailed description for Common API framework (CAPIF) for 3GPP northbound APIs that are applicable to any service APIs when used by northbound entities, as well as any interactions between the CAPIF and the service APIs themselves are defined in 3GPP TS 23.222 [244]. The CAPIF applies to both EPS and 5GS, can be hosted within a PLMN or SNPN, and is independent of the underlying 3GPP access (e.g. E-UTRA, NR).The figure 4.1.2 about the functional model for the CAPIF is present.

Figure 4.1.2: Functional model for the CAPIF

Next Change

4.X	Common API Framework converged charging architecture

As per the caluse 6.3.3 in TS 23.222 [244], the CAPIF Core function (CCF) in CAPIF is responsible for collecting the logs of the service API invocations for charging purpose. The CCF embedded CTF generate charging events and forwards them to the CHF.The architectural options for CAPIF Core Funtion (CCF) converged charging are depicted in figure 4.X.1 in service-based representation:

Figure 4.X.1: Common API Framework converged charging architecture

Details on the interfaces and functions can be found in TS 32.240 [1] for the general architecture components, Ga is described in clause 5.2.4 and Bea in clause 5.2.5 of this document, and Nchf is described in TS 32.290 [57].

Figure 4.x.2 depicts the CAPIF Core Funtion (CCF) converged charging architecture for non-roaming in reference point representation:

Figure 4.X.2: Common API Framework converged charging architecture non-roaming reference point representation

End of Change