3GPP TSG-SA5 Meeting #160	S5-251484

Goteborg, Sweden, 07 - 11 April 2025

# Source:	Huawei

# Title:	Discussion paper on management services exposure

# Document for:	Endorsement

Agenda Item:	6.19.21.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		3GPP TR 28.879: Study on OAM for service management and exposure to external consumers

[2]		3GPP TS 28.533: Architecture framework

[3]		S5-250295 Reply-LS on Internal 5G Core information expose to trusted AF

[4]		3GPP TS 33.501: Security architecture and procedures for 5G system

[5]		3GPP TS 23.501: System architecture for the 5G System (5GS)

[6]		3GPP TS 23.502: Procedures for the 5G System (5GS)

[7]		3GPP TS 29.522: Network Exposure Function Northbound APIs; Stage 3

[8]		3GPP TS 29.122: T8 reference point for Northbound APIs

[9]		3GPP TS 28.622: Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

[10]	3GPP TS 28.537: Management capabilities

[11]	3GPP TS 28.319: Access Control for Management services

# 3	Rationale

Regarding the management services exposure, this discussion paper lists some observations and proposes way forward for endorsement.

In the context of 3GPP management services exposure, there are two kinds of MnS consumers as observation 1 below:

##### Observation 1	Two kinds of MnS consumers

- Internal MnS consumer – within 3GPP operator domain

  - MnS consumer within 3GPP management system, e.g. SA5 Provisioning MnS consumer, SA5 Fault management MnS consumer

  - MnS consumer outside 3GPP management system but within the 3GPP operator domain, e.g. SA2 NWDAF, SA6 SEAL, VAE, Edge Services from operator

- External MnS consumer - outside 3GPP operator domain

  - MnS consumers outside 3GPP operator domain, e.g. GSMA OP, CAMARA, TMF

In TR 28.879 [1], figure 4.1.1-1 illustrates the example of two kinds of MnS consumers as below:

In TS 28.533 [2], clause 5.5 specifies an architecture reference model using Management Services to support multiple players interoperability, see below figure, which illustrates an example showing the MnSs may be used by PLMM Organizations (e.g., Operator A, Operator B) and enterprise customers (e.g., Vertical). The interoperability in a multiple players scenario supported by using management services includes:

a)	Interoperability within systems of a single PLMN Organisation

b)	Interoperability between systems belonging to different organizations

If we consider the MnS consumer outside 3GPP management system but within 3GPP operator domain, e.g. SA2 NWDAF, SA6 SEAL/VAE/Edge Services, the above figure may be enhanced like the following figure:

##### Observation 2	Different exposure security requirements for the two kinds of MnS consumers

According to the LS from SA3, see [3], "As far as information exposure to an AF is of concerns, it is allowed to expose “Internal 5G Core information” to an AF within the operator’s domain. Besides, it is also possible to expose internal 5G Core information to an AF outside the operator’s domain after proper security protection or privacy protection. For example, an “S-NSSAI” can be mapped to an AF-Service-Identifier before exposing to an AF outside the 3GPP operator domain (i.e., the “S-NSSAI” shall not be exposed directly in this case to be in line with the security requirements specified in the TS 33.501)".

- Therefore, for the external MnS consumer, which is outside 3GPP operator domain, the “S-NSSAI” shall not be exposed directly in this case to be in line with the security requirements specified in the TS 33.501 [4].

- This S-NSSAI mapping work has been done in SA2 and CT groups for the stage 2 and stage 3 work, see TS 23.501 [5], TS 23.502 [6], TS 29.522 [7] and TS 29.122 [8]. For example, in TS 23.501 [5], one of NEF functionality is following:

  - -	Translation of internal-external information:

    - It translates between information exchanged with the AF and information exchanged with the internal network function. For example, it translates between an AF-Service-Identifier and internal 5G Core information such as DNN, S-NSSAI, as described in clause 5.6.7 of TS 23.501.

    - In TS 29.522 [7], under the clause 4.4 Procedures over NEF Northbound Interface,

      - 4.4.2	Procedures for Monitoring

- Similarly, the translation of internal-external information shall be done as one of EGMF functionality defined in SA5 to fulfil operators' security requirements.

##### Observation 3	SA5 DMS + MSAC solutions for Internal MnS consumer

3GPP SA5 has defined a unified management capabilities registration/discovery/AC mechanism as below for internal MnS consumers:

In TS 28.622 [9]: IOC MnSInfo represents an available Management Service (MnS) and provides the data required to support its discovery.  It is name-contained by MnsRegistry.

TS 28.537 [10] specifies use cases, requirements and procedures for management capabilities of 5G networks. To enable communication between MnS Consumers and MnS Producers, MnS Consumers need mechanisms to discover management service information available in the 3GPP management system, and their management capabilities. To this end, MnS Producers and their management capabilities need to be exposed in the 3GPP management system. The requirements of Discovery of Management Services (DMS) are captured in TS 28.537 [10] clause 5.2.2 as below:

TS 28.319 [11] specifies access control for management services . See figure 4-2 (copy as below) of TS 28.319 [11] which shows MnS consumers belonging to different domains (e.g. NOP domain and Vertical domain) are assigned to different roles respectively. For each role, there is an associated access rule list which shows the allowed accessing scope of MnSs. MnS 1, MnS 2 and MnS 3 are different with each other in at least one of the component A, component B and component C.

##### Observation 4	Overview of 3GPP management services exposure

For Rel-19 Enhanced OAM for management exposure to external consumers WI, the scope is management services exposure to external consumers through CAPIF. Considering the above observation 1, 2 and 3, it is proposed that for exposing MnS:

- For MnS consumers within operator’s domain

  - Using DMS + MSAC solutions

- For MnS consumers outside operator’s domain

  - Using CAPIF solution

- Capture this whole picture as below in one place, such as in TS 28.533 [2]

# 4	Detailed proposal

It is proposed to endorse the following:

To expose 3GPP MnS:

- For MnS consumers within operator’s domain

  - Using DMS + MSAC solutions

- For MnS consumers outside operator’s domain

  - Using CAPIF solution

- Capture this whole picture as below in one place, such as in TS 28.533 [2]