3GPP TSG-SA5 Meeting #160	S5-251491

Goteborg, Sweden, 07 - 11 April 2025

# Source:	China Mobile, Huawei

# Title:	TS28.561 NDT support for network automation

# Document for:	Approval

Agenda Item:	6.19.5.1

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

None.

3	Rationale

Update the NDT support to automation in clause 5.2 and add the corresponding requirements.

4	Detailed proposal

Start of Change

5.2	NDT support for network automation

5.2.1	Description

NDTs can be used to support automation use cases (e.g. MDA, SON, etc.). An NDT may be integrated into a network automation function, or it may be external to the network automation function. In the case where the NDT is external to the network automation function, it could be possible for the network automation function to configure the NDT and the scenario that could be modeled by the NDT. NDTs do not execute any actions in form of configuration of the live network but support decision-making.

5.2.2	Use cases

5.2.2.1	General capabilities on NDT support for network automation

An NDT, depending upon the network or service management use case and scenario to be modeled and simulated, might need data originating from various sources (network data, environment data, analytic, UEs data) and suitable hardware/software resources to function properly. MnS consumers would need to specify needed NDT characteristics or configurations to the NDT tailored to fulfil consumer specific needs i.e. to define the consumer preference for the specific NDT. For example, consumer preferences may be related to environment data sources e.g. weather, synthetic data etc, data characteristics (e.g. robustness, data granularity, maximum tolerable latency), required NDT output latency, characteristics of the service to be twinned, resource constraints (HW/SW), etc. Furthermore, in the case that consumer's preference on NDT characteristics or configuration may change over time and MnS consumer may update the NDT with the needed changes.

The performance achieved by NDT may depend on NDT characteristics, such as network load and time. The simulation results would change based on the network load, the target time of the day, energy saving state, and whether the simulated node is running in energy saving mode.

The automation scenarios that the NDT might support are described in following sub-clauses.

5.2.2.2	Network failure and risk prediction

Each operations for network optimization and maintenance on mobile network may cause potential network failures and risks, especially high-risk operations, such as potentially dangerous configuration modification, policy modification, software version upgrade, and board switching, which may cause network congestion and network breakdown. Operators cannot carry out the high-risk network operations or perform direct optimization in the physical network. Therefore, utilization of NDT is a better way for evaluation of high-risk network operations.

Using NDT, operations causing potential network failures can be identified. NDT can also be utilized for optimization and verification of network policies and solutions for the further risk avoidance. After simulating/emulating a scenario, NDT can report the results back to the MnS consumer. Based on results, the MnS consumer can generate possible network policies and solutions to minimize the impact of high-risk operations.

As an example, the NDT can predict SLA degradation and single node failures in mobile networks. When the prediction indicates insufficient network resources to maintain SLA compliance or potential hardware resource failures, the NDT results help operators take appropriate preventive actions to avoid network outages..

5.2.2.3	Support to prediction of failure events including Signaling storm

The NDT can be used to support the failure event refers to the situation where users in the network are completely unable to get service from the network. A typical example of a failure event is the signaling storm, which is the situation where a large number of signaling messages suddenly surge in the mobile communication network, overloading the network's processing capacity and consequently degrading the network performance and stability. During this period, users will repeatedly try to establish the connection until reconnected, thus generating a large number of signaling messages surge suddenly, causing signaling storm. When a signaling storm occurs on the network, network Functions (NFs) such as AMF, SMF, and UDM in the 5G network might all be potential impact points. Therefore, when received the request with network object information from MnS consumer for signaling storm analysis, the NDT can execute the modelling and validation based on network object related information (network performance, KPIs, S-NSSAIs, etc.), which synchronized from the network resources, and provide the report with the modelling results to MnS consumer. The NDT can be used to validate whether the proposed solution based on the analysing of failure events can resolve, predict and prevent the signaling storm issue.

The NDT can model network behaviors and provide insights into the potential impacts of network failures, including surges in signaling requests. For instance, in case of the signaling storm, the information can enable identification of optimal flow control parameters for each signaling impact point.

5.2.2.4	Network issue inducement capability

This use case describes how a network issue can be induced by using NDT. In order to develop a resilient network, the behavior and performance of the network should be monitored during certain network failure issue, e.g. node/functionality failure, service degradation etc. In order to plan for the optimal network configuration in case of such network failure issue, the scenario itself need to be induced in the network. It is desirable, to use NDT for such an inducement process. A particular issue can be induced in the NDT instead of real network. After a particular issue is induced, the performance of the network can be monitored, other degradation/faults/failure can be identified and the mitigation actions can be decided. The following are some of the examples of the issues that can be induced.

The network slice performance degradation in terms of low PDU session establishment success rate or in terms of high latency can be induced, in a NDT, to see how the related network functions will behave when the PDU session establishment success rate is degraded. The remedial actions can be decided to mitigate the problem.

The coverage hole can be induced, in a NDT, to see how the related services are getting effected. The remedial actions can be decided to mitigate the problems arising due to the induced coverage hole.

NDT can be used for fault injection experiments avoiding impact on the physical network while measuring and monitoring the impact of each injected fault in the NDT simulation. This could be leveraged, to build a training data set for enhancing and enriching detection and diagnosing systems capabilities. In addition, NDT could be leveraged for improving root causes analysis.

5.2.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NDTAUTO-01: | The management system should support a capability for an authorized NDT MnS consumer to configure the parameters and characteristics of the network scenario that should be modelled. | General capabilities on NDT support for network automation (Clause 5.2.2.1) |
| REQ-NDTAUTO-02: | The management system should support a capability for an authorized NDT MnS consumer to request getting report/output from NDT. | General capabilities on NDT support for network automation (Clause 5.2.2.1) |
| REQ-NDTAUTO-03: | The management system should support a capability for an authorized NDT MnS consumer to request simulation/emulation of operations that could cause network failure from NDT. | Network failure and risk prediction (Clause 5.2.2.2) |
| REQ-NDTAUTO-04: | The management system should support a capability for an authorized NDT MnS consumer to configure the network scenario to be modelled for the consumer to evaluate a network failure from NDT. | Network failure and risk prediction (Clause 5.2.2.2) |
| REQ-NDTAUTO-05: | The management system should support a capability for an authorized MnS consumer to define a network configuration to be modelled for checking the network response to a signaling storm from NDT. | Support to prediction of failure events including Signaling storm (Clause 5.2.2.3) |
| REQ-NDTAUTO-06: | The NDT should have the capability to report potential behaviour of a network facing a failure event including a signaling storm. | Support to prediction of failure events including Signaling storm (Clause 5.2.2.3) |
| REQ-NDTAUTO-07: | The NDT should have the capability to provide a report/output for NDT simulation/emulation achieving certain target performance threshold and QoS. | Network issue inducement capability (Clause 5.2.2.4) |
| REQ-NDTAUTO-08: | The management system should support a capability for an authorized NDT MnS consumer to measure and monitor the impact of the injected issue in a simulation environment from NDT. | Network issue inducement capability (Clause 5.2.2.4) |
| REQ-NDTAUTO-9: | The management system should have a capability to provide a report on topology data used for simulating and/or emulating network failures in NDT. | Network issue inducement capability (Clause 5.2.2.4) |
| REQ-NDTAUTO-10: | The NDT should support a capability enabling a network issue (e.g. fault/failure) to be induced in the NDT. | Network issue inducement capability (Clause 5.2.2.4) |