3GPP TSG-SA5 Meeting #160	S5-251518

Goteborg, Sweden, 7 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.271 | CR | 0037 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.271 Addition of charging principles for LCS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of charging principles for LCS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removal of limitations to ranging and sidelink positioning charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The charging principles will be unspecified for LCS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.4.1.1, and 5.4.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 5.1	LCS charging principles

Charging information in the Service domain for LCS is collected for inter-operator charging purpose by the GMLC. The basic principle is that a network requesting location information may be charged by the network that provides the location information.

The GMLC shall collect the following charging information:

-	Identity of the mobile subscriber to be located and of the entity requesting the location;

-	Identity of the GMLC or PLMN serving the LCS Client;

-	QoS Requested/Delivered: the charging information shall describe the quality of the location requested and delivered to the LCS client;

-	Request Timestamp: the charging information shall record the date and time the location procedure was requested by the LCS client;

-	Location services requested: the charging information shall describe the service types for which the LCS client is allowed to locate the particular UE;

-	Usage of continuous/periodic tracking;

-	Charging for Location Based Services (LBS): the charging information shall describe the service specific information in addition to the above location resource information.

The information listed above is captured for use cases in relation to:

-	Mobile Originated Location Request (MO-LR)

-	Sidelink Mobile Originating Location Request (SL-MO-LR)

-	Mobile Terminated Location Request (MT-LR)

-	Sidelink Mobile Terminating Location Request (SL-MT-LR)

-	Network Induced Location Request (NI-LR)

Refer to TS 23.271 [201], TS 23.273 [202], and TS 23.586 [206] for further details on the above LCS transactions.

* * * Next Change * * * *

#### 5.4.1.1	General

Converged charging may be performed by the GMLC interacting with CHF using Nchf specified in TS 32.290 [55] and TS 32.291 [56]. In order to provide the data required for the charging activities outlined in TS 32.240 [1] (Credit-Control, accounting, billing, statistics etc.).

The Charging Data Request and Charging Data Response are exchanged between the GMLC and the CHF, based on PEC scenarios specified in TS 32.290 [55]. The Charging Data Request is issued by the GMLC towards the CHF when certain conditions (chargeable events) are met.

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in the following clauses.

A detailed formal description of the converged charging parameters defined in the present document is to be found in TS 32.291 [56].

A detailed formal description of the CDR parameters defined in the present document is to be found in TS 32.298 [51].

* * * Next Change * * * *

##### 5.4.1.2.1	General

When a charging event is issued towards the CHF, it includes details such as Subscriber identifier (e.g. SUPI).

Each trigger condition (i.e. chargeable event) defined for the LCS converged charging functionality, is specified with the associated behaviour when they are met.

Table 5.4.1.2.1.1 summarizes the set of default trigger conditions and their category. For "immediate report" category, the table also provides the corresponding Charging Data Request message sent from GMLC towards the CHF.

Table 5.4.1.2.1.1: Default Trigger conditions in GMLC

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Receiving a provide location response | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |
| Sending a location update response | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |
| Sending a LCS Service response | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] |

* * * End of Changes * * * *