3GPP TSG-SA5 Meeting #160	S5-251585

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.540 | CR | 0036 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.540 Add secure backhaul requirements for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Canada Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NTN_OAM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add new requirements for NTN management to support secure backhaul |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new requirements as recommended in TR 28.874. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NTN requirements would not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 5.9.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage 2".

[6]	3GPP TS 28.531: "Management and orchestration of 5G networks; Provisioning".

[7]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[8]	GSMA NG.116 – "Generic Network Slice Template" v1.0 (2019-05-23).

[9]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[10]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[y]	3GPP TS 28.314: “Plug and Connect; Concept and requirements”.

2nd change

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G Core Network: Defined in 3GPP TS 23.501 [2].

AMF Region: Defined in 3GPP TS 23.501 [2].

AMF Set: Defined in 3GPP TS 23.501 [2].

en-gNB: Defined in 3GPP TS 37.340 [5].

gNB: Defined in 3GPP TS 38.300 [3].

gNB Central Unit (gNB-CU): Defined in 3GPP TS 38.401 [4].

gNB-CU-Control Plane (gNB-CU-CP): Defined in 3GPP TS 38.401 [4].

gNB-CU-User Plane (gNB-CU-UP): Defined in 3GPP TS 38.401 [4].

gNB Distributed Unit (gNB-DU): Defined in 3GPP TS 38.401 [4].

ng-eNB: Defined in 3GPP TS 38.300 [3].

NG-RAN: Defined in 3GPP TS 23.501 [2].

RedCap UE: defined in 3GPP TS 38.331 [X].

eRedCap UE: defined in 3GPP TS 38.331 [X].

Plug and Connect: defined in 3GPP TS 28.314 [y].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core network

5GS	5G System

AMF	Access and Mobility Management Function

EN-DC	E-UTRA-NR Dual Connectivity

EPS	Evolved Packet System

MR-DC	Multi-Radio Dual Connectivity

NG-RAN	NG Radio Access Network

NR	New Radio

PCF	Policy Control Function

PnC	Plug and Connect

SeGW	Security Gateway

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

3rd change

## 4.10	Management of NTN

### 4.10.1	Management of transparent mode feature

Satellite Transparent Mode is a communication mode where a satellite functions as a simple relay or "bent-pipe" that merely forwards signals between the user equipment (UE) and the terrestrial network, without any signal processing or modification by the satellite. The 3GPP management system should provide the satellite ephemeris parameters to the gNB providing non-terrestrial access as outlined in TS 38.300 [3] to support management of NTN function. Satellite assistance information (e.g. ephemeris information) can be used for the handling of coverage holes or discontinuous satellite coverage in a power efficient way.

For a UE using a Non-Terrestrial Network that provides discontinuous coverage, 3GPP management system needs to support UE location verification, NTN access restriction and NTN mobility management.

### 4.10.2	Management of Backhaul feature

The AMF report the satellite backhaul category and indicates the satellite backhaul category change to SMF as outlined in TS 23.501 [2]. And the AMF can determine the Satellite backhaul category based on the information within Global RAN Node IDs associated with satellite backhaul. The 3GPP management system should provide a capability to allow the MnS consumer to configure satellite backhaul information for NTN node.

### 4.10.2.x	Management of secure backhaul feature

In a Radio Access Network (RAN) gNodeB are deployed in cell sites which are typically part of an untrusted network domain. By contrast, core network (CN) nodes are deployed in a trusted domain. These two network domains are logically and physically separated by means of Security Gateways (SeGW) as defined in 3GPP TS 23.501 [1]. The communication between the domains, including the network management traffic, should be secured and is carried over a logical connection referred to as the 'backhaul'.

In ground based terrestrial networks the connectivity between RAN nodes and SeGW is based on physical connectivity. As a result, the underlying IP network design seldom changes, and the logical connectivity between the RAN nodes and SeGW remains relatively stable.

However, in airborne Non-Terrestrial Networks (NTN) the connectivity between RAN nodes and SEGW is not stable since the RAN nodes are moving. E.g. a satellite in LEO, MEO or GEO orbit. A non-terrestrial node provides a service link and relies on a feeder link to communicate with other nodes comprising the NTN, including the SeGW.

As NTN nodes move the availability of terrestrial connectivity is subject to change. This has potential impact to the security associations between the NTN node, SeGW and CN.

Figure 4.10.x-1	Secure connectivity between non-terrestrial RAN node and terrestrial CN node(s)

As NTN nodes move their security associations should be updated subject to the availability of new terrestrial connectivity to a new SeGW. In some cases, such SeGW transitions may be able to be performed in anticipation of the upcoming feeder link update (i.e. "make then break") whereas in others the new connectivity may not be available in advance (i.e. "break then make"). Both scenarios should be supported, and different information may be required to setup and maintain the secure associations subject to which security protocols and features are configured.

Figure 4.10.x-2	Impact of feeder link switchover between NTN node, SeGW and CN

The IP configuration requires correlation with the state of connect/disconnect of the IP link itself. For example, for satellite-based NTN node the following may need to be considered:

- satellite may only ever connect to ground stations allowed by policy. As a result, when the satellite moves out of ground station's catchment location, it loses the ground connectivity via the feeder link and the IP transport during such period, and then reestablishes the IP connectivity.

- when the satellite orbit is passing over the earth surface (e.g. oceans, mountains, deserts, forests etc.) where there is no ground station and supporting infrastructure, the satellite may by-design route data over to Inter Satellite Link (ISL) to other satellites that may have connectivity with the ground infrastructure over a feeder link, but the ground infrastructure may reside in jurisdiction not permitted by regulations.

To further ensure the security associations are maintained, additional information should also be made available to the NTN node about the anticipated terrestrial connectivity based on criteria such as flight path and/or time windows.

In summary, movement of the NTN nodes mean the backhaul connection should traverse multiple feeder links and its security associations should be maintained throughout lifecycle phases of IP connectivity. As a result, NTN nodes require information not only to setup the initial secure communications channel, but to maintain such communications as the NTN moves.

4th change

## 5.9	Requirements for management of NTN

### 5.9.1	Management of transparent mode feature

#### 5.9.1.1	Management of transparent mode feature for NG-RAN

REQ-NTN_TRANS_NRNRM-001: The NR NRM definitions should support management of NTN NG-RAN node.

REQ-NTN_TRANS_NRNRM-002: The NR NRM definitions should support NTN mobility management.

#### 5.9.1.2	Management of transparent mode feature for 5GC

REQ-NTN_TRANS_5GCNRM-001: The 5GC NRM definitions should support NTN UE location verification.

REQ-NTN_TRANS_5GCNRM-002: The 5GC NRM definitions should support NTN access restriction.

REQ-NTN_TRANS_5GCNRM-003: The 5GC NRM definitions should support NTN mobility management.

### 5.9.2	Management of Backhaul feature

REQ-NTN_BACKHAUL-001: The 5GC NRM definitions should support a capability to allow the MnS consumer to configure satellite backhaul information for NTN node.

REQ-NTN-SEC-BH-001：IP configuration data shall be configured for the secure communications channel association between RAN nodes on-board satellite and SeGW.

REQ-NTN-SEC-BH-002：IP configuration data shall be updated when transitioning between terrestrial networks, to ensure the secure association between NTN node and SeGW.

REQ-NTN- SEC-BH-003：IP configuration data shall be maintained at NTN node independent of feeder link availability.

### 5.9.3	Management of UE-Satellite-UE communication feature

REQ-NTN_USU_5GCNRM-001: The 5GC NRM definitions should support NTN UE-Satellite-UE communication feature.

### 5.9.4	Management of regenerative mode feature

REQ-NTN_REGEN-001: The NR NRM definitions shall have the capability to allow the MnS consumer to configure NTN neighbour cells considering the satellite movement.

End of changes