3GPP TSG-SA5 Meeting #160	S5-251702

Goteborg, Sweden, 07 - 11 April 2025

# Source:	Nokia

# Title:	Rel-19 pCR TS 28.579 Add use case solution for logging the management service API invocations to the CCF

# Document for:	Approval

Agenda Item:	6.19.21.1

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[1] 3GPP TS 28.579, "Management services exposure to external consumers through CAPIF".

# 3	Rationale

This pCR proposes to add solution for logging the management service API invocations to the CCF to TS 28.579[1].

# 4	Detailed proposal

It is proposed to add the following use case solution for logging the management service API invocations to the CCF  to TS 28.579[1].

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]                       3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[Z]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; stage 3".

[Y]                       IETF RFC 7231: “Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content”.

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

Second Change

# 6 Solutions for exposing management services using CAPIF

## 6.Y Logging the management service API invocations to the CCF

### 6.Y.1 Introduction

To monitor the service API invocations by the external MnS consumers, the following steps shall occur:

- The creation of the service API invocation log(s) using the “CAPIF_Logging_API_Invocation_API” data type (see clause 8.7 of 3GPP TS 29.222 [Z]) by the AEF of the MSED.

- Sending the created service API invocation log(s) to the CCF via the CAPIF-3 interface by the AEF of the MSED.

Authorized consumers of the service API invocation logs (e.g., the AMF of the MSED and the charging function (CHF)) can access the logs for auditing and charging purposes.

Clause 6.Y.2 describes how the AEF of MSED can create the service invocation logs. Clause 6.Y.3 describes the service operations to be supported by the AEF of MSED to enable the logging of the service API invocations by the external MnS consumers.

### 6.Y.2	Creation of the service API invocation logs

Table 6.Y.2-1 presents the attributes of the InvocationLog data type (see clause 8.7.4.2.2 of TS 29.222 [Z]) and clarifies which attributes (of the InvocationLog data type) can be mapped from the published service API information (see clause 6.X.2). Refer to Table 8.7.4.2.2-1 of TS 29.222 [Z] for detailed information on the attributes of the InvocationLog data type (e.g., the attribute data type, presence indicator, cardinality, description and applicability information).

Table 6.Y.2-1: Mapping of CAPIF InvocationLog data type with the published service API information

| Attribute name | Attribute additional information | Equivalent published ServiceAPIDescription data type attribute/comments |
| --- | --- | --- |
| aefId | The data type of this attribute is defined as "string" and presence qualifier is defined as "M". | The AEF identifier provided by the CCF upon MSED registration. Corresponds to the “aefId” attribute of the AefProfile data type (see Table 6.X.2-2). |
| apiInvokerId | The data type of this attribute is defined as "string" and presence qualifier is defined as "M". | Provided by the external MnS consumer to the AEF during the service API invocation request via the CAPIF-2e interface. |
| logs | The data type of this attribute is defined as "array (Log)" and presence qualifier is defined as "M". | See Table 6.Y.2-2 |
| supportedFeatures | The data type of this attribute is defined as "SupportedFeatures" and presence qualifier is defined as "O". | N/A |

Table 6.Y.2-2 presents the attributes of the Log data type (see clause 8.7.4.2.3 of TS 29.222 [Z]) and clarifies which attributes (of the Log data type) can be mapped from the published service API information (see clause 6.X.2). Refer to table 8.7.4.2.3-1 of TS 29.222 [Z] for detailed information on the attributes of the Log data type (e.g., the attribute data type, presence indicator, cardinality, description and applicability information).

Table 6.Y.2-2: Mapping of CAPIF Log data type with the published service API information

| Attribute name | Attribute additional information | Equivalent published ServiceAPIDescription data type attribute/comments |
| --- | --- | --- |
| apiId | The data type of this attribute is defined as "string" and presence qualifier is defined as "M". | The id of the service API that was invoked by the external MnS consumer. This attribute corresponds to the “apiId” attribute of the serviceAPIDescription data type (see Table 6.X.2-1). |
| apiName | The data type of this attribute is defined as "string" and presence qualifier is defined as "M". | Name of the service API that was invoked. This attribute corresponds to the “apiName” attribute of the serviceAPIDescription data type (see Table 6.X.2-1). |
| apiVersion | The data type of this attribute is defined as "string" and presence qualifier is defined as "M". | Version of the service API that was invoked by the external MnS consumer. This attribute corresponds to the “apiVersion” attribute of the Version data type (see Table 6.X.2-3). |
| resourceName | The data type of this attribute is defined as "String" and presence qualifier is defined as "M". | The name of the resource that was invoked by the external MnS consumer. This attribute corresponds to the “resourceName” attribute of the Resource data type (see Table 6.X.2-4). |
| uri | The data type of this attribute is defined as "Uri" and presence qualifier is defined as "O". | The URI of the resource that was invoked by the external MnS consumer. This attribute corresponds to the “uri” attribute of the Resource data type (see Table 6.X.2-4). |
| protocol | The data type of this attribute is defined as "Protocol" and presence qualifier is defined as "M". | Represents the protocol used by the service API that was invoked by the external MnS consumer. This attribute corresponds to the “protocol;” attribute of the AefProfile data type (see Table 6.X.2-2). |
| operation | The data type of this attribute is defined as "Operation" and presence qualifier is defined as "C". | The HTTP method that was invoked on the resource by the external MnS consumer. Corresponds to one of the HTTP methods listed in the "operations" attribute of the Resource data type (see Table 6.X.2-4). |
| result | The data type of this attribute is defined as "string" and presence qualifier is defined as "M". | The result of the service API invocation request by the external MnS consumer at the AEF. Allowed values are the HTTP status codes defined in clause 6 of IETF RFC 7231[Y]. |
| invocationTime | The data type of this attribute is defined as "DateTime”, and presence qualifier is defined as "O". | Date and time at which the service API invocation request from the external MnS consumer is received at the AEF of the MSED. |
| invocationLatency | The data type of this attribute is defined as "DurationMs”, and presence qualifier is defined as "O". | The time interval between the reception of the service API invocation request and the sending of the service API invocation response at the AEF of the MSED. |
| inputParameters | The data type of this attribute is defined as "ANY TYPE" and presence qualifier is defined as "O". |  |
| OutputParameters | The data type of this attribute is defined as "ANY TYPE" and presence qualifier is defined as "O". |  |
| srcInterface | The data type of this attribute is defined as "InterfaceDescription" and presence qualifier is defined as "O". |  |
| destInterface | The data type of this attribute is defined as "InterfaceDescription" and presence qualifier is defined as "O". |  |
| fwdInterface | The data type of this attribute is defined as "string" and presence qualifier is defined as "O". |  |

### 6.Y.3 Service operation supported by the AEF of MSED to enable the logging functionality

To log the service API invocations (from clause 6.Y.2) to the CCF, the AEF of MSED invokes the Log_API_Invocation_API service operation as described in clause 5.8.2.2 of TS 29.222 [Z] via the CAPIF-3 interface. The body of the HTTP POST request shall include the invocation log details provided in clause 6.Y.2.

End of Changes