3GPP TSG-SA5 Meeting #160	S5-251742

Goteborg, Sweden, 07 - 11 April 2025

Title:	Reply LS on Continuous MDT

Response to:	S5-251326-R3-250891

Release:	Rel-19

Work Item:

Source:	TSG-SA5 Meeting #160

To:	TSG-RAN3

Cc:	SA2

Contact person:	Sreekumar Pothera Kalloor

sreekumar.pk@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA5 would like to thank TSG RAN3 for their LS in R3-250891 on Continuous MDT.  SA5 has discussed the two problem statements. SA5 confirms the feasibility of solutions in line with RAN3 requirements, based on:

- OAM provides the means to RAN to identify that a Management based MDT session is for continuous MDT.

- In order to maintain the Continuous MDT configuration during UE transitions from RRC Idle state to RRC Connected state Immediate MDT configuration is stored in the core network.

- Reuse Cell Traffic Trace mechanism: This means that a gNB sends CELL TRAFFIC TRACE message to the AMF with anonymization level set to a new identifier for trace correlation every time it allocates a TR/TRSR for Immediate or Logged MDT. This identifier is allocated by the AMF. Every time a gNB allocates a TR/TRSR for Logged or Immediate MDT the AMF sends to the TCE the new correlation identifier together with the TR/TRSR of the Logged or Immediate MDT configuration.

For further work in SA5, SA5 would like to ask RAN3 the following questions:

- Is the following understanding from SA5 on the scenario for “Continuous MDT” correct?

Management based Immediate and Logged MDT is configured in a gNB. While the UE is in RRC Connected state, the gNB selects a UE for Continuous MDT and configures the UE with an Immediate and a Logged MDT Configuration. Immediate MDT is activated while the UE is in RRC Connected state. The UE changes from RRC Connected state to RRC Idle /RRC Inactive state, the UE stops Immediate MDT measurements and starts logging Logged MDT measurements according to the Logged MDT configuration. When the UE comes back to RRC Connected state, the gNB will need to reconfigure the UE with the “same” Immediate MDT Configuration since upon transition to RRC Idle state the Immediate MDT Configuration will be erased from the UE. The UE will continue measurements in RRC Connected State according to the configured Immediate MDT Configuration.

- Which measurements shall be supported by “Continuous MDT”?
Is continuous MDT applicable for all measurements that can be configured to a UE in RRC Connected or RRC Idle states or is it only applicable when there exists at least one measurement that is common in RRC Idle and RRC Connected states e.g., RSRP measurements?

How to link Immediate and Logged MDT Configurations?
With regards to RAN3 requirements to avoid UE impacts, SA5 would like to ask RAN3 to clarify requirements on the measurement configuration for continuous MDT. Can SA5 assume that there may be MDT Configurations for which a certain measurement is reported with different configuration parameters, e.g., periodicities, that are equivalent with respect to Continuous MDT? Namely, can there be equivalent MDT configurations for Continuous MDT, e.g., for a given Logged MDT Configuration can there be multiple Immediate MDT configurations that are considered equivalent?

Furthermore, SA5 wants to inform RAN3 that SA5 discusses to consider a different Area Scope over which Continuous MDT applies to avoid solutions requiring that OAM configures Continuous MDT to all RAN nodes within the area scope.

# 2	Actions

To RAN3

ACTION: 	SA5 would like to ask RAN3 to take the above agreements from SA5 into account and provide answers to the above listed questions.

# 3	Dates of next TSG SA WG5 meetings

SA5#161		19 May - 23 May 2025				Fukuoka, Japan

SA5#162		25 August - 29 August 2025		Goteborg, Sweden