3GPP TSG-SA5 Meeting #159	S5-251751

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

# Source:	Ericsson

# Title:	Rel-19 pCR TS 28.572 Plan Configuration Examples

# Document for:	Approval

Agenda Item:	6.19.9.1 - Management of planned configurations

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]		3GPP TS 28.572: "Management of planned configurations"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.572[1].

Begin of modifications

## 6.1	Planned configurations

### 6.1.1	Definition

The current configuration is the configuration currently used by the managed system. State data nodes are not included. The format of the configuration is defined by (stage 2) NRM definitions and (stage 3) schema definitions.

Editor's note: The name of the current configuration is tbc. May also be actual configuration or operational configuration.

A planned configuration is a configuration that can be manipulated without impacting the current configuration of the managed system.

Next modification

## 7.1	PlannedConfigurationDescriptor

### 7.1.1	Definition

This definition represents a planned configuration descriptor.

### 7.1.2	Information Elements

The following table specifies the information elements of a planned configuration descriptor.

| Information element name | S | Documentation and Allowed Values | Properties |
| --- | --- | --- | --- |
| id | M | The identifier of the planned configuration. | type: String multiplicity: 1 isInvariant: True isWritable: False |
| name | M | The name of the planned configuration. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| version | M | The version of the planned configuration. Its format is implementation specific. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| description | M | The textual human-readable description of the planned configuration. | type: String multiplicity: 0..1 isInvariant: False isWritable: True |
| customProperties | M | The container allowing to specify additional consumer defined properties (key value pairs) describing and qualifying the planned configuration. | type: String multiplicity: * isOrdered: False isUnique: True isInvariant: False isWritable: True |
| configurationContentType | M | The format of the planned configuration. | type: String multiplicity: 1 isInvariant: True isWritable: True |
| applyMode | M | Specifies if the operations in the operation set are treated as atomic, best effort, or if processing shall stop on the occurrence of the first error. allowedValues: - ATOMIC - BEST_EFFORT - STOP_ON_ERROR | type: ENUM multiplicity: 1 isInvariant: True isWritable: Truei |
| lastModifiedAt | M | The date and time at which the planned configuration descriptor was modified the last time by a MnS consumer. | type: DateTime multiplicity: 1 isInvariant: False isWritable: False |
| configuration | M | The planned configuration to be applied to the managed system. | type: String multiplicity: 1 isInvariant: False isWritable: True |
| validationState | M | The outcome of the last validation of the planned configuration, or "NOT_VALIDATED" if the planned configuration has not been validated yet. allowedValues: - NOT_VALIDATED - VALID - PARTIALLY_VALID - INVALID | type: ENUM multiplicity: 1 isInvariant: False isWritable: False |
| lastValidatedAt | M | The date and time at which the planned configuration was validated (the last time). The information element is absent, when the planned configuration has not been validated yet. | type: DateTime multiplicity: 0..1 isInvariant: False isWritable: False |

The configurationContentType specifies the type of the content in the 'configuration'.  This is required such that the MnS producer may correctly interpret the provided plan configuration data.  The configurationContentType application/3gpp-yang-patch shall be one such supported content type.  This type is based on the yang patch specification as per RFC 8072 but with the relaxation of the specification for the following properties as defined in RFC-8072.:

'target' : As per section 2.4 of RFC-8072 but with the relaxation of the definition of the target to allow for the omission of the module prefix.  The MnS Producer shall accept this relaxed form of the target if it determines the target is unambiguous (e.g. it is possible to specify a target like '/_3gpp-common-subnetwork:SubNetwork=Irl/_3gpp-common-mecontext:MeContext=Dublin-1/_3gpp-common-managed-element:ManagedElement=Dublin-1/_3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1' or in the relaxed form '/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1').

'value' : Encoded according to RFC-7951 but with the the relaxation of the definition of the value to allow for the omission of the module prefix.  The server shall accept this relaxed form of the value if it determines the overall value is unambiguous.

Next modification

## A.1	RESTful HTTP-based solution set

Format of planned configurations

A planned configuration might target MnS producers whose data node tree is described using YANG or OpenAPI.

When described in OpenAPI, the value of the "configuration-content-type" is either "application/vnd.3gpp.json-patch+json" defined in TS 32.158 [2] or "application/json-patch+json".

When described in YANG, the value of the "configuration-content-type" is "application/3gpp-yang-patch+json", defined in RFC 8072 [3].

A single planned configuration shall only include one configuration-content-type. If some items are modelled in YANG while others in OpenAPI these should be placed in separate planned configurations and made members of a plan configuration group.

Editor's note: Can the content types above be used given that a key is added for identifying an operation in the operation set?

Note that JSON Patch is not used together with HTTP Patch. Therefore, the atomicity and synchronicity properties of HTTP Patch do not apply here.

Plan Management operations

Resource structure

The resource structure on the MnS producer is as follows:

…/{MnSName}/{/MnSVersion}/plan-descriptors/{id}

…/{MnSName}/{/MnSVersion}/plan-group-descriptors/{id}

…/{MnSName}/{/MnSVersion}/plan-validation-jobs/{id}

…/{MnSName}/{/MnSVersion}/plan-validation-jobs/{id}/status

…/{MnSName}/{/MnSVersion}/plan-validation-jobs/{id}/plan-descriptor

…/{MnSName}/{/MnSVersion}/plan-validation-jobs/{id}/result-details

…/{MnSName}/{/MnSVersion}/plan-activation-jobs/{id}

…/{MnSName}/{/MnSVersion}/plan-activation-jobs/{id}/status

…/{MnSName}/{/MnSVersion}/plan-activation-jobs/{id}/plan-descriptor

…/{MnSName}/{/MnSVersion}/plan-activation-jobs/{id}/result-details

Editor's note: When aligned with the guidelines in TS 32.158 then the resource structure looks like, which is ffs

…/{MnSName}/{/MnSVersion}/mopc={id}/plan-descriptor={id}

…/{MnSName}/{/MnSVersion}/mopc={id}/plan-group-descriptor={id}

…/{MnSName}/{/MnSVersion}/mopc={id}/plan-validation-job={id}

…/{MnSName}/{/MnSVersion}/mopc={id}/plan-activation-job={id}

Editor's note: The exact name for "mopc" is ffs, might be also "plan-management".

The data node tree for plan management resources on the MnS producer is as follows.

{ "plan-descriptors": {}, "plan-group-descriptors": {}, "plan-validation-jobs": {}, "plan-activation-jobs": {} }

### A.1.1  Plan Descriptor

This section shows examples for create, update and delete of planned descriptor related resources

A new item of the collection resource "plan-descriptors" is created by MnS consumers using HTTP POST.  The following highlights some examples :

POST {apiRoot}/ProvMnS/1900/plan-descriptors HTTP/1.1 Host: example.org Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "user-label": "Plan-created-by-Fred-for-rollout-in-area-xy.", "description": "This is the plan for the new BTS 10.", "applyMode": ATOMIC, "configurationContentType": "application/vnd.3gpp.json-patch+json", "currentConfAddress": "http://example.org/3gpp/ProvMnS/1900", "configuration": [ { "edit-id": "op1", "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] }

The MnS producer allocates the identifier "p1" for the new resource and returns the response. The location header contains the URI of the new resource. The response body contains the representation of the new resource which is equal to the representation received in the request with the "last-modified" property added.

HTTP/1.1 201 Created Date: Wed, 21 Aug 2024 15:39:57 GMT Location: http://example.org/3gpp/ProvMnS/1900/plan-descriptors/p1 Content-Type: application/json { "name": "NewBts10Plan", "version": "2.0", "userLabel": "Plan-created-by-Fred-for-rollout-in-area-xyz", "description": "This is the plan for the new BTS 10.", "applyMode": "ATOMIC", "configurationContentType": "application/vnd.3gpp.json-patch+json", "currentConfAddress": "http://example.org/3gpp/ProvMnS/1900", "lastModified": "2024-08-21T16:39:57-01:00" "configuration": [...] }

The resource structure on the MnS producer contains the new resource.

{ "plan-descriptors": { "p1": { "name": "NewBts10Plan", "version": "2.0", " userLabel": "Plan-created-by-Fred-for-rollout-in-area-xy.", "description": "This is the plan for the new BTS 10.", "applyMode": "ATOMIC", "configurationContentType": "application/vnd.3gpp.json-patch+json", "currentConfAddress": "http://example.org/3gpp/ProvMnS/1900", "lastModified": "2024-08-21T16:39:57-01:00" "planConfiguration": [ { "edit-id": "op1", "op": "add", "path": "/SubNetwork=SN1/ManagedElement=ME10", "value": { "id": "ME10", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "Castle Charlottenburg" } } } ] } }, "plan-group-descriptors": {}, "plan-validation-jobs": {}, "plan-activation-jobs": {} } }

The next example shows how the value of the "location" attribute in the operation can be changed from "Castle Charlottenburg" to "Summer palace Charlottenburg".

PATCH {apiRoot}/ProvMnS/1900/plan-descriptors/p1 HTTP/1.1 Host: example.org Content-Type: application/json-patch+json [ { "op": "replace", "path": "/configuration/0/value/attributes/location", "value": "Summer palace Charlottenburg" } ]

To delete the planned configuration "p1" a MnS consumer might send.

DELETE {apiRoot}/ProvMnS/1900/plan-descriptors/p1 HTTP/1.1 Host: example.org

In case of success, the MnS producer returns the following message.

HTTP/1.1 204 No Content Date: Wed, 21 Aug 2024 16:12:45 GMT

The next example shows how a planned configuration can be created for the case where the configurationContentType=application/3gpp-yang-patch+json (see 7.1.2).

POST {apiRoot}/ProvMnS/1900/plan-descriptors HTTP/1.1 Host: example.org Content-Type: application/json Accept : application/json,application/problem+json { "name" : "planxyz", "applyMode" : "BEST_EFFORT", "customProperties" : { " custom-property-1" : "prop-value-1", " custom-property-2" : "prop-value-2" }, "configurationContentType" : "application/3gpp-yang-patch+json" "planConfiguration": { "patch-id" : "NRCellDU", "edit" : [ { "operation": "create", "edit-id" : "opId-001", "target": "/_3gpp-common-subnetwork:SubNetwork=Irl/_3gpp-common-mecontext:MeContext=Dublin-1/_3gpp-common-managed-element:ManagedElement=Dublin-1/_3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1", "value": { "NRCellDU": [ { "id": "1", "attributes": { " nRPCI": 15, ... } }, { "operation": "merge", "edit-id" : "opId-002", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=2", "value": { "attributes": { " nRPCI": 17, ... } } }, { "operation": "remove", "edit-id" : "opId-003", "target": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=3" }, ] } } }

### A.1.1  Plan Activation Job

This section shows examples for plan activation job related resources.  This example uses the 'inline' form of the plan descriptor (planDescriptor) :

# Create plan-activation-job POST {apiRoot}/ProvMnS/1900/plan-activation-jobs HTTP/1.1 Host: example.org Content-Type: application/json Accept : application/json,application/problem+json { "name" : "job-xyz", "description" : "optimize the Dublin area network", "isFallbackEnabled" : true, "planConfigDescrId" : "planxyz" } Response : { "id": "myjob-111", "name" : "job-xyz", "description" : "optimize the Dublin area network", "isFallbackEnabled" : true, "planConfigDescrId" : "planxyz", "status" : { "state" : "RUNNING", "startedAt" : "<some-time>", "result" : "NOT_AVAILABLE", "isCancelled" : false }, "resultDetails": { "href" : "{apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/result-details"}, "status": { "href" : "{apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/status"}, "_links" : { "self" : { "href" : "{apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111"}, "planDescriptor" : "{apiRoot}/ProvMnS/1900/…/planxyz "}, "fallback": { "href" : "…"} } } # GET plan-activation-job status (COMPLETE) GET {apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/status { "state": "COMPLETE", "startedAt": "2024-12-02T13:16:54.088Z", "stoppedAt": "2024-12-02T13:16:58.088Z", "isCancelled" : false, "resultStatus": "FINISHED_SUCCESSFUL" } # GET plan-activation-job result-details - Configuration edits/operations still being applied GET {apiRoot}/ProvMnS /1900/plan-activation-jobs/myjob-111/result-details { "patch-id" : "NRCellDU-changes-001", "resultState" : "RUNNING", "summary" : { "total": 3, "completed": 1, "succeeded": 1, "failed": 0 } } # GET plan-activation-job result-details - All configuration edits/operations applied successfully GET {apiRoot}/ProvMnS /1900/plan-activation-jobs/myjob-111/result-details { "patch-id" : "NRCellDU-changes-001", "resultState" : "FINISHED_SUCCESSFUL", "summary" : { "total": 3, "completed": 3, "succeeded": 3, "failed": 0 } } # GET plan-activation-job result-details with verbose option (expand=all) - All configuration edits/operations applied successfully GET {apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/result-details?expand=all { "patch-id" : "NRCellDU-changes-001", "edit-status" : { "edit" : [ { "edit-id" : "opId-001", "state" : "SUCCEEDED" }, { "edit-id" : "opId-002", "state" : "SUCCEEDED" }, { "edit-id" : "opId-003", "state" : "SUCCEEDED" } ] }, "resultState" : "FINISHED_SUCCESSFUL", "summary" : { "total": 3, "completed": 3, "succeeded": 3, "failed": 0 } } # GET plan-activation-job result-details ( with failure ) GET {apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/results-details?expand=all { "patch-id" : "NRCellDU-changes-001", "edit-status" : { "edit" : [ { "edit-id" : "opId-001", "state" : "FAILED", "errors" : { "error" : [ { "error-type": "application", "error-tag": "data-exists", "error-path": "/_3gpp-common-subnetwork:SubNetwork=Irl/_3gpp-common-mecontext:MeContext=Dublin-1/_3gpp-common-managed-element:ManagedElement=Dublin-1/_3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1/_3gpp-nr-nrm-nrcelldu:NRCellDU=1", "error-message": "Data already exists; cannot be created" } ] } }, { "edit-id" : "opId-002", "state" : "SUCCEEDED" }, { "edit-id" : "opId-003", "state" : "SUCCEEDED" } ] }, "resultState" : "FINISHED_PARTIALLY_SUCCESSFUL", "summary" : { "total": 3, "completed": 3, "succeeded": 2, "failed": 1 } } # GET plan-activation-job result-details with complete configuration edit/operation failure GET {apiRoot}/ProvMnS/1900/plan-activation-jobs/myjob-111/results-details { "patch-id" : "NRCellDU-changes-001", "edit-status" : { "edit" : [ { "edit-id" : "opId-001", "state" : "FAILED", "errors" : { "error" : [ { "error-type": "application", "error-tag": "data-exists", "error-path": "/_3gpp-common-subnetwork:SubNetwork=Irl/_3gpp-common-mecontext:MeContext=Dublin-1/_3gpp-common-managed-element:ManagedElement=Dublin-1/_3gpp-nr-nrm-gnbdufunction:GNBDUFunction=1/_3gpp-nr-nrm-nrcelldu:NRCellDU=1", "error-message": "Data already exists; cannot be created" } ] } }, { "edit-id" : "opId-002", "state" : "FAILED", "errors" : { "error" : [ { "error-type": "application", "error-tag": "data-exists", "error-path": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=2", "error-message": "Data already exists; cannot be created" } ] } }, { "edit-id" : "opId-003", "state" : "FAILED", "errors" : { "error" : [ { "error-type": "application", "error-tag": "data-exists", "error-path": "/SubNetwork=Irl/MeContext=Dublin-1/ManagedElement=Dublin-1/GNBDUFunction=1/NRCellDU=3", "error-message": "Data already exists; cannot be created" } ] } } ] }, "resultState" : "FINISHED_FAILED", "summary" : { "total": 3, "completed": 3, "succeeded": 0, "failed": 3 } }

End of modifications