3GPP TSG-SA5 Meeting #160	S5-251755

Goteborg, Sweden, 07 - 11 April 2025

# Source:	NEC

# Title:	pCR DraftCR Rel-19 TS 28.105 Editorials corrections to the requirements of Managing ML models in use in a live network

# Document for:	Approval

Agenda Item:	6.19.1.1

# 1	Decision/action requested

The group is requested to check and approve the proposal.

# 2	References

[1] 		S5-250780 “DraftCR Rel-19 TS 28.105 AIML management – phase 2”.

[2]	      3GPP TS 28.105: Artificial Intelligence / Machine Learning (AI/ML) management".

# 3	Rationale

The table of requirements is added separately in the draft CR [1] while it should have been added to existing requirements for AI/ML inference performance management - Table 6.5.1.3-1 [2].

Added reference clause numbers for the relevant use case in the table of requirements.

# 4	Detailed proposal

Next change

## 6.5	AI/ML inference

### 6.5.x	Managing ML models in use in a live network

#### 6.5.x.1	Description

These use cases deal with managing ML models that are in-use or deployed in the live network. These use cases will apply to all supported inference capabilities.

#### 6.5.x.2 			Use cases

##### 6.5.x.2.1	Handling of underperforming ML trained models in live networks

Actions may need to be taken by a network operator once trained ML models has been identified that is contributing towards non-optimal running of the network. These actions may involve for example, without service interruptions, reverting to running of the network without ML based optimizations (i.e deavtivating the AIMLInferenceFunction) or replacing current ML model with an earlier model one that was performing better (i.e deploying a new MLmodel).

##### 6.5.x.2.2	Performance monitoring of Network Functions with ML trained models in live networks

Several trained ML models maybe in use in an operator network with each one of them influencing network performance. The Network Functions with these ML models need to be monitored to ensure network is running optimally with these models in use. KPIs for evaluating runtime performance of Network Functions using ML models should be provided for this purpose as part of the model and these KPIs would need to be collectable by the network operator.

#### 6.5.1.3	Requirements for AI/ML inference performance management

Table 6.5.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- AI/ML_INF_PE-01 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to get the inference output provided by an AI/ML inference function (e.g., MDAF, NWDAF or RAN function). | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ- AI/ML_INF_PE-02 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to get the performance evaluation of an AI/ML inference output as measured by a defined set of performance metrics | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ- AI/ML_INF_PE-03 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to provide feedback about an AI/ML inference output expressing the degree to which the inference output meets the consumer's expectations. | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ- AI/ML_INF_PE-04 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to be informed about the executed actions that were triggered based on the inference output provided by an AI/ML inference function (e.g., MDAF, NWDAF or RAN function). | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ- AI/ML_INF_PE-05 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to obtain the performance data related to an ML model or an AI/ML inference function (e.g., MDAF, NWDAF or RAN function). | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ-AI/ML_PERF-SEL-1 | The ML training MnS producer shall have a capability allowing an authorized MnS consumer to discover supported AI/ML performance measurements related to AI/ML inference and select some of the desired measurements based on the MnS consumer’s requirements. | AI/ML performance measurements selection based on MnS consumer policy (clause 6.5.1.2.2) |
| REQ-AI/ML_PERF-POL-1 | The AI/ML MnS producer shall have a capability allowing the authorized MnS consumer to indicate a performance policy related to AI/ML inference. | AI/ML performance measurements selection based on MnS consumer policy (clause 6.5.1.2.2) |
| REQ-AI/ML_INF-LIVES-01 | The 3GPP management system should have a capability enabling an authorized consumer to identify ML Model(s) that caused performance measurement and/or KPI degradation. | Handling of underperforming ML trained models in live networks (6.5.x.2.1) |
| REQ-AI/ML_INF-LIVES-02 | The 3GPP management system should have a capability to recommend remedial actions to address performance measurement and/or KPI degradation caused by ML model(s). | Handling of underperforming ML trained models in live networks (6.5.x.2.1) |
| REQ-AI/ML_INF-LIVES-03 | The 3GPP management system should be able to collect, from the network, performance data pertaining to Network Functions actively utilizing ML models. | Performance monitoring of Network Functions with ML trained models in live networks (6.5.x.2.2) |

Next change