3GPP TSG-SA5 Meeting #160	S5-251802

Goteborg, Sweden, 7 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0619 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.291 Addition of domain features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHFSeg |  |  |  |  |  | Date: |  |  | 2025-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of domain feature |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of a domains as features, there are legacy domains can be used without indication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support for indicating domains will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-251509. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Nchf_ConvergedCharging API. They shall be negotiated using the extensibility mechanism defined in subclause 6.6 of 3GPP TS 29.500 [299].

Table 6.1.8-1: Supported Features

| Feature number |  | Feature Name |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | CHFCQM |  | CHF-controlled quota management i.e. support for temporary offline. |  |
| 2 |  | AF_Charging_Identifier |  | Indicates the support of long character strings as charging identifiers. |  |
| 3 |  | 5GIEPC_CH |  | 5GS interworking with EPC. |  |
| 4 |  | ATSSS |  | This feature indicates support of Access Traffic Steering, Switching, Splitting (ATSSS). |  |
| 5 |  | ETSUN |  | This feature indicates support of Enhancing Topology of SMF and UPF in 5G Networks (ETSUN). |  |
| 6 |  | EnhancedDiagnostics |  | Support the enhanced diagnostics. |  |
| 7 |  | AMF_subs_PRA |  | PRA(s) subscription by CHF in AMF. |  |
| 8 |  | FilterRuleList |  | Support of multiple filter rules in the final unit indication. |  |
| 9 |  | TEI17_NIESGU |  | This feature indicates support of GERAN/UTRAN access. |  |
| 10 |  | IMS |  | This feature indicates support of IP Multimedia Subsystem (IMS). (NOTE 1). |  |
| 11 |  | QoSMonitoring |  | This feature indicates support of QoS Monitoring. |  |
| 12 |  | Announcement |  | This feature indicates support of announcements. (NOTE 1) |  |
| 13 |  | 5GLAN |  | This feature indicates support of 5G LAN-type services. |  |
| 14 |  | URLLC |  | This feature indicates support of URLLC. |  |
| 15 |  | NotifyInfoResponse |  | This feature indicates support of response with information for a notification. |  |
| 16 |  | ES4xx |  | Extended Support of HTTP 400, 403, 404 allowing use of either ChargingDataResponse or ProblemDetails in the response. |  |
| 17 |  | ES3xx |  | Extended Support of HTTP 307 and 308 redirections, an NF that does not support this feature does only support HTTP redirection as specified for 3GPP Release 15 and 16. |  |
| 18 |  | EdgeComputing |  | This feature indicates support of edge computing domain charging. (NOTE 1) |  |
| 19 |  | 5GSCIoT |  | This feature indicates support of 5GS control plane CIoT optimization. |  |
| 20 |  | SMF_Charging_Id |  | Indicates the support of strings as SMF charging identifiers. |  |
| 21 |  | SNPN |  | This feature indicates support of Stand-alone Non-Public Network. |  |
| 22 |  | IDC_CH |  | This feature indicates support of IMS Data Channel charging. |  |
| 23 |  | 5MBS_CH |  | This feature indicates 5G multicast-broadcast services charging. (NOTE 1) |  |
| 24 |  | SatelliteAccess |  | This feature indicates support of NR satellite access. |  |
| 25 |  | NSREP |  | This feature indicates support of Network slice replacement charging. |  |
| 26 |  | TSN |  | This feature indicates support of time sensitive networking. (NOTE 1) |  |
| 27 |  | 5GSATB |  | This feature indicates support of satellite backhaul. |  |
| 28 |  | NSAC_CH |  | This feature indicates support of Network slice admission control charging. (NOTE 1) |  |
| 29 |  | NSSAA |  | This feature indicates support of Network slice-specific authentication and authorization charging. (NOTE 1) |  |
| 30 |  | ProSe |  | This feature indicates support of 5G ProSe. (NOTE 1) |  |
| 31 |  | INTER_CHF |  | This feature indicates support of inter-CHF communication. |  |
| 32 |  | RangingSL |  | This feature indicates support of Ranging and Sidelink Positioning. |  |
| 3x |  | EE_NS_CH |  | This feature indicates support of energy information for network slice. |  |
| NOTE 1: The feature is used to indicate a charging domain or subsystem. |  |  |  |  |  |

* * * End of Changes * * * *