3GPP TSG-SA5 Meeting #160	S5-251816

Goteborg, Sweden, 07 - 11 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0062 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 32.254 Addition of charging architecture for CAPIF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CH_CAPIF_EXT |  |  |  |  |  | Date: |  |  | 2025-04-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of CCF in the charging architecture for CAPIF charging. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of CCF to both service based and reference point representation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CCF will not be able to charge for exposure services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.1, and 4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-251512. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [100] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [100].

AI/ML	Artificial Intelligence/Machine Learning

AS	Application Server

BD	Billing Domain

CAPIF	Common API Framework

CCF	CAPIF Core Function

CDF	Charging Data Function

CGF	Charging Gateway Function

CTF	Charging Trigger Function

ECUR	Event Charging with Unit Reservation

IE	Information Element

IEC	Immediate Event Charging

NEF	Network Exposure Function

PEC	Post Event Charging

SCEF	Service Capability Exposure Function

SCS	Services Capability Server

SGSN	Serving GPRS Support Node

RCAF	RAN Congestion Awareness Function

* * * Next Change * * * *

## 4.1	High-level Service Exposure Function architecture

The overall architecture and detailed description for Service Capability Exposure which enables the 3GPP network to securely expose its services and capabilities provided by 3GPP network interfaces to external 3rd party service provider applications are defined in 3GPP TS 23.682 [243], and figure 4.1.1 is highlighting T8 involved for interworking between SCEF and SCS/AS.

Figure 4.1.1: Northbound API of 3GPP Architecture Service Capability Exposure

The overall architecture and detailed description for Common API framework (CAPIF) for 3GPP northbound APIs that are applicable to any service APIs when used by northbound entities, as well as any interactions between the CAPIF and the service APIs themselves, are defined in 3GPP TS 23.222 [202]. The CAPIF applies to both EPS and 5GS, can be hosted within a PLMN or SNPN, and is independent of the underlying 3GPP access (e.g. E-UTRA, NR). The figure 4.1.2 describes the functional model for the CAPIF.

Figure 4.1.2: Functional model for the CAPIF

* * * Next Change * * * *

## 4.4	Northbound API converged charging architecture

The architectural options for Northbound API (including CAPIF) converged charging are depicted in figure 4.4.1 in service-based representation:

Figure 4.4.1: Northbound API converged charging architecture

Architectural options of figure 4.4.1 apply to any Northbound API converged charging architecture of this clause.

Details on the interfaces and functions can be found in TS 32.240 [1] for the general architecture components, Ga is described in clause 5.2.4 and Bea in clause 5.2.5 of this document, and Nchf is described in TS 32.290 [57].

Figure 4.4.2 depicts the Northbound API converged charging architecture for non-roaming in reference point representation:

Figure 4.4.2: Northbound API converged charging architecture non-roaming reference point representation

* * * End of Changes * * * *