3GPP TSG-SA5 Meeting #160 	S5-251962

Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.401 | CR | 0044 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 32.401 PM Counter Metadata |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussed in SA5#159, the CR proposes to enhance the collection methods with additional clairfications as part of the Measurement Methods description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of Measurement methods used in the counter |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lacking machine-readable information for a performance measurement |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

### 4.2.2	Measurement result generation

Each measurement job will be collecting result data at a particular frequency, known as the granularity period of the measurement job. At the end of the granularity period a scheduled result report is generated for each measurement job that is actively collecting performance measurement result data, i.e. for all the measurement types and measured resources covered by the job.

The measurement result data can be collected with a number of collection methods:

-	cumulative incremental counters triggered by the occurrence of the measured event;

-	status inspection (i.e. a mechanism for high frequency sampling of internal counters at pre-defined rates);

-	gauges (i.e. high tide mark, low tide mark);

-	discrete event registration, where data related to a particular event is captured ;

-	object mapping, where the measured object of the measurement is mapped from one to another.

These collection methods are:

-	Cumulative Counter (CC): The NE maintains a running count of the event being counted. The counter is reset to a well-defined value (usually "0") at the beginning of each granularity period.

-	Status Inspection (SI): Network elements maintain internal counts for resource management purposes. These counts are read at a predetermined rate, the rate is usually based upon the expected rate of change of the count value. Status inspection measurements shall be reset at the beginning of the granularity period and will only have a valid result at the end of the granularity period.

-	Gauge: Gauges represent dynamic variables that may change in either direction. Gauges can be integer or real valued. If a gauge is required to produce low and high tide marks for a granularity period (e.g. minimum and maximum call duration), then it shall be reinitialised at the beginning of each granularity period. If a gauge is required to produce a consecutive readout over multiple granularity periods (e.g. cabinet temperature), then it shall only be reinitialised at the start of a recording interval (see definition of "recording interval" in clause 5. 4.1. 3).

-	Discrete Event Registration (DER): Data related to a particular event is captured. Every nth event is registered, where n can be 1 or larger. The value of n is dependent on the frequency of occurrence of the event being measured. 
DER measurements shall be reset at the beginning of each granularity period and will only have a valid result at the end of the granularity period.

-	Object Mapping (OM): The 3GPP entity maps the measured object(s) for the measurements, that are collected by a method defined by another SDO, from the non-3GPP defined MOI(s) to 3GPP defined MOI(s) representing the managed 3GPP NF(s), and generates the 3GPP measurement(s) for the 3GPP defined MOI with or without processing of the mapped (source) measurements.

Collection method provides extended information on the measurement methods it employs, indicating whether its output is derived from the Sum, Average, Minimum, Maximum, Peak or Mean value of the samples within the granularity period.

The measurement result data can be collected in a non-3GPP defined NE of the network with a number of collection methods which are not defined in this document. These collection methods are referred to as “externally defined collection methods”

This following item describes the collection method to be used for collecting measurements result data from a non-3GPP defined NE.

Transparent Forwarding (TF): The non-3GPP defined NE maintains a count based on the NE’s “externally defined collection method”. The 3GPP system maintains a measurement count that is a snapshot/reading of the non-3GPP defined NE count at each granularity period.

*** END OF CHANGE ***