3GPP TSG-SA5 Meeting #160																		S5-252023

Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0382 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.532 PM File extension |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The file extension definiation is missing in the Performance data file naming convention. This may be an issue when a consumer try to open the file. In some use cases, e.g., multiple compressions applied on the file output, the consumer needs to know on the sequence of operations done on the original file. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add notes that File output shall have an extension to indicate its format |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Consumer will not be aware the file format in some use cases e.g. multiple compressions have been applied to the file |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.2.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

##### 11.3.2.1.4	Performance data file naming convention

This clause defines a rule that shall be applied for constructing names for files containing performance data.

<Type><Startdate>.<Starttime>-[<Enddate>.]<Endtime>[_-<jobIdList>][_<UniqueIdList>][_-_<RC>]<.extension>

1)	The "Type" field indicates if the file contains measurement results for single or multiple measured objects and/or granularity periods where:

-	"A" means single measured object, single granularity period (this is used when granularity period is equal to reporting period);

-	"B" indicates multiple measured objects, single granularity period (this is used when granularity period is equal to reporting period);

-	"C" signifies single measured object, multiple granularity periods (this is used when reporting period is multiples of the granularity period and will contain multiple measurement reports);

-	"D" stands for multiple measured objects, multiple granularity periods (this is used when reporting period is multiples of the granularity period and will contain multiple measurement reports).

2)	The "Startdate" field indicates the date when the granularity period began if the "Type" field is set to A or B. If the "Type" field is either "C" or "D" then "Startdate" contains the date when the first granularity period of the measurement results contained in the file started. The "Startdate" field is of the form YYYYMMDD, where:

-	YYYY is the year in four-digit notation;

-	MM is the month in two digit notation (01 - 12);

-	DD is the day in two-digit notation (01 - 31).

3)	The "Starttime" field indicates the time when the granularity period began if the "Type" field is set to A or B. If the "Type" field is either "C" or "D" then "Starttime" contains the time when the first granularity period of the measurement results contained in the file began. The "Starttime" field is of the form HHMMshhmm, where:

-	HH is the two-digit hour of the day (local time), based on 24-hour clock (00 - 23);

-	MM is the two digit minute of the hour (local time), based on 60-minutes clock (00 - 59);

-	s is the sign of the local time differential from UTC (+ or -), in case the time differential to UTC is 0 then the sign may be arbitrarily set to "+" or "-";

-	hh is the two-digit number of hours of the local time differential from UTC (00-23);

-	mm is the two digit number of minutes of the local time differential from UTC (00-59).

4)	The "Enddate" field shall only be included if the "Type" field is set to "C" or "D", i.e. measurement results for multiple granularity periods are contained in the file. It identifies the date when the last granularity period of these measurements ended, and its structure corresponds to the "Startdate" field.

5)	The "Endtime" field indicates the time when the granularity period ended if the "Type" field is set to A or B. If the "Type" field is either "C" or "D" then "Endtime" contains the time when the last granularity period of the measurement results contained in the file ended. Its structure corresponds to the "Starttime" field.

6)	The "UniqueIdList" field indicates the DNs of the measured objects.

7)	The "RC" field is a running count, starting with the value of "1", and shall be appended only if the filename is otherwise not unique, i.e. more than one file is generated and all other parameters of the file name are identical. Therefore it may only be used by the EM, since the described situation cannot occur with NE generated files. Note that the delimiter for this field, _-_, is an underscore character (_), followed by a minus character (-), followed by an underscore character (_).

8)	The "jobIdList" indicates the measurement job id(s) that the performance data file is associated with.

9) 	The file shall have a file extension indicating its format (for example, ".xml"). If the file has been further processed by the MnS producer before it is made available to the MnS consumer (for example, the file has been compressed), the file extension shall indicate the latest processing step (for example, the file extension becoming ".zip").

If additional processing steps have been applied by the MnS producer, the respective resulting file extension shall always indicate the respective latest processing step, thereby allowing the MnS consumer to reverse the processing.

Some examples describing file-naming convention:

1)	file name:	A20000626.2315+0200-2330+0200_gNBId.xml, 
meaning:	file in format of xml, produced for gNB <gNBId> on June 26, 2000, granularity period 15 minutes from 23:15 local to 23:30 local, with a time differential of +2 hours against UTC.

2)	file name:	B20021224.1700-1130-1705-1130_-job10_S-NSSAI.xml,
meaning:	file in format of xml, containing results for multiple measured objects, generated for measurement job job10, produced for NSI <S-NSSAI> on December 24, 2002, granularity period 5 minutes from 17:00 local to 17:05 local, with a time differential of –11:30 hours against UTC.

3)	file name:	D20050907.1030+0000-20050909.1500+0000_SubnetworkId_-_2.xml 
meaning:	file in format of xml, containing results subnetwork <SubnetworkId>, start of first granularity period 07 September 2005, 10:30 local, end of last granularity period 09 September 2005, 15:00 local, with a time differential of 0 against UTC. This is the second file for this subnetwork/granularity period combination.

4)	file name:	C20050907.1030+0000-20050909.1500+0000_gNBId.xml,
meaning:	file in format of xml, produced for the gNB <gNBId>, start of first granularity period 07 September 2005, 10:30 local, end of last granularity period 09 September 2005, 15:00 local, with a time differential of 0 against UTC.

5)	file name: A20250226.2315+0200-2330+0200_gNBId.gz

meaning: file produced for gNB <gNBId> on Feb 26, 2025, granularity period 15 minutes from 23:15 local to 23:30 local, with a time differential of +2 hours against UTC. The file extension ".gz" indictes to the MnS consumer that the file has been compressed. After un-compressing the file, "A20250226.2315+0200-2330+0200_gNBId.xml" is the resulting file. The MnS consumer now can extract performance data from the resulting XML file and process it as desired.

6)	file name: A20250226.2315+0200-2330+0200_gNBId.zip

meaning: file produced for gNB <gNBId> on Feb 26, 2025, granularity period 15 minutes from 23:15 local to 23:30 local, with a time differential of +2 hours against UTC. The file extension ".zip" indictes to the MnS consumer that the file has been compressed. After un-compressing the file, "A20250226.2315+0200-2330+0200_gNBId.xml" is the resulting file. The MnS consumer now can extract performance data from the resulting XML file and process it as desired.

7)	file name: A20250226.2315+0400-2330+0400_gNBId.zip

meaning: file produced for gNB <gNBId> on Feb 26, 2025, granularity period 15 minutes from 23:15 local to 23:30 local, with a time differential of +4 hours against UTC. The file extension ".zip" indictes to the MnS consumer that the file has been compressed. After un-compressing the file, "A20250226.2315+0400-2330+0400_gNBId.tar" is the resulting file. The file extension ".tar" indictes to the MnS consumer that the file has been tarred. After further un-taring, "A20250226.2315+0400-2330+0400_gNBId.xml" is resulting file. The MnS consumer now can extract performance data from the resulting XML file and process it as desired.

*** END OF CHANGE ***