3GPP TSG-SA5 Meeting #160	S5-252091

Goteborg, Sweden, 07 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.560 | CR | 0008 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the description of the attribute stmTargetUri |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Monstra-OAM |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The definition of attribute stmTargetUri refers to the definition of URI as of TS 32.158, which defines the URI of MnS with the scheme either http or https, and therefore restricts the protocols for signalling monitoring to http and https. This does not fit to any of the current protocols used for signalling monitoring (UDP or TCP). The current definition does not allow to configure the protocol to be used (UDP or TCP) nor the IP port to be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The definition of the attribute stmTargetUri will be changed to an URI with a 3GPP-specific URI scheme part , i.e. a scheme that is not registered by IANA. This is acceptable and no problem, since in the present use case the URI scheme is only used to format the string-valued attribute into protocol, IP address, and port number. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current definition of stmTargetUri does not allow the MnS consumer to configure the target where the captured signalling messages shall be send to. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.532: "Generic management services".

[3]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[4]	3GPP TS 33.501: " Security architecture and procedures for 5G System ".

[5]	3GPP TS 28.622: "Generic Network Resource Model (NRM); Integration Reference Point (IRP); Information Service (IS)".

[6]	void.

[7]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[8]	IETF RFC8086: GRE-in-UDP Encapsulation.

[9]	IETF draft-ietf-opsawg-pcapng-04: PCAP Next Generation (pcapng) Capture File Format.

[10]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

[11]	3GPP TS 28.623: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

Second Modification

### 6.3.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| reportingNFList | List of Network Function Distinguished Name. allowedValues: N/A | Type: DN multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| networkInterfaceTypeList | List of network interface type. The applicable network interface type names are specified based on subclause 4.2.3 of 3GPP TS 23.501 [7]. The value "ALL" is specified for the case if all the applicable interface type of the network function shall be monitored. allowedValues: ALL, N4, N5, N7, N8, N10, N11, N12, N13, N14, N15, N22, N58, N59, N80, N81. | Type: String multiplicity: 1..* isOrdered: N/A isUnique: True defaultValue: ALL isNullable: False |
| stmTargetUri | It specifies the Uniform Resource Identifier (URI) of the streaming target where the signalling traffic shall be sent. allowedValues: N/A | type: Uri multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| administrativeState | Administrative state of a managed object instance. The administrative state describes the permission to use or prohibition against using the object instance. The administrative state is set by the STM consumer. allowedValues: LOCKED, UNLOCKED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| operationalState | Operational state of manged object instance. The operational state describes if an object instance is operable ("ENABLED") or inoperable ("DISABLED"). This state is set by the object instance or the STM producer and is hence READ-ONLY. allowedValues: ENABLED, DISABLED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLED isNullable: False |

End of Modifications