3GPP TSG-SA5 Meeting #161	S5-252470

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0709 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Add new filter for PDCP PDU data volume measurements for RedCap service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RedCap_OAM |  |  |  |  |  | Date: |  |  | 2025-04-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Solution for the enhancement on the existing resource load measurements to support to monitoring the performance of RedCap service is investigated in TR 28.876 and the related normative work are recommended. This contribution proposes to add a new filter for data volume measurements to implement the above recommendation accordingly in TS 28.552. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new filter for data volume measurements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements of data volume measurements for RedCap service cannot be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

##### 5.1.3.6.1	PDCP PDU data volume Measurement

5.1.3.6.1.1	DL PDCP PDU Data Volume

a)	This measurement provides the Data Volume (amount of PDCP PDU bits) in the downlink delivered from GNB-CU to GNB-DU. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI or per UE type.

b)	CC.

c)	This measurement is obtained by counting the number of DL PDCP PDU bits sent to GNB-DU. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI or per UE type.

The measurements of DL Cell PDCP PDU Data Volume in Dual-Connectivity scenarios is not included.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels or multiplied by the number of supported S-NSSAIs.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form QosFlow.PdcpPduVolumeDL_Filter,where filter is a combination of PLMN and QoS level and S-NSSAI, or a combination of PLMN and QoS level and S-NSSAI and UEType,

where PLMN represents the PLMN ID, QoS represents the mapped 5QI or/and QCI level, and S-NSSAI represents S-NSSAI, and UEType refers to allowed values of uECellBarredAccess in TS 28.541 clause 4.4.1.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

5.1.3.6.1.2	UL PDCP PDU Data Volume

a)  This measurement provides the Data Volume (amount of PDCP PDU bits) in the uplink delievered from GNB-DU to GNB-CU. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI or per UE type. The unit is Mbit (1MBits=1000*1000 bits).

b)	CC

c)	This measurement is obtained by counting the number of bits entering the GNB-CU. The measurement is performed at the PDCP PDU level.  The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI or per UE type.

The measurements of UL Cell PDCP PDU Data Volume in Dual-Connectivity scenarios is not included.

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels or multiplied by the number of supported S-NSSAIs.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form QosFlow.PdcpPduVolumeUl_Filter,where filter is a combination of PLMN and QoS level and S-NSSAI, or a combination of PLMN and QoS level and S-NSSAI and UEType,where PLMN represents the PLMN ID, QoS represents the mapped 5QI or/and QCI level, and S-NSSAI represents S-NSSAI, and UEType refers to allowed values of uECellBarredAccess in TS 28.541 clause 4.4.1.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

End of Change