3GPP TSG-SA5 Meeting #161	S5-252522

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0221 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.104 Fix editorial errors |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2024-05-09 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Editorial improvements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fix incorrect text colour, incorrect table numbering, spelling errors, and incorrect spaces. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lower quality of specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6, 6.3, 7.2.1.1.3, 7.2.2.6.2, 7.2.3.1.3, 7.2.7.1.2, 7.2.7.2.2, 7.2.9.1.2, 8.4.2.1.3, 8.4.2.2.3, 8.4.2.5.2, 8.4.2.6.1, 8.4.2.6.2, 8.4.4.1.3, 8.4.7.1.1.3, 8.4.7.1.2.1, 8.4.7.1.2.3, 8.4.7.1.3.3, 8.4.9.1.2, 8.5.10.2, 8.5.13.1, 8.5.15.2, 8.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 5.6	AI/ML support for MDA

The MDA process may utilize AI/ML technologies. An MDA Function may optionally be deployed as one or more AI/ML inference function(s) in which the relevant ML entities are used for inference per the corresponding MDA capability. Specifications for MDA ML model training to enable ML model deployments are given in TS 28.105 [24].

* * * Next Change * * * *

## 6.3	MDA role in cross-domain service assurance

Cross-domain MDA may base its analysis on the outputs from one or multiple single-domain MDA including analytics output and other input data (e.g., PM and alarm notifications.). To facilitate service assurance the cross-domain MDA may consume output from one or multiple single-domain MDA(s). Figure 6.3-1 shows the simplest case, where a cross-domain MDA consumes the results of single-domain MDA(s).

Figure 6.3-1 Cross-domain MDA based on single-domain MDA

Figure 6.3-2 shows the case where a cross-domain MDA incorporates the results of single-domain MDA(s) which are embedded within single-domain control loopservice(s). Service assurance control loop services may be performed at single-domain where analytics part is done by MDA. The cross-domain MDA may further leverage the output from one or more single-domain control loop services for the analytics of the e2e service.

Cross domain MDA

Analytics

Single domain control loop service

Single domain control loopservice

Single domain control loopservice

Analytics

Execution

Observation

Decision

Domain-level analysis

Figure 6.3-2: Cross-domain MDA based on single-domain control loopservice

Figure 6.3-3 shows the case where a cross-domain MDA is part of a cross-domain control loop service. Also in this case, cross-domain MDA consumes the results of single-domain MDA(s). Service assurance control loop service may be performed at the cross-domain level in which the analytics is done by MDA. The cross-domain control loop may consume output from one or more multiple single-domain MDA(s) for the e2e service.

Cross domain control service

Analytics

Execution

Observation

Decision

Single domain MDA

Single domain MDA

Single domain MDA

Analytics

Domain-level analysis

Figure 6.3-3: Cross-domain control loop service based on single-domain MDA(s)

Figure 6.3-4 shows another case where a cross-domain MDA is part of a cross-domain control service. In this case, cross-domain MDA consumes the results of single-domain MDA(s) which are embedded within single-domain control service(s). Service assurance control loop service may be conducted at both levels where analytics is done by MDA, i.e. at the cross-domain and single-domain. The cross-domain MDA may consume output from one or more single-domain MDA(s) for the e2e service.

Cross domain control loop service

Analytics

Execution

Observation

Decision

Single domain control loop service

Single domain control loop service

Single domain control loop service

Analytics

Execution

Observation

Decision

Domain-level analysis

Figure 6.3-4: Cross-domain control loop service based on single-domain control loop service(s)

* * * Next Change * * * *

##### 7.2.1.1.3	Requirements

Table 7.2.1.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-COV_MDA-01 | MDA capability for coverage problem analysis shall include providing analytics for issues including, weak coverage, coverage holes, pilot pollution, overshoot coverage, or DL and UL channel coverage mismatch. | Coverage problem analysis |
| REQ-COV_MDA-02 | MDA capability for coverage problem analysis shall include providing analytics for area specific coverage problem analysis. | Coverage problem analysis |
| REQ-COV_MDA-03 | MDA capability for coverage problem analysis shall include providing a radio environment map that graphically describes the radio coverage characteristics (e.g. RSRP or SINR) of the selected cluster of cells. | Coverage problem analysis |
| REQ-COV_MDA-04 | MDA capability for coverage problem analysis shall include providing optimum configurations of a RAN node based on the radio environment map that graphically describes the radio coverage characteristics (e.g. RSRP or SINR) of a selected cluster of cells. | Coverage problem analysis |

* * * Next Change * * * *

##### 7.2.2.6.2	Description

With the development of diverse communication services and the increasing number of connections, user data volume demanded by end users grows rapidly. As the user traffic increases, user experience deteriorates significantly. Therefore, UE throughput related measurements need to be analysed to identify the potential traffic congestion time and place by monitoring the network and provide the corresponding optimization recommendation that can be implemented to avoid potential traffic congestion.

Two types of traffic congestion as follows.

-	Regular traffic congestion: Congestion that have heavy traffic which occurs at a fixed time or place every day, for example, during peak hours of daily morning, heavy traffic will result in a degraded user experience

-	Non-Regular traffic congestion:  user traffic increases sharply in a specific area, for example, in commercial activities and sports events, a large number of users gather for a period of time, resulting in rate experience deterioration.

The occurrence of such traffic congestion problem may seriously affect user experience and can be detected based on the UE throughput related measurements analysis.

Figure 7.2.2.6.2-1 UE throughput analytics

Due to the complexity of 5G network and wireless environment, multiple types of performance deterioration are related with high user traffic volume. In such scenarios, the related performance measurements such as UE throughput, Radio resource utilization (i.e PRB utilization), number of RRC Connections can be used to analyze the situation. As shown in the Figure 7.2.2.6.2-1, if the preset threshold is reached, it may be considered that there is a traffic congestion problem. The user experience related performance should be analysed and identified to help to resolve improve the user experience. In this case, the consumer may need to be able to set the policy to help producer to analyze the congestion problems. The policy could be, for example, the threshold of the UE throughput related performance measurements (threshold and absolute deviation of the traffic in the figure).

The MDAS producer for radio network may also provide the traffic congestion severity and the recommendations to solve the UE throughput issues identified by the analytics congestion scenarios. The traffic congestion severity may be for example the value can be critical, major, minor, warning etc. The recommended actions may be for example to change the mobility parameters (e.g. MRO or LBO) or antenna RF parameters (e.g. CCO) such as downtilt and azimuth to where they are needed.

* * * Next Change * * * *

##### 7.2.3.1.3	Requirements

Table 7.2.3.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-FAILURE_PRED_MDA-01 | Void | Void |
| REQ-FAILURE_PRED_MDA-02 | MDA capability for failure prediction shall be able to obtain basic health maintenance knowledges (including, the relationship between the failures or potential failures and the related maintenance actions) through predefined expertise or model training. | Failure prediction |
| REQ-FAILURE_PRED_MDA-03 | MDA capability for failure prediction shall be able to provide the analytics output including predictions of potential service failures, as well as the possible recommendation actions to prevent failures. | Failure Prediction |
| REQ-FAILURE_PRED_MDA-04 | MDA capability for failure prediction should include the ability to predict failures across or within domains and provide analytics outputs for predicted failures. | Failure Prediction |
| REQ-FAILURE_PRED_MDA-05 | MDA capability for failure prediction should be able to provide the analytics output including information used for failure prediction based on the threshold information provided by the consumer. | Failure Prediction |
| REQ-FAILURE_PRED_MDA-06 | MDA capability for failure prediction should have a capability to indicate the management data to be collected for the predicted failure. | Failure Prediction |
| REQ-FAILURE_PRED_MDA-07 | MDA capability for failure prediction should be able to provide the analytics output including trend indication about the predicted failure and predicted end time of the failure. | Failure Prediction |

* * * Next Change * * * *

##### 7.2.7.1.2	Use case

The 3GPP system is a resource limited system, no matter whether the NF is working on virtualized resources or physical resources.

Resource shortage would affect the QoS and potentially impact users’ quality of experience (QoE), e.g., by lowering the users’ data throughput, prolonging the users’ data delay, raising the rejections for the establishment of new connections (e.g., RRC connection), sessions (e.g., PDU session) and resources (e.g., QoS flows, DRBs, etc.) and increasing the drops of the existing connections, sessions, and resources. This may also consequently lead to risking or failing SLAs.

On the other hand, resource excess would cause wastage that leads to additional CapEx and OpEx.

Therefore, it is imperative to ensure optimum and efficient resource utilization for the NFs.

The resource utilization of an NF is heavily dependent on load or traffic patterns, which could vary in different coverage areas (e.g., business area, entertainment area, and residential area) and in different time periods (weekdays and time of the day). It is desirable that the spare resource of the low-usage areas can be allocated to the busy areas. It is expected that MDA can perform an analysis of the resource utilization for physical resources or virtualized resources for the 3GPP NFs (in a specific domain or cross domains) to indicate the resource usage patterns in the past and predict the resource usage trend for some time periods in the future. The physical resources to be analyzed may include hardware resources (e.g., CPU), DL and UL PRBs (for gNB), etc., while the virtualized resources to be analyzed may include virtual CPU, virtual memory, virtual disk, etc. It is also very useful that MDA correlates the resource analytics across 3GPP NFs and provides recommendations that can be utilized to efficiently orchestrate the resources among NFs between the low usage and high usage areas for some time periods. The recommended actions could be for example to optimise, i.e., increase or decrease the capacity of gNB to enhance allocation of the physical resources or to schedule the "scale in" and "scale out" of VNFs via ETSI MANO system to optimize the allocation of the virtualized resources.

* * * Next Change * * * *

##### 7.2.7.2.2	Use case

As described in TS 23.501 [28], a 5GC NF can become overloaded when it is operating over its nominal capacity resulting in diminished performance (including impacts to handling of incoming and outgoing traffic). Some mechanism, such as control plane congestion control as described in TS 23.501 [28] is designed for the purpose of avoiding and handling of 5GC NF overload. For example, as described in clause 5.19.7 of TS 23.501 [28], when the AMF is under overload conditions, it may reject the received request from the UE depending on various aspects. And the UE will send a new request after some time. It is possible that the new request will be rejected again because of the load of the AMF. In virtualized environment, the signaling request may be rejected due to inadequacy of available resources at the target 5GC NF e.g. AMF or SMF. If such situation can not be resolved, it will probably cause signalling storm for the whole network and affect the services (e.g. calls and data connections) provided by the network.

It is desirable to use MDA to assist control plane congestion analysis in order to detect, prevent or resolve identified congestion issue happened at the control plane. MDAS producer may utilize the collected PM, FM, network topology data, virtual resource information provided from ETSI NFV MANO and etc.) for control plane congestion analysis and provides analytics report containing identified or predicted congestion issue for the target 5GC NF (e.g. AMF, SMF). The analytics report also provides recommended actions to optimize the target 5GC NF for avoiding or resolving congestion issue. Based on the recommendation in the report, 3GPP management system can adjust (e.g., scale-up the virtual resource) the resources to better facilitate processing of the received control plane messages. MDA MnS consumer may need to take further action on control plane congestion issue based on the analytics output from MDAF. The root cause of the congestion issue and the predicted duration of congestion issue may be of interest to the consumer and can be provided in the analytics report.

* * * Next Change * * * *

##### 7.2.9.1.2	Use case

For ML model training, collecting a large volume of measurement data instances does not necessarily enhance training performance. The measurement data collected for ML training may exhibit high correlation (linear or non-linear), resulting in significant redundancy. Consequently, using the entire dataset for model training can lead to unnecessary consumption of computational resources and energy.

Optimizing training data preparation based on correlation analysis and redundancy information can be very helpful. Correlation analysis can identify redundancy patterns within the measurement data for ML training, enabling more efficient model training. This may be achieved in the following ways:

-	For a given task (e.g. analytics, model training), correlation analysis can identify relationships among the data, resulting in a reduced dataset that can be used for training the ML model. This approach may improve training efficiency while managing the impact on model performance compared to using the full dataset. The analysis may also provide recommendations, such as optimizing data collection for training purposes.

-	Regularly updating the correlation analytics may be required, as correlation relationships may evolve over time. This is very useful when there is a recurring need to re-train the ML model.

* * * Next Change * * * *

##### 8.4.2.1.3	Analytics output

The specific information elements of the analytics output for service experience analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.2.1.3-1.

Table 8.4.2.1.3-1: Analytics output for Service experience analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| serviceExperienceId | The identifier indicates the analytics report is related with service experience analysis. | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceInformation | This field include the service information related to this analysis such as service name. See NOTE 1. | O | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceExperienceIssueType | Indication of the service experience issue type. allowedValues: - RAN_ISSUE; - CN_ISSUE; - OTHER_ISSUE | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| affectedObjects | The managed object instances where the service experience is applicable, e.g. SubNetwork Instance, NetworkSlice Instance, NetworkSlice subnetwork Instance. The subset values of this field may be different due to cross domain management and domain management. | O | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| serviceExperienceStatistics | The statistics of the level of service experience for a service in a certain time period, e.g. there are five levels which are represented by 1, 2, 3, 4, 5 where level 1 represents the users are enduring bad experience while level 5 represents the users' requirements are perfectly satisfied. allowedValues:LEVEL_1, LEVEL_2, LEVEL_3, LEVEL_4, LEVEL_5 | O | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceExperiencePredictions | The predictions of the level of service experience for a service in a certain time period. | O | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE 1: This field of serviceInformation is used for MDA MnS producer to include the names of e2e services (e.g., browsing, video streaming etc.) and detail information (specific information of an e2e service). |  |  |  |

* * * Next Change * * * *

##### 8.4.2.2.3	Analytics output

The specific information elements of the analytics output for network slice throughput analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.2.2.3-1.

Table 8.4.2.2.3-1: Analytics output for network slice throughput analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| networkSliceThroughputAnalysisId | Network slice throughput analysis identifier | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| networkSliceThroughputIssueType | Indication of the network slice throughput issue type allowedValues: NONE, RAN_ISSUE, CN_ISSUE, BOTH_RAN_CN_ISSUE | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| networkSliceThroughputUserStatistics | The statistics of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of users, for which the required SLS throughput is met. allowedValues: 0 to 100 | O | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| networkSliceThroughputTimeStatistics | The statistics of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of time, during which the required SLS throughput is met. allowedValues: 0 to 100 | O | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| networkSliceThroughputUserPredictions | The predictions of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of users, for which the required SLS throughput is predicted to be met. allowedValues: 0 to 100 | O | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| networkSliceThroughputTimePredictions | The predictions of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of time, during which the required SLS throughput is predicted to be met. allowedValues: 0 to 100 | O | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

* * * Next Change * * * *

##### 8.4.2.5.2	Enabling data

The enabling data for SLSAnalysis.NetworkSliceLoadAnalysis MDA type are provided in table 8.4.2.5.2-1.

Table 8.4.2.5.2-1: Enabling data for network slice load analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Number of PDU sessions of network slice | Mean number of PDU sessions of network and network Slice Instance (clause 6.4.1 in TS 28.554 [5]). |
|  | Number of PDU Sessions successfully setup | Number of PDU Sessions successfully setup (clause 5.1.1.5 in TS28.552 [4]). |
|  | Mean Number of PDU sessions | Number of PDU sessions (Mean) (clause 5.3.1.1 in TS 28.552 [4]). |
| Network Data Analytics | Analysis results from the control plane produced by NWDAF | Analytics data from NWDAF in TS 23.288 [10] including e.g. Slice load level related network data analytics clause 6.3, and the analytics for user plane performance (i.e. average/maximum traffic rate, average/maximum packet delay, average packet loss rate in clause 6.14. |
| Configuration data | MOIs of the cells, NW slice/NW slice subnet, 5GC NFs | NRM information TS 28.541 [15]. |

* * * Next Change * * * *

##### 8.4.2.6.1	MDA type

The MDA type for UE throughput analysis is: UEThroughputAnalytics.UEThroughputProblemAnalysis.

##### 8.4.2.6.2	Enabling data

The enabling data for UEThroughputAnalytics.UEThroughputProblemAnalysis MDA type are provided in table 8.4.2.6.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.2.6.2-1: Enabling data for UE throughput analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | UE Throughput | UE Throughput including DL and UL throughput in gNB (clause 5.1.1.3 in TS 28.552 [4]) |
|  | Radio resource utilization | Radio resource utilization including DL and UL PRB usage (clause 5.1.1.2 in TS 28.552 [4]) |
|  | RRC connection number | RRC connection number (clause 5.1.1.4 in TS 28.552 [4]) |
| MDT reports | MDT reports containing RSRPs of the serving cell and neighbour cells, and UE location. | RSRPs and UE location of M1 measurements for NR in TS 32.422 [6] and TS 32.423 [7]. |
| Geographical data | The geographical information (longitude, latitude, altitude) of the deployed RAN. | The geographical information (longitude, latitude, altitude) information (see the peeParametersList attribute of the ManagedFunction IOC in TS 28.622 [19]). |

* * * Next Change * * * *

##### 8.4.4.1.3	Analytics output

The specific information elements of the analytics output for energy saving analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.4.1.3-1.

Table 8.4.4.1.3-1: Analytics output for energy saving analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| energyEfficiencyProblematicObject | Indication of NR cells or NFs where the energy efficiency issues occurred or potentially occur. | M | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| energyEfficiencyProblemType | Indication of type of the energy efficiency issues. allowedValues: HIGH_ENERGY_CONSUMPTION, LOW_ENERGY_CONSUMPTION, OTHER, UNKNOWN. | M | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trafficLoadTrends | The predictions of the trends of traffic load in a certain time period. The predictions include the traffic load of the issue cell(s) and neighboring cell(s). | M | type: TrafficLoadTrend multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| rANenergySavingRecommendations | For ES on NR cells. It may contain a set of: - Recommended NR Cell (ES-Cell) to enter energySaving state. - Recommended candidate cells with precedence for taking over the traffic of the ES-Cell. - The time to enter and terminate the energy saving state. - The load threshold to enter and terminate the energy saving state for the ES-Cell. This exist only in case of RAN energy saving is supported. | CM | type: EsRecommendationsOnNRcell multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| cNenergySavingRecommendations | For ES on UPFs. It contains a set of: - Recommended UPF (ES-UPF) to conduct energy saving. - Recommended candidate UPFs with precedence for taking over the traffic of the ES-UPF. - The time to conduct energy saving for the ES-UPF. This exist only in case of CN energy saving is supported. | CM | type: EsRecommendationsOnUPF multiplicity: 1..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| statisticsOfCellsEsState | The statistic result of current energy saving state of the cells at a certain time, which can be used by consumers to make analysis (e.g. observed service experience analysis made by NWDAF) or to make decision (e.g. enter/exit the energy saving state based on the current energy saving state). | O | type: StatisticOfCellEsState multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |

* * * Next Change * * * *

###### 8.4.7.1.1.3	Analytics output

The specific information elements of the analytics output for virtualized resource utilization analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.7.1.1.3-1.

Table 8.4.7.1.1.3-1: Analytics output for virtualized resource utilization analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| lowVRUsageNFs | The NFs with low virtualized resource usage (see Note 1) during some time periods in the past. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| highVRUsageNFs | The NFs with high virtualized resource usage (see Note 1) during some time periods in the past. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| predictedVRUsageForNFs | The predicted virtualized resource usage for NFs during some time periods in the future. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendedActions | The recommended actions to orchestrate the resource allocation for NFs. The recommended action may be (but not limited to): - scale in a list of NFs; - scale out a list of NFs. | M | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NOTE 1: It is up to the MDA MnS producer to decide the thresholds for low and high usage. |  |  |  |

* * * Next Change * * * *

###### 8.4.7.1.2.1	MDA type

The MDA type for physical resource utilization analysis is: ResourceAnalytics.PhysicalResourceUtilizationAnalysisNF.

* * * Next Change * * * *

###### 8.4.7.1.2.3	Analytics output

The specific information elements of the analytics output for physical resource utilization analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.7.1.2.3-1.

Table 8.4.7.1.2.3-1: Analytics output for physical resource utilization analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| lowPRUsageNFs | The NFs with low physical resource usage (see Note 1) during some time periods in the past. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| highPRUsageNFs | The NFs with high physical resource usage (see Note 1) during some time periods in the past. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| predictedPRUsageForNFs | The predicted physical resource usage for NFs during some time periods in the future. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendedActions | The recommended actions to orchestrate the resource allocation for NFs. The recommended action may include (but not limited to) optimising the capacity of gNB (e.g., increasing or decreasing physical resources). | M | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NOTE 1: It is up to the MDA MnS producer to decide the thresholds for low and high usage. |  |  |  |

* * * Next Change * * * *

###### 8.4.7.1.3.3	Analytics output

The specific information elements of the analytics output for control plane congestion analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.7.1.3.3-1.

Table 8.4.7.1.3.3-1: Analytics output for 5GC control plane congestion analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| affectedObject | Indication of 5GC NFs where congestion issues occurred or potentially may occur. | M | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cPCongestionIssueID | This field holds the ID of the control plane congestion issue which is reported. | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| recommendedActions | The recommended actions to orchestrate the resource allocation for 5GC NFs. The recommended action may be (but not limited to): - scale out a list of 5GC NFs; | O | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| rootCause | The root cause of control plane congestion issue. Allowed values: NETWORK_FAILURE, SIGNALLING_OVERLOAD | O | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| predictedCongestionEndTime | It indicates the predicted end time of the congestion if the recommended actions are not performed. | O | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

* * * Next Change * * * *

##### 8.4.9.1.2	Enabling data

The enabling data for traffic steering analytics are provided in table 8.4.9.1.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.9.1.2-1: Enabling data for traffic steering analytics

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Packet loss rate | DL packet loss rate on Uu interface (clause 5.1.1.35 of TS 28.552 [4]) |
|  | SDU loss rate and F1U loss rate | UL PDCP SDU loss rate and DL/UL F1-U packet loss rate (clause 5.1.3.1 of TS 28.552 [4]) |
|  | GTP packet loss rate | Incoming and outgoing GTP packet loss rate (clauses 5.4.1.7, 5.4.1.8 from TS 28.522 [4]) |
|  | Packet delay measurements | GTP packet delay (clause 5.4.1.9 from TS 28.552 [4]) |
|  | UE level performance measurements | packet delay and packet loss rate (clauses 6.2.2.1, 6.3.1.1, 6.3.1.2, 6.3.1.3 from [30]) |
| MDT reports | MDT reports containing RSRPs of the serving cell and neighbour cells, and UE location. | M6 and M7 measurements for NR in TS 32.422 [6] and TS 32.423 [7]. |

* * * Next Change * * * *

#### 8.5.10.2	Information elements

Table 8.5.10.2-1

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| projectionTime | The time duration for which the projections are made. | M | type: ProjectionDuration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uPFProjections | This specifies the traffic projection of a UPF in the slice. It shall be present only if the analysis target contains CN part. | CM | type: UPFProj multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBProjections | This specifies the traffic projection of a gNB in the slice. It shall be present only if the analysis target contains AN part. | CM | type: gNBProj multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sMFProjections | This specifies the projected number of PDU session of a SMF in the slice. It shall be present only if the analysis target contains CN part. | CM | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aMFProjections | This specifies the projected number of registered subscribers of an AMF in the slice. It shall be present only if the analysis target contains CN part. | CM | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

* * * Next Change * * * *

#### 8.5.13.1	Definition

This data type specifies the information about the target cell and gNB for handover.

The attribute isOptimal specify if the cell (served by gNB) is optimal for handover considering the current virtual, physical and radio resource consumption by the gNB and/or the cell. The value TRUE imply that the target is not resource deprived at present and can be selected for handover.

The attribute futureOptimalInfo specify if the cell (served by the gNB) will be optimal for handover at a future point of time considering the future virtual and radio resource consumption by the gNB and/or the cell. This will also provide projection of future virtual, and radio resource consumptions.

* * * Next Change * * * *

#### 8.5.15.2	Information elements

Table 8.5.15.2-1

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| virtualCPU | It indicates the average number of virtual CPU (see definition of numVirtualCpu in clause 7.1.9.2.3.2 of ETSI GS NFV-IFA 011 [26]) usage over the time duration indicated by FutureOptimalTime attribute. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| virtualMemory | It indicates the average virtual memory size (see definition of virtualMemSize in clause 7.1.9.2.3.2 of ETSI GS NFV-IFA 011 [26]) usage over the time duration indicated by FutureOptimalTime attribute. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| virtualDisk | It indicates the average virtual storage size (see definition of sizeOfStorage in clause 7.1.9.2.3.2 of ETSI GS NFV-IFA 011 [26]) usage over the time duration indicated by FutureOptimalTime attribute. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

* * * Next Change * * * *

### 8.6.1	MDAType <<enumeration>>

Table 8.6.1-1: <<enumeration>> MDAType

| Enumeration value | Description |
| --- | --- |
| "COVERAGEANALYTICS_COVERAGEPROBLEMANALYSIS" | Indicates that the MDA type for the Coverage problem analysis defined in clause 8.4.1.1 |
| "COVERAGEANALYTICS_PAGINGOPTIMIZATION" | Indicates that the MDA type for the Paging Optimization defined in clause 8.4.1.2 |
| "SLSANALYSIS_SERVICEEXPERIENCEANALYSIS" | Indicates that the MDA type for the Service experience analysis defined in clause 8.4.2.1 |
| "SLSANALYSIS_NETWORKSLICETHROUGHPUTANALYSIS" | Indicates that the MDA type for the Network slice throughput analysis defined in clause 8.4.2.2 |
| "SLSANALYSIS_NETWORKSLICETRAFFICANALYSIS" | Indicates that the MDA type for the Network slice traffic prediction defined in clause 8.4.2.3 |
| "SLSANALYSIS_E2ELATENCYANALYSIS" | Indicates that the MDA type for the E2E latency analysis defined in clause 8.4.2.4 |
| "SLSANALYSIS_NETWORKSLICELOADANALYSIS" | Indicates that the MDA type for the Network slice load analysis defined in clause 8.4.2.5 |
| "MDAASSISTEDFAULTMANAGEMENT_FAILUREPREDICTION" | Indicates that the MDA type for the MDA assisted failure prediction defined in clause 8.4.3.1 |
| "MDAASSISTEDENERGYSAVING_ENERGYSAVINGANALYSIS" | Indicates that the MDA type for the Energy saving analysis defined in clause 8.4.4.1 |
| "MOBILITYMANAGEMENTANALYTICS_MOBILITYPERFORMANCEANALYSIS" | Indicates that the MDA type for the Mobility performance analysis defined in clause 8.4.5.1 |
| "MOBILITYMANAGEMENTANALYTICS_HANDOVEROPTIMIZATION" | Indicates that the MDA type for the Handover Optimization analysis defined in clause 8.4.5.2 |
| "MAINTENANCE_MAINTENANCEANALYTICS" | Indicates that the MDA type for the Maintenance Analytics defined in clause 8.4.6.1 |
| "RESOURCEANALYTICS_VIRTUALIZEDRESOURCEUTILIZATIONANALYSISNF" | Indicates that the MDA type for the Virtualized resource utilization analysis defined in clause 8.4.7.1.1 |
| "RESOURCEANALYTICS_PHYSICALRESOURCEUTILIZATIONANALYSISNF" | Indicates that the MDA type for the Physical resource utilization analysis defined in clause 8.4.7.1.2 |
| "RESOURCEANALYTICS_5GCCONTROLPLANECONGESTIONANALYSIS" | Indicates that the MDA type for the 5GC Control plane congestion analysis defined in clause 8.4.7.1.3 |
| "PREDICTIONS_PMDATA" | Indicates that the MDA type for the MDA assisted PM predictions defined in clause 8.4.8.1 |
| "UETHROUGHPUTANALYTICS_UETHROUGHPUTPROBLEMANALYSIS" | Indicates that the MDA type for the UE throughput analysis defined in clause 8.4.2.6 |

* * * Next Change * * * *

#### 9.3.2.1	Definition

The IOC MDARequest represents the MDA output request created by an MnS consumer.

The attribute requestedMDAOutputs contains one or multiple MDAOutputPerMDAType elements, and each MDAOutputPerMDAType element supports filtering of MDA output for a certain MDA type.

The attribute reportingTarget may contain the target address, which instructs the MDA MnS producer to create, on behalf of the MnS consumer, a subscription of MDA report.

The MDA MnS consumer may provide the performance threshold information to MDA MnS producer for collecting and reporting the statistics information related to failure prediction. The attribute performanceThresholdInfo can contain one or multiple performance metrics information. If already existing ThesholdMonitor instances (e.g., ThesholdMonitor instance contained by MLTrainingFunction from TS 28.105 [24] ) are available to use, the attribute thresholdMonitorRefList can be used.

The MDA MnS consumer may state a filter to exclude recommended actions targeted towards entities for which the MDA MnS consumer cannot execute actions. The IE “recommendationFilter” can contain a list of DNs as “ManagedEntitiesScope” or can contain a list of geographical areas indicating all the entities falling under these areas are not considered for analytics recommendations. The “recommendationFilter” indicates the list of objects for which no actions can be taken and thus no prescriptive recommendations should be given.

* * * End of Changes * * * *