3GPP TSG-SA5 Meeting #161	S5-252560

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Input to DraftCR Rel-19 TS 28.105 Update on the use case of managing ML models in use in a live network |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT_Ph2 |  |  |  |  |  | Date: |  |  | 2024-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The solution for impact management in S5-252108 targets to satisfy the reqirements of managing ML models in use in a live network. However, the proposed solution is to enhance the AIMLInferenceReport by adding the potential impcat information generated by the ML Model. But the requirements propose to identify ML Model(s) that caused performance degradation and recommend remedial actions to address performance measurement and/or KPI degradation caused by ML model(s). In other words, REQ-AI/ML_INF-LIVES-01 and REQ-AI/ML_INF-LIVES-03 is not satisfied and REQ-AI/ML_INF-LIVES-02 is partially satisfied. This contribution proposes to update the use case and reqirement to align with the proposed solution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update on the use case of managing ML models in use in a live network |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The req is not aligned with the solution. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No impact on Stage 3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the First Change

## 6.5	AI/ML inference

6.5.X1	Managing ML models in use in a live network

6.5.X1.1	Description

ML Model running in the live network may cause potential network performance degradation. For example, an ML model for energy saving may conflict with an ML model for SLA assurance.  Specifically, if an energy-saving ML model is activated while an SLA assurance ML model is already operating in the live network, network performance degradation is likely to occur.  To mitigate this risk, the AI/ML Inference Producer should be capable of reporting potential impact information, including impacted managed objects and network performance metrics. Then, the MnS Consumer can manage the network performance based on such information and local policy, i.e. give up performing the recommended action in the inference report to prevent network performance degradation or take remedial actions when network performance degradation appears.

These use cases below deal with managing ML models that are in-use or deployed in the live network. These use cases will apply to all supported AI/ML inference capabilities.

6.5.X1.2 				Handling of underperforming ML trained models in live networks

Actions may need to be taken by a network operator once trained ML models has been identified that is contributing towards non-optimal running of the network. These actions may involve for example, without service interruptions, reverting to running of the network without ML based optimizations (i.e deavtivating the AIMLInferenceFunction) or replacing current ML model with an earlier model one that was performing better (i.e deploying a new MLmodel).

6.5.X1.3	Performance monitoring of Network Functions with ML trained models in live networks

Several trained ML models maybe in use in an operator network with each one of them influencing network performance. The Network Functions with these ML models need to be monitored to ensure network is running optimally with these models in use. KPIs for evaluating runtime performance of Network Functions using ML models should be provided for this purpose as part of the model. These KPIs correspond to specific network functions of 5GC AI/ML supported functions and NG-RAN AI/ML supported functions, and would need to be collectable by the network operator. To help the AI/ML Inference Consumer perform network performance monitoring, the AI/ML Inference Producer should support to report potential impact information including impacted managed objects and network performance metrics. Remedial actions maybe taken when network performance degradation is monitored.

Next change

6.5.1.3	Requirements for AI/ML inference performance management

Table 6.5.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ- AI/ML_INF_PE-01 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to get the inference output provided by an AI/ML inference function (e.g., MDAF, NWDAF or RAN function). | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ- AI/ML_INF_PE-02 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to get the performance evaluation of an AI/ML inference output as measured by a defined set of performance metrics | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ- AI/ML_INF_PE-03 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to provide feedback about an AI/ML inference output expressing the degree to which the inference output meets the consumer's expectations. | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ- AI/ML_INF_PE-04 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to be informed about the executed actions that were triggered based on the inference output provided by an AI/ML inference function (e.g., MDAF, NWDAF or RAN function). | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ- AI/ML_INF_PE-05 | The MnS producer responsible for AI/ML inference management shall have a capability enabling an authorized consumer to obtain the performance data related to an ML model or an AI/ML inference function (e.g., MDAF, NWDAF or RAN function). | AI/ML inference performance evaluation (clause 6.5.1.2.1) |
| REQ-AI/ML_PERF-SEL-1 | The ML training MnS producer shall have a capability allowing an authorized MnS consumer to discover supported AI/ML performance measurements related to AI/ML inference and select some of the desired measurements based on the MnS consumer’s requirements. | AI/ML performance measurements selection based on MnS consumer policy (clause 6.5.1.2.2) |
| REQ-AI/ML_PERF-POL-1 | The AI/ML MnS producer shall have a capability allowing the authorized MnS consumer to indicate a performance policy related to AI/ML inference. | AI/ML performance measurements selection based on MnS consumer policy (clause 6.5.1.2.2) |
| REQ-AI/ML_INF-LIVES-01 | The 3GPP management system should have a capability enabling an authorized consumer to identify ML Model(s) that caused performance measurement and/or KPI degradation. | Handling of underperforming ML trained models in live networks (6.5.X1.2.1) |
| REQ-AI/ML_INF-LIVES-02 | The 3GPP management system should have a capability to recommend remedial actions to address performance measurement and/or KPI degradation caused by ML model(s). | Handling of underperforming ML trained models in live networks (6.5.X1.2.1) |
| REQ-AI/ML_INF-LIVES-03 | The 3GPP management system should be able to collect network related performance data pertaining to Network Functions actively utilizing ML models. | Performance monitoring of Network Functions with ML trained models in live networks (6.5.X1.2.2) |
| REQ-AI/ML_INF-LIVES-04 | The 3GPP management system should enable the producer to report potential impact information for each AIMLInferenceName, including impacted managed objects and network performance metrics. | Performance monitoring of Network Functions with ML trained models in live networks (6.5.X1.2.2) |

Note: the requirements without feasible solution in the release should be removed.

End of the First Change